BOOTS.DOC -- Changes from V22(105) to V23(114)

June 1979

COPYRIGHT (C) 1971,1975,1979 BY

DIGITAL EQUIPMENT CORPORATION, MAYNARD, MASS.

THIS SOFTWARE IS FURNISHED UNDER A LICENSE AND MAY BE USED AND  COPIED

ONLY  IN  ACCORDANCE  WITH  THE  TERMS  OF  SUCH  LICENSE AND WITH THE

INCLUSION OF THE ABOVE COPYRIGHT NOTICE.  THIS SOFTWARE OR  ANY  OTHER

COPIES  THEREOF MAY NOT BE PROVIDED OR OTHERWISE MADE AVAILABLE TO ANY

OTHER PERSON.  NO TITLE TO AND OWNERSHIP OF  THE  SOFTWARE  IS  HEREBY

TRANSFERRED.

THE INFORMATION IN THIS SOFTWARE IS SUBJECT TO CHANGE  WITHOUT  NOTICE

AND  SHOULD  NOT  BE  CONSTRUED  AS  A COMMITMENT BY DIGITAL EQUIPMENT

CORPORATION.

DIGITAL ASSUMES NO RESPONSIBILITY FOR THE USE OR  RELIABILITY  OF  ITS

SOFTWARE ON EQUIPMENT WHICH IS NOT SUPPLIED BY DIGITAL.

BTS23.DOC                                                       Page 2

BOOTS.DOC -- Changes from V22(105) to V23(114)

June 1979

1.0  SUMMARY

This version of BOOTS incorporates full support for the KS10 processor

and  additions  to  support  all  eight  RH20  controllers on the KL10

processor.  Support for the KA10 processor has been removed.

2.0  EXTERNAL CHANGES

None.

3.0  KNOWN BUGS AND DEFICIENCIES

If BOOTS is assembled for the KL10 or KS10 with FT22BIT turned on, the

version  which  is  written  onto disk packs will go into a page fault

loop whenever the monitor reads it to do an auto dump/reload  sequence

if the CCL text at BOOTXT in COMMON crosses a page boundry.

4.0  INSTALLATION INSTRUCTIONS

None.

5.0  INTERNAL CHANGES

A new feature test  switch  FTKS10  has  been  added  for  conditional

assembly  of  BOOTS  that  supports the KS10 with RH11.  Code has also

been added to support eight RH20's.

6.0  SUGGESTIONS

None.

[End of BTS23.DOC]

[BTS22.DOC is appended as an integral part of BTS23.DOC]

BTS22.DOC                                                       Page 3

BOOTS.DOC -- Changes from V22(104) to V22(105)

August 1978

1.0  SUMMARY

The main reason for this release is to fix 2 bugs in V22(104).

2.0  EXTERNAL CHANGES

None.

3.0  KNOWN BUGS AND DEFICIENCIES

If BOOTS is assembled for the KL10 with FT22BIT turned on, the version

which  is  written  onto  disk  packs  will  go into a page fault loop

whenever the monitor reads it to do an auto  dump/reload  sequence  if

the CCL text at BOOTXT in COMMON crosses a page boundry.

4.0  INSTALLATION INSTRUCTIONS

None.

5.0  INTERNAL CHANGES

A bug in V22(104) which could cause BOOTS to hang, without ever typing

its  prompt,  when the KL10 was just powered up has been fixed.  A bug

has been fixed which would cause BOOTS not to be able to read from  an

RH20 after a data error on that controller.

6.0  SUGGESTIONS

None.

[End of BTS22.DOC]

[BTS22.DOC is appended as an integral part of BTS22.DOC]

BTS22.DOC                                                       Page 4

BOOTS.DOC -- Changes from V21C to V22(104)

March 1977

1.0  SUMMARY

The main reason for this release of BOOTS is to fix a bug  in  version

21  which  caused  BOOTS  to have to be restarted under certain errors

which can occur on an RP04/RP06.

2.0  EXTERNAL CHANGES

Paper-tape BOOTS (or  the  version  gotten  by  loading  it  from  the

front-end  of a KL10) now starts by typing its version number.  Before

every command line is asked for BOOTS  now  types  a  prompt  "<BTS>",

which  indicates that it is ready for a command.  Paper-tape BOOTS can

now be assembled so that it will work on either an RH10  or  an  RH20.

The  BOOTS  which  is  distributed on the KL10 front-end tape has been

assembled to work on either.  The names  of  the  bootstraps  are  now

BT128K and BT256K, for use on a 128K or 256K KL10.

Note that the BOOTS which is written on blocks 4-7 of a disk pack does

not  have these changes, it still must be assembled for either an RH10

or RH20, and will not work on the "wrong" one.  Neither does  it  type

its version number nor prompt before each command.

3.0  KNOWN BUGS AND DEFICIENCIES

If BOOTS is assembled for the KL10 with FT22BIT turned on, the version

which  is  written  onto  disk  packs  will  go into a page fault loop

whenever the monitor reads it to do an auto  dump/reload  sequence  if

the CCL text at BOOTXT in COMMON crosses a page boundary.

4.0  INSTALLATION INSTRUCTIONS

None.

5.0  INTERNAL CHANGES

A bug in version 21C, distributed with the 6.02A combined  LIR,  which

made  it necessary to type the structure name as well as the file name

has been fixed.  If an RP04/RP06 is disabled and then  enabled,  BOOTS

will still be able to read from it without restarting.

BTS22.DOC                                                       Page 5

6.0  SUGGESTIONS

None.

[End of BTS22.DOC]

[BTS21C.DOC is appended as an integral part of BTS22.DOC]

BTS21C.DOC                                                      Page 6

BOOTS.DOC -- Changes from V21 to V21C

October 1976

1.0  SUMMARY

The main reasons for this release of BOOTS are to  support  the  1088,

RP06, and RH20 LIRs.

2.0  EXTERNAL changes

Since BOOTS V21C will support either an RH10 or an RH20 but not  both,

a  new  set of .A10, .CCL, and .CMD files are being distributed on the

KL10 front end bootstrap tape.  These files are:

B11281.CMD     Load   micro-code,   configure   memory   1    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 128K RH10 BOOTS.

B11282.CMD     Load   micro-code,   configure   memory   2    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 128K RH10 BOOTS.

B11284.CMD     Load   micro-code,   configure   memory   4    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 128K RH10 BOOTS.

B21281.CMD     Load   micro-code,   configure   memory   1    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 128K RH20 BOOTS.

B21282.CMD     Load   micro-code,   configure   memory   2    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 128K RH20 BOOTS.

B21284.CMD     Load   micro-code,   configure   memory   4    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 128K RH20 BOOTS.

B12561.CMD     Load   micro-code,   configure   memory   1    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 256K RH10 BOOTS.

B12562.CMD     Load   micro-code,   configure   memory   2    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 256K RH10 BOOTS.

B12564.CMD     Load   micro-code,   configure   memory   4    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 256K RH10 BOOTS.

BTS21C.DOC                                                      Page 7

B22561.CMD     Load   micro-code,   configure   memory   1    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 256K RH20 BOOTS.

B22562.CMD     Load   micro-code,   configure   memory   2    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 256K RH20 BOOTS.

B22564.CMD     Load   micro-code,   configure   memory   4    way

interleaved,   configure  cache,  load  and  start

RSX10-F and 256K RH20 BOOTS.

BT1281.CCL     Load and start RSX10-F and 128K RH10 BOOTS.

BT1282.CCL     Load and start RSX10-F and 128K RH20 BOOTS.

BT2561.CCL     Load and start RSX10-F and 256K RH10 BOOTS.

BT2562.CCL     Load and start RSX10-F and 256K RH20 BOOTS.

BT1281.A10     128K RH10 BOOTS.

BT1282.A10     128K RH20 BOOTS.

BT2561.A10     256K RH10 BOOTS.

BT2562.A10     256K RH20 BOOTS.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

None.

BTS21C.DOC                                                      Page 8

5.0  INTERNAL changes

A new feature test switch has been added FTRH20  which  defaults  off.

If  this  feature  test  switch  is  turned  on, BOOTS will be able to

load/dump from/to an RH20 control/channel.  However, it will no longer

be  able  to  operate  an  RH10  controller  (See  section  3 for more

details).  Also, BOOTS has been changed to support loading and dumping

from/to  RP06  packs.   Finally,  to  implement 1088 and RH20 support,

BOOTS  V21C  uses  exec  page  table  relative  addressing   for   DTE

communications  rather  than  page  0  communications locations.  This

requires that MON.BIN (V5A or later) and KLDCP (V11 or later) be  used

with  BOOTS  V21C.   These  files  are  included on the KL10 front end

DECtape distributed with the LIRs.

[End of BTS21C.DOC]

[BTS21.DOC is appended as an integral part of BTS21C.DOC]

BTS21.DOC                                                       Page 9

BOOTS.DOC -- Changes from V20 to V21

October 1975

1.0  SUMMARY

The main purpose of this release of BOOTS is to fix a bug  in  version

20  of  BOOTS which caused it to go into a page fault loop when loaded

by the 6.02 monitor on a KI10 if the feature test  switch  FT22BIT  is

turned  off in BOOTS.  Version 21 also supports loading and dumping on

a KL10 with more than 256K of memory, and,  with  this  version,  auto

dump/reload wfile no longer apply.

5.0  EXTERNAL CHANGES

None.

[End of BTS21.DOC]

[BTS20.DOC is appended as an integral part of BTS21.DOC]

BTS20.DOC                                                      Page 10

BOOTS.DOC -- Changes from V17 to V20

July 1975

1.0  SUMMARY

A number of feature test switches have been incorporated into BOOTS in

order  that  it  may  support bootstraping from and dumping to several

devices from files in SAV and/or EXE format on  KA10,  KI10,  or  KL10

processors.   In  particular, with version 20 it is possible to make a

paper tape BOOTS which in addition to supporting the  KL10  processor,

can  read/write  SAV or EXE files from/to RM10B's, RD10s, RP02s, RP03s

and RP04s.  It is also possible to make a BOOTS  to  be  written  onto

disk  packs  by WTBOOT which is capable of dumping up to 4096K of core

on KI10 or KL10 based systems (the 6.02 monitor is required  for  this

feature).   Also, BOOTS has been changed to display a fixed error code

between 1 and 16 in the lights (type the letters B through  O  on  the

operator console of the KL10) whenever it encounters an error while it

is loading or dumping.

2.0  EXTERNAL CHANGES

BOOTS must be assembled  with  the  appropriate  feature  test  switch

settings  to produce a BOOTS.REL file to be loaded with WTBOOT to take

advantage of features which are available in BOOTS but which  are  not

normally  turned  on.   The  paper  tape version of BOOTS is now three

pages (page=512 words) long (2 code pages and a data page) so  dumping

crashes  with paper tape BOOTS is not recommended if it can be avoided

since an additional page of information which would be dumped  by  the

BOOTS  read  from  a  disk  pack  will  be  lost.   BOOTS displays the

following error codes in the lights (or types the specified letter  on

the  KL10)  to  indicate  what  specific  problem it encountered while

reading or writing.

BTS20.DOC                                                      Page 11

ERROR CODE    ERROR        MEANING

DISPLAYED IN   LETTER

THE LIGHTS

1           B     NO COMMA INSIDE []

2           C     ILLEGAL SWITCH OR NO START ADDRESS

SPECIFIED BEFORE /G

3           D     INPUT ERROR

4           E     INPUT ERROR READING THE EXE DIRECTORY

5           F     FIRST PAGE OF THE FILE ISN'T AN EXE

DIRECTORY OR IS AN EXE DIRECTORY WHICH IS

TOO LONG

6           G     PAGES DESCRIBED IN THE EXE DIRECTORY

AREN'T MONOTONICALLY INCREASING

7           H     PREMATURE END OF FILE

10           I     OUTPUT ERROR

11           J     FILE NOT FOUND

12           K     BAD RIB

13           L     PREMATURE END OF FILE READING THE UFD

14           M     ATTEMPT TO READ/WRITE BLOCK 0

15           N     LOGICAL BLOCK NUMBER EXCEEDS SIZE OF THE

DISK

16           O     END OF FILE ON OUTPUT

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

See WTB4.DOC.

5.0  INTERNAL CHANGES

There are now 10 feature test switches in BOOTS.  For the  purpose  of

making  a paper tape version of BOOTS, these feature test switches can

be ignored.  However, to make a BOOTS.REL which is to be  loaded  with

WTBOOT  to write BOOTS on blocks 4-7 of disk packs, it is necessary to

turn off some of the switches.  The feature test  switches  which  are

currently implemented in BOOTS are:

FTKA10,   include support for KA10 processors

FTKI10,   include support for KI10 processors

FTKL10,   include support for KL10 processors

FTSAVE,   include support for SAV files

FTEXE,    include support for EXE files

FTFHD,    include support for RM10Bs and RD10s

FTDPC,    include support for RP02s and RP03s

FTDHX,    include support for RP04s

BTS20.DOC                                                      Page 12

FT22BIT,  include support for dumping more than 256K

FTWTBOOT, assemble BOOTS to be loaded with standard (field image)

WTBOOT.   Defaults  to on and would normally be on when

configuration dependent BOOTS.REL files are produced to

be loaded with WTBOOT.

In the paper tape version of BOOTS, all  of  the  above  feature  test

switches  (except  FT22BIT) are normally turned on so paper tape BOOTS

will support reading/writing both SAV and EXE format files from/to any

supported device on a KA10, KI10 or KL10 processor.  See BOOTS.MEM for

assembly instructions for producing a paper tape version of BOOTS.

In general, it is not possible to assemble BOOTS to load  with  WTBOOT

such  that  it will support EXE files and more than one type of device

and still have it be small enough to fit on blocks 4-7 of a disk pack.

Therefore,  the  BOOTS  configuration which will most commonly be used

should be assembled and loaded with WTBOOT and  written  on  all  disk

packs,  e.g.,  RP02/RP03  BOOTS  should be written on all packs if the

monitor is normally on an RP02/RP03  pack  and  crashes  are  normally

dumped on CRASH.SAV or CRASH.EXE on an RP02/RP03 pack.

[End of BTS20.DOC]

[BTS17.DOC is appended as an integral part of BTS20.DOC]

BTS17.DOC                                                      Page 13

BOOTS.DOC -- Changes from V16 to V17

January 1975

1.0  SUMMARY

BOOTS version 17 supports RP04's.  However, to keep the size of  BOOTS

under  1K, something else had to go.  The choice was to put fixed head

disk support (RC10, RM10B) under a conditional  assembly  switch.   If

FTFHD=-1  (the default value), then BOOTS will talk to RC10's, RP02's,

and RP03's (but NOT RP04's).  If FTFHD=0, then BOOTS talks to  RP02's,

RP03's, and RP04's (but NOT RC10's).

BOOTS version 17 supersedes all previous versions of BOOTS.

2.0  EXTERNAL CHANGES

BOOTS can now access RP04's.   All  commands  and  operator  interface

remain unchanged.

3.0  KNOWN BUGS AND DEFICIENCIES

BOOTS will only dump or load the first 255K of physical core.  This is

true even if 4096K exist.

BOOTS may not correctly dump into a file which is on a multi-unit file

structure if the file is split across units.

BOOTS still does not support the RS04.

A self-relocating paper tape version is still  recognized  as  a  good

idea, but has not been implemented.

4.0  INSTALLATION INSTRUCTIONS

Since the default value of FTFHD produces BOOTS without RP04  support,

one  must  assemble  BOOTS  with a parameter file defining FTFHD==0 to

obtain an RP04 BOOTS.  The version of WTBOOT.CTL which accompanies the

RP04  LIR  contains the command sequence to accomplish this.  It is as

follows:

BTS17.DOC                                                      Page 14

.MAKE BPARAM.MAC

*I FTFHD==0

*$EX$$

.COMPILE BPARAM+BOOTS

The installation instructions for paper tape  versions  of  BOOTS  are

similarly affected.  See BOOTS.CTL and WTBOOT.CTL.

5.0  INTERNAL CHANGES

The only internal changes are the definitions of the  FTFHD  and  FTDH

feature  test  switches and the use of these switches to conditionally

assemble either the existing code for fixed head disks or the new code

which handles RP04's.

[End of BTS17.DOC]

[BTS16.DOC is appended as an integral part of BTS17.DOC]

BTS16.DOC                                                      Page 15

BOOTS.DOC -- Changes from V15 to V16

January 1974

1.0  SUMMARY

This release adds 3 new features for 5.07/6.01:

1.  BOOTS now supports up to 4096K  memory  using  the  DF10C  in

22-bit mode.

2.  BOOTS may be given a command string by storing a byte pointer

to  the  string  in  location  777  relative  to the starting

address of BOOTS.  This lets the monitor reload itself.

3.  BOOTS stores the serial number of the CPU in low core.   This

allows a 1077 monitor to use the same CPU as the master.

BOOTS version 16 supersedes all  previous  versions  of  BOOTS.   This

version supports the DATE75 project.

2.0  EXTERNAL CHANGES

None.

3.0  KNOWN BUGS AND DEFICIENCIES

BOOTS will only dump or load the first 255K of physical core.  This is

true even if 4096K exist.

BOOTS may not correctly dump into a file which is on a multi-unit file

structure if the file is split across units.

BOOTS does not support the RS04 or RP04.

4.0  INSTALLATION INSTRUCTIONS

None.

5.0  INTERNAL CHANGES

None

[End of BTS16.DOC]

[BTS15.DOC is appended as an integral part of BTS16.DOC]

BTS15.DOC                                                      Page 16

BOOTS.DOC -- Changes from V13 to V15

March 1972

1.0  SUMMARY

This release enables BOOTS to service RP03 disk  drives.   Version  15

does  not  support RP01's.  No bugs have been reported to the released

version 13.

2.0  EXTERNAL CHANGES

None.

3.0  KNOWN BUGS AND DEFICIENCIES

RP01 drives no longer supported.  In the  future  make  a  paper  tape

bootstrap  loader  which determines how big memory is so can have only

one size for paper tape too.  Version 15  still  requires  a  separate

assembly for each size paper tape version.

4.0  INSTALLATION INSTRUCTION CHANGES

None.

5.0  INTERNAL CHANGES

All references to RP01 drives have  been  removed.   Bit  R.TYPE=1  in

AC(F) now means drive is RP03 if RP10 controller, instead of RP01.

[End of BTS15.DOC]

[BTS13.DOC is appended as an integral part of BTS15.DOC]

BTS13.DOC                                                      Page 17

BOOTS.DOC -- Changes from V4 to V13

April 1971

1.0  SUMMARY

This release adds a few new features to  BOOTS.   No  bugs  have  been

reported to the released version 4.  BOOTS is now location independent

so that a single copy can be written on a disk and loaded into the top

of memory no matter how big memory is.  BOOTS now prints ?  as well as

bell on all errors.  BOOTS indicates the type of error  by  displaying

in  the  lights an error PC.  See new, faster monitor crash proecdures

in 5.03 monitor release.  See also WTBOOT writeup available with  5.03

release.   WTBOOT  is a program to write BOOTS onto blocks 4 through 7

of disk packs.

2.0  EXTERNAL CHANGES

Whenever an error occurs, BOOTS types both a bell and a ?  followed by

a  carriage  return  line-feed.   It also displays the error PC in the

lights.  The error PCs are as follows:

XXX061 no comma inside [ ]'s

XXX131 on /G no starting address specified previously

XXX134 not a legal command or decimal number

XXX151 file not found on /L or cr command

XXX173 data error on read

XXX201 file not found on /D

XXX236 data error on write

XXX305 bad RIB

XXX326 RIB does not have code word 777777

XXX334 premature EOF reading RIB

XXX622 attempt to read block 0

XXX713 logical block number exceeds size of disk

3.0  KNOWN BUGS AND DEFICIENCIES

None.  For future make a paper tape bootstrap loader which  determines

how  big  memory  is  so  can  have  only one size for paper tape too.

Version 12 still requires a separate assembly for each size paper tape

version.

BTS13.DOC                                                      Page 18

4.0  INSTALLATION INSTRUCTIONS

To make a location independent version to be loaded  with  WTBOOT  for

purposes  of  writing  it  onto  blocks  4 through 7 of disk packs, no

special assembly instruction are needed.

.COM BOOTS

See WTBOOT.DOC for loading instructions for WTBOOT.

To make an absolute paper tape version, define core to be:

40000  for 16K

100000 for 32K

140000 for 48K

200000 for 64K

240000 for 80K

300000 for 96K, etc

.MAKE FTBOOT.MAC

ICORE=240000

$$

EX$$

.R MACRO

PTP:_FTBOOT,BOOTS

5.0  INTERNAL CHANGES

All references to locations in BOOTS are indexed by register  R  which

is loaded up with the lowest address of BOOTS data area.

[End of BTS13.DOC]