DC72NP.DOC

September 1976

Copyright (C) 1975,1976

Digital Equipment Corporation, Maynard, Mass.

This software is furnished under a license for use only  on  a  single

computer system and may be copied only with the inclusion of the above

copyright notice.  This software, or any other copies thereof, may not

be provided or otherwise made available to any other person except for

use on such system and to one  who  agrees  to  these  license  terms.

Title  to  and  ownership of the software shall at all times remain in

DEC.

The information in this software is subject to change  without  notice

and  should  not  be  construed  as  a commitment by Digital Equipment

Corporation.

DEC assumes no responsibility  for  the  use  or  reliability  of  its

software on equipment which is not supplied by DEC.

DC72NP.DOC                                                      Page 2

DC72NP.DOC

September 1976

1.0  SUMMARY

This document reflects the software as of version 5.

The purpose of this release is to support  the  6.02  release  of  the

TOPS10  monitor.   The DC72NP code executes in a DC72NP remote station

for the DECsystem-10.

2.0  ASSEMBLING THE DC72NP CODE

DC72NP code is assembled from 2 files, C.PAL and DC72NP.PAL.  C.PAL is

a  configuration  file  which  is supplied by the user.  DC72NP is the

actual DC72NP source code.

2.1  C.PAL for a DC72NP

For a DC72NP a typical configuration file would be:

PDP8=2                   /DEFINES THIS IS AN PDP8E

TTYN=20                  /DEFINES THERE ARE 16 TTYS PLUS CTY

OURNNM=15                /DEFINES NODE NUMBER FOR DC72NP TO BE 15

DEFINE DC72ID "F;"O;"O>  /DEFINES DC72NP NODE NAME

2.2  Assembling DC72NP code

Typical commands to assemble the DC72NP code are:

.R PAL10

*DC72NP.BIN,DC72NP.LST/C=C.PAL,DC72NP.PAL

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

Place the  DC72NP  binary  file,  which  resulted  from  the  assembly

described in 2.2 above, on SYS:  and then load it into the DC72NP with

NETLDR.   Typical  command  dialogues  with  NETLDR   are   given   in

NETLDR.DOC.

[End of DC72NP.DOC]