DC76.DOC -- Changes from V10 to V21

September 1975

Copyright (C) 1974,1975,1977

Digital Equipment Corporation, Maynard, Mass.

This software is furnished under a license for use only  on  a  single

computer system and may be copied only with the inclusion of the above

copyright notice.  This software, or any other copies thereof, may not

be provided or otherwise made available to any other person except for

use on such system and to one  who  agrees  to  these  license  terms.

Title  to  and  ownership of the software shall at all times remain in

DEC.

The information in this software is subject to change  without  notice

and  should  not  be  construed  as  a commitment by Digital Equipment

Corporation.

DEC assumes no responsibility  for  the  use  or  reliability  of  its

software on equipment which is not supplied by DEC.

D7621.DOC                                                       Page 2

DC76.DOC -- Changes from V10 to V21

September 1975

1.0  SUMMARY

This version of the DC76 code is coupled to the 6.02  release  of  the

TOPS10  monitor.  This version implements no new features but corrects

several deficiencies in version 10.  This version has not been  tested

with  pre-6.02 monitors although there is every reason to believe that

such a combination would operate successfully.

Bibliography:

OPERATION SYSTEM COMMANDS     SOFTWARE NOTEBOOK 2

MONITOR CALLS MANUAL          SOFTWARE NOTEBOOK 4

OPERATOR'S GUIDE              SOFTWARE NOTEBOOK 13

2.0  EXTERNAL CHANGES

MCO# 4965 - Eliminates  input  of  garbage  character  from  2741

terminal in APL mode when attention key is depressed.

MCO# 4987 - Enables typeout of DC76 messages in upper/lower case.

MCO# 5271 - Provides the ablilty to generate  backslash  on  2741

terminals by using ^/.

MCO# 5274 - Corrects operation of TIDY  mode  on  2741  terminals

with element type 963 to conform to documentation.

3.0  KNOWN BUGS AND DEFICIENCIES

There are no known bugs or deficiencies.

4.0  INSTALLATION INSTRUCTIONS

First, for  non-auto  baud  lines,  build  a  parameter  file  C76.P11

defining transmit and receive line speeds.

These definitions are of the form:

T<line#>XS==speed  ;for xmit speed

T<line#>RS==speed  ;for receive speed

e.g.,

T01XS==^D1200

T01RS==^D150

D7621.DOC                                                       Page 3

sets line #1 with a transmit speed of 1200 baud and a receive speed of

150 baud.

Note:  This inhibits the auto baud detect feature for this line.

The DC76 code is assembled using MACDLX.  For  example  to  produce  a

binary and listing:

.R MACDLX

*DC76.BIN,DC76.LST/CREF=C76.P11,DC76.P11

The file DC76.BIN is loaded into the PDP-11 using BOOT11.  For example

to load a DC76 on port 0 of the DL10:

.R BOOT11

*FILE:DC76.BIN/PORT:0

Complete documentation on BOOT11 is found in Notebook 10.

5.0  INTERNAL CHANGES

MCO# 4909 - Adds check and associated error trap for overflow  of

the "to-10" message queue.

MCO# 5016 - Increases timeout to prevent  spurious  "DECsystem-10

not running" messages.

MCO# 5064 - Changes dataset logic to  hang  up  a  line  only  on

command from the -10.

MCO# 5257 - Prevents DC76 from hanging on DM11-BB failure.

MCO# 5317 - Reverts line to auto baud if framing error on a  2741

is  not  a  break  character.   This  prevents  ASCII

terminals from getting locked into 2741 mode if their

speed is taken to be 134.5 baud.

[End of D7621.DOC]

[D7610.DOC is appended as an integral part of D7621.DOC]

D7610.DOC                                                       Page 4

DC76.DOC -- V10

SEPTEMBER 1974

1.0  SUMMARY

This is the first release of the DC76 code.

This software is designed to support any standard DC76  configuration.

The code does not require assembly-time specification of configuration

parameters such as number of lines, etc.

DC76 features include:

-  Control of up to 128 full-duplex  or  full-duplex  with  local

copy asynchronous lines.  These may be local or dial-up lines.

-  Individual line speeds program-selectable  from  50  to  9,600

baud.

-  Split-speed (transmit/receive) operation program-selectable on

a per-line basis.

-  Automatic baud detection for 110,  134.5,  150,  and  300-baud

lines.

-  ASCII, EBCDIC,  APL,  or  Correspondence  Codes  independently

selectable on each line.

2.0  EXTERNAL CHANGES

None.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

The DC76 program is distributed as a binary file (DC76.BIN).

Use BOOT11 to load the DC76.  If the DC76 is on DL10 port  0  use  the

following commands:

.R BOOT11

/CLEAR/PORT:0

DC76.BIN/PORT:0

D7610.DOC                                                       Page 5

Further information on BOOT11 may be found in the software notebooks.

5.0  INTERNAL CHANGES

None.

6.0  SPECIAL CONSIDERATIONS

6.1  Auto-Baud Detection

All DC76 lines will initially be defined  as  auto-baud  lines.   This

means  that the DC76 will scan incoming characters until it finds what

appears to be a carriage return.  When  the  DC76  determines  a  line

speed the monitor will type out:

PLEASE LOGIN OR ATTACH

At this point SET TTY SPEED commands may be used to redefine the  baud

rate.

At times it may be desirable to initialize a line at a  specific  baud

rate  rather  than  auto-baud.   This may be done by using the TTY.INI

file and the INITIA program.

6.2  Dataset Lines

All dataset lines must be declared to MONGEN.   Lines  which  are  not

declared will answer and then hang up 30 seconds later.

6.3  2741 Lines

If the DC76 detects a baud rate of 134.5, it will define the line as a

2741  line with default element type.  At this point a SET TTY ELEMENT

command may be  used  to  define  the  element  being  used.   If  the

PLEASE LOGIN  or ATTACH message is garbled, the system may not be able

to recognize the SET TTY  command.   To  eleviate  this  problem  type

LOGINCARRIAGE  return>.   This will give the system enough information

to be able to decode SET TTY commands.

[End of D7610.DOC]