DN92.DOC -- Changes from V7 to V10

January 1978

COPYRIGHT (C) 1977,1978 BY

DIGITAL EQUIPMENT CORPORATION, MAYNARD, MASS.

THIS SOFTWARE IS FURNISHED UNDER A LICENSE AND MAY BE USED AND  COPIED

ONLY  IN  ACCORDANCE  WITH  THE  TERMS  OF  SUCH  LICENSE AND WITH THE

INCLUSION OF THE ABOVE COPYRIGHT NOTICE.  THIS SOFTWARE OR  ANY  OTHER

COPIES  THEREOF MAY NOT BE PROVIDED OR OTHERWISE MADE AVAILABLE TO ANY

OTHER PERSON.  NO TITLE TO AND OWNERSHIP OF  THE  SOFTWARE  IS  HEREBY

TRANSFERRED.

THE INFORMATION IN THIS SOFTWARE IS SUBJECT TO CHANGE  WITHOUT  NOTICE

AND  SHOULD  NOT  BE  CONSTRUED  AS  A COMMITMENT BY DIGITAL EQUIPMENT

CORPORATION.

DIGITAL ASSUMES NO RESPONSIBILITY FOR THE USE OR  RELIABILITY  OF  ITS

SOFTWARE ON EQUIPMENT WHICH IS NOT SUPPLIED BY DIGITAL.

DN9210.DOC                                                      Page 2

DN92.DOC -- Changes from V7 to V10

January 1978

1.0  SUMMARY

DN92 is a remote  station  on  the  DECsystem-10  network.   It  is  a

sequential  node  based  on  a  PDP8A  processor.  It was developed to

replace the DC72NP remote station.

The DN92 requires a DECsystem-10 with a DC75NP, DN85,  DN87  or  DN87S

front  end,  and a synchronous line between the front end and the DN92

It requires TOPS-10 version 6.02 or later monitors.

Specific documentation on the  DN92  can  be  found  in  the  Software

Notebooks  write  up  entitled  Networks and Software Guide and in the

file  named  DN92.RNO.   The  DN92.RNO  file  briefly  describes   the

differences  in  implementation of the software between the DC72NP and

the DN92.

2.0  EXTERNAL CHANGES

The DN92 is now  being  released  through  the  Software  Distribution

Center  (SDC)  and  is  no longer available as a special release.  The

first version being released through SDC is  version  10.   Two  other

versions have been distributed to the field, version 3 and version 7.

Changes have been made to  version  10  which  correct  the  following

problems:

1.  The card reader was not sending the last column of a card  to

the host (SPR 10-24343).

2.  The line printer was not handling form control for  the  last

six  lines  on  a  page  if  they  only contained CRLF'S (SPR

10-24343).

3.  DN92 was not answering data sets reliably (SPR 10-24274).

4.  Assembly errors if FTDEBUG=0 (SPR 10-2473).

5.  Losing input on a high speed asynchronous line.  The code was

changed  to  process a device control message which causes an

X-OFF character to  be  issued  to  the  device  as  soon  as

possible.

6.  Error message for line printer  was  not  being  sent  if  it

occurred  on  initial  output.   The line printer routine was

modified to send an off-line status message.

7.  DP8E bus errors were  caused  by  the  line  printer  routine

disabling   the   interrupt  for  too  long  when  compressed

DN9210.DOC                                                      Page 3

characters  were  being  flushed.   Zeroing  the   repetitive

character  count  for  flushed  characters  seemed  the  best

solution.  Another solution is to stop the line printer,  let

the flushed character time out, and the clock routine restart

output to the line printer.  Hoewever, while eliminating  all

bus errors, this solution seems to degrade the line printer's

performance and shouldn't be  installed  until  the  definite

problem exists.

8.  The calculation for fill timing was modified to  reflect  the

real-time  clock  on  the  DN92  which interrupts every 10ms.

instead of every 16ms.  (SPR 10-25554).

9.  DAS92 station name was changed to DN92.

3.0  KNOWN BUGS AND DEFICIENCIES

The DN92 does not support trace feature DP8ETR.  It is always  set  to

0.

Programs requiring frequent changes in and out of image mode on a  TTY

line do not always get processed correctly.

4.0  INSTALLATION INSTRUCTIONS

The installation instructions are found in the  DECsystem-10  Software

Notebooks   Installation  Guide  and  in  the  DN92.SIG  file  on  the

distribution tape.

5.0  INTERNAL CHANGES

The DN92 was sending a NAK response for an idle REP.  It now sends  an

ACK  response.   The routine which ships status words to the host only

sends it if it has changed values.  Instead of halting, the  DN92  now

ignores a REQUEST LOAD station control message.

6.0  SUGGESTIONS

Be sure  to  read  the  Network  Software  Installation  Guide  before

installing the system software.  Also be sure to set up the NETLDR.INI

file correctly if there are multiple DN92'S in the network.

[End of DN9210.DOC]