1.0  INTRODUCTION

The 7.02 release of the TOPS-10 monitor contains unsupported

code  to  run DMR11s on a KS10.  Although this code has been

used successfully on in-house systems, it is not an official

part of the Monitor.

This document describes the known limitations and advantages

of  using  a DMR11 on a KS10.  It also describes the process

necessary  to  install  and  use  the  software.    Hardware

installation considerations are also discussed.

No representation is made that this document is complete  or

correct  in  all  respects.   Although every effort has been

made by  the  author  to  ensure  its  correctness,  Digital

disclaims  any  responsibility  for  the  use of unsupported

hardware and/or software, whether or  not  such  use  is  in

accordance  with  information contained in this document, or

any other.

2.0  LIMITATIONS AND RESTRICTIONS

Use of a DMR11 in a KS10 represents a  change  to  both  the

hardware  and  software  environment,  and  may  impact  the

perceived performance of the system.

The  DMR11  hardware's  inherent  performance  exceeds   the

ability  of  the  2020  to  drive  it.   As  a  consequence,

configuring a 2020 which includes DMRs requires evaluating a

complex  set  of  trade-offs.   Although  this document will

attempt to present some of the  considerations  involved  in

this  evaluation,  you should enlist the help of a qualified

network analyst in applying them to your situation.

The "standard"  2020  configuration  has  extremely  limited

backplane  space.  It is not possible to install DMR11s in a

maximally configured 2020 system  without  the  purchase  of

additional  hardware.  Installing DMR11s in a smaller system

may limit future expansion.

We strongly urge any site considering installing this option

to  consult  with  their  local  Field  Service and Software

Services representatives prior to making any final decision.

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 2

3.0  ADVANTAGES

The  DMR11  provides  the  following  advantages  over   KDP

(KMC/DUP) synchronous interfaces:

o  The DMR11 supports V.35,  RS-422-A,  RS-423-A,  and

coaxial cable interfaces, in addition to RS-232-C.

o  The coax DMR11 can operate over a distance of up to

6 km (19,685 feet) without an external modem.

o  The DMR11 will operate in half-duplex  mode.   Half

duplex  mode  is  supported  by  the  software  for

DECnet, ANF-10, and Program use.

o  The DMR11 will operate over  a  switched  (dial-up)

line, and will provide the necessary modem control.

o  The DMR11 runs self diagnostics upon startup.   The

monitor   will   not   use   a   DMR   that   fails

microdiagnostics.

o  The  DMR11  does  all  DDCMP  protocol   (including

retransmissions)  without  help  from the KS10 CPU.

This results in slightly lower overhead.

o  The  DMR11  can  have   more   than   two   buffers

outstanding at any one time.

o  The DMR11 does not require  a  user-mode  microcode

loader.

4.0  PERFORMANCE CONSIDERATIONS

Preliminary testing indicates  that  the  performance  of  a

DMR11  on a 2020 is approximately the same as that of a KDP.

Thus, although the  DMR11  hardware  is  capable  of  higher

speeds  than  the KDP, the DMR should not be viewed as a way

of getting higher network thruput from a 2020.

Setting the speed of a DMR link higher  than  the  2020  can

accept  data  will  cause  retransmissions,  and if taken to

extremes, may cause the other node to declare the link down.

Such  retransmissions  cause  unnecessary  Unibus traffic on

both nodes, and unnecessary KSBus traffic on the 2020.  Such

traffic  can impact the performance of other applications on

the system(s).

Although there is no experimental  data  supporting  it,  we

expect  that the DMR11 may perform significantly better than

a KDP over links which have a high  error  rate,  and  which

pipeline   effectively.   (eg.   Satellite  links)  This  is

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 3

because  the  format  of  network  messages  is  changed  to

accomodate  the  KS10  UBA whenever a message is given to an

interface for transmission.   This  is  a  fairly  expensive

operation,  and  is  one  of  the limits of a 2020's network

performance.  Because the DMR holds  the  buffer  containing

the  external  form  of  the message until it has been DDCMP

ACK'd, the DMR's retransmissions  have  lower  cost  than  a

KDP's.   A  DMR can also manage up to 64 outstanding buffers

(if you can afford the core).

5.0  HARDWARE OPTIONS

The DMR11 used for a 2020 is  the  same  as  that  used  for

PDP-11  and  VAX  applications.   The  basic DMR11 subsystem

(DMR11-AD)  includes  the  microprocessor  and   line   unit

modules,   documentation,  test  connectors,  and  a  module

interconnect cable.  Customers should NOT order the DMR11-AD

option.   Instead,  one of the four DMR11 packages should be

ordered, depending on the  interface  type  desired.   These

are:

o  DMR11-AA - RS-232-C/RS-423-A

The  RS-232-C/RS-423-A  distribution  panel  mounts

inside   the   KS10  cabinet.   Connection  to  the

external modem  requires  purchase  of  a  BC05D-25

cable  for RS-232-C modems, or a BC55D-33 cable for

RS-423-A modems.

o  DMR11-AB - CITT V.35

This package includes a 25  foot  (BC05Z-25)  cable

that  terminates  in  a  37  pin  DataPhone Digital

Service (DDS) connector.

o  DMR11-AC - Integral modem (Coax)

Interconnecting two DMRs requires  TWO  cables  for

full  duplex operation.  Only one cable is required

for half duplex.  For  2020  data  rates,  purchase

BC55N-98  twinax  cable.   (98  feet  long)  Longer

cables can  be  purchased  on  special  order  thru

Digital's Accessories and Supplies group.

o  DMR11-AE - RS-422-A

The RS-422-A distribution panel mounts  inside  the

KS10  cabinet.   Connection  to  the external modem

requires purchase of a BC55D-33 cable.

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 4

6.0  HARDWARE INSTALLATION

6.1  Backplane Requirements

Each DMR11 requires 2 hex-height SPC slots on  UBA  3.   The

DMR11  can co-exist with KDP lines.  A fully configured 2020

has  no  free  hex-height  slots.    Reasonable   trade-offs

include, but are not limited to:

o  1 DMR and no KDP lines.  Use slots 2 and 4.

o  2 DMRs and no KDPs.  Use slots 2,3,4, and 5.   This

implies  that no more than 24 terminal (DZ11) lines

exist.

Of course, it is possible to add a DNHXX-AA/AB expansion box

to  hold  the  DMRs.  In this case, you must order a DD11-DK

backplane, as the expansion box comes empty.  The DNHXX will

be  mounted  to  the left of the current CPU box to keep the

Unibus  length  within  specifications.   This  may  require

changing Massbus cables to disks or tapes.

6.2  Power Requirements

Each DMR11 requires 8 Amps of +5 volts, .1A  of  +15  Volts,

and  .2A  of -15 Volts.  The BA11-K mounting boxes which are

used in the KS10 have  a  capacity  of  50A  of  +5V  (split

between  2  25A  regulators),  4A  of +15V, and 10A of -15V.

Field Service must verify that any DMRs added to a system do

not  require  more  power  than is available in the box, and

that the power capacity of the backplanes  involved  is  not

exceeded.  (A DD11-DK can power no more than three DMR11s.)

6.3  Unibus Loading

Each DMR11 represents 1 Unibus DC  load,  and  5  Unibus  AC

loads.

6.4  Address/Vector/Priority

The KS puts DMR11s into the Unibus "user" address  space  in

the IO page.  DMR addresses are assigned starting at 764000,

and proceed sequentially in increments of 10  for  each  DMR

installed.   DMR vectors start at 610.  DMRs are expected to

operate at their default BR level, 5.

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 5

6.5  Installation Considerations

The information presented here  is  intended  to  supplement

that  presented  in  the  DMR11 Technical Manual.  It is not

complete,  and  should  be  used  in  conjunction  with  the

instructions in chapter 2 of that manual.

Backplane  voltages  should  be  checked  with  a  DMM,  and

adjusted  if necessary both before and after installation of

each DMR11.  Both slots of each DMR should be  checked.   +5

is on C1A2, +15 is on C1U1, and -15 is on C1B2.

The NPG backplane jumper must be removed from CA1-CB1 of the

backplane   slot  used  for  the  microprocessor  (M8207-RA)

module.

Caution

Failure to remove this  jumper

may  damage the M8207, as well

as result in incorrect  system

operation.

6.6  Jumpers and Switches

6.6.1  M8207 (Microprocessor Module) -

The "normal" configuration  from  the  technical  manual  is

correct, except with respect to addressing.  The KS does not

use floating addresses for the DMR.  See above.

Thus, W1, W2 are installed.  E28-7 is ON, E28-8 is OFF,  and

E85-1 is ON.

6.6.2  M8203 (Line Unit Module) -

Jumpers should be exactly as specified in table 2-8  of  the

technical  manual.   Note  in  particular that W17 should be

OUT, even for "half duplex" operation.  Some DMRs have  been

known to arrive mis-jumpered, so check!

Switch pack E39 - set per customer requirements.

Switch pack E121 - SW1-8 should all be ON.  SW9-10 should be

ON,  unless  a DMC is on the other end of the link, in which

case SW9 is OFF and SW10 is ON.

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 6

Switch pack E134 - SW1-8 should all be OFF.  SW10 should  be

OFF.   SW9  should  be  ON  for  non-switched lines (eg null

modems, leased lines), OFF otherwise (dial-up).   Except  in

unusual  cases,  everything  will  work with SW9 OFF even on

non-switched lines.

For DMR11-AAs, refer to table 2-13 (p 2-27) for distribution

panel jumper selections.

For DMR11-ACs, Make sure that the terminators are  installed

per  figure  2-7  or  2-8  (p  3-35/6).   Note that only one

terminator is installed per cable, and that it goes  at  the

RCV  end.   Also  ensure  that  the  HDX/FDX  switch  is set

correctly on the distribution panel.

6.6.3  Ribbon Cables And Berg Connectors -

All of the ribbon cables and other Berg connectors in a  DMR

can be inserted two ways.  ONLY ONE OF THEM WORKS.  Here are

a few hints on getting it right the first time:

o  The ribbon cable that goes between  the  line  unit

and  the  microprocessor modules has no twists when

installed correctly.  The cable is  marked  on  the

smooth  side at each end with AA and VV.  Make sure

that the AA edge of the cable is inserted into  the

AA  end  of  the  module connector, and that the VV

edge of the cable is inserted into the  VV  end  of

the module connector.

o  The  Ribbon  cables  that  go  from  the  M8203  to

RS232/RS442   or   RS423  distribution  panels  are

similarly marked.  But note that  the  Ribbed  side

faces out.

o  The COAX cables  that  go  to  the  Integral  Modem

(DMR11-AC)      distribution      panel     install

counter-intuitively;  when correctly inserted,  the

cable  from  the connector runs toward the FRONT of

the BA11 box and  is  squeezed  tightly  along  the

M8203  edge.   The  plastic tab faces away from the

module.

6.6.4  Mounting Distribution Panels -

The DMR11-AA/AB/AE (eg all but V.35) all run a ribbon  cable

from  the  M2803  to a metal distribution panel.  For proper

operation, the distribution panel must be properly  grounded

to the frame of the 2020 cabinet.  Scrape both the panel and

the cabinet rail, and use lockwashers liberally.  Also, make

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 7

sure  that  the  spot  you  pick  will  allow  clearance and

sufficient cable slack for the BA11 box to slide in/out  and

rotate  for  service,  and  that  the rear cabinet door will

close with connectors inserted into the panel.

6.6.5  Site Management Guide -

Installation of DMR11(s) on a 2020 system  is  not  complete

until  the site management guide has been updated to reflect

the new hardware and software.

The following is recommended:

o  General Information Page

Add an item stating that unsupported DMR11 hardware

is installed.

o  Bus Layout/Configuration

Add the new device(s) to the bus  in  their  proper

locations.  Add the BR, CSR, and vector assignments

both here and on the label on top of the BA11 box.

o  System and Diagnostic Software

Insert a copy of this  document,  and  the  console

output   of   a  complete  successful  run  of  the

diagnostics.    Include   the   location   of   the

diagnostics.

7.0  DIAGNOSTICS

7.1  Availability

Diagnostics for the DMR11 are  available  to  Digital  Field

Service personnel from the DMR11 TOPS-10 developer.

7.2  Restrictions

The 2020 DMR11 diagnostics run in PDP-11 compatibility mode.

Certain  diagnostics  (noted  below) will incorrectly report

failures.

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 8

7.3  Reference Documentation

Information on operation of the  DMR11  diagnostics  can  be

found  in the DMR11 Synchronous Controller Technical Manual,

and in the Maintenance Documentation Service microfiche set.

7.4  Operation

The DMR11 diagnostics are loaded from a TOPS-10 system  pack

by  MONBTS.   They  are  provided  as PDP-10 .EXE files, and

operate under a special version of DDT11.

Five diagnostics are provided.  These are:

o  ZDMPB0 - M8207 Static Diagnostic #1

This  diagnostic  is  equivalent  to   the   PDP-11

diagnostic CZDMPB0.  It performs basic tests of the

microprocessor, and should be the first  diagnostic

run.

o  ZDMQB0 - M8207 Static Diagnostic #2

This  diagnostic  is  equivalent  to   the   PDP-11

diagnostic  CZDMQB0.  It performs extended tests of

the  microprocessor,  and  should  be  the   second

diagnostic run.

You should note that the normal position of the Run

switch  (SW7  E28) is ON, while the default for the

diagnostic's prompt is OFF.  Tests 9,  27,  and  42

will   fail  if  the  run  switch  is  ON  and  the

diagnostic is not told.

Test 31 will usually fail even with a  good  M8207;

it  is  dependent  on CPU timing, and a KS is not a

PDP-11.  The expected error is "ERR 100 CLOCK  TIME

TOO FAST".

Test 32 will also fail;  the KS  does  not  support

remote  load  detect.  The expected errors are "ERR

17 FORCE POWER FAIL ERROR" and "ERR 35 FORCE  POWER

FAIL ERROR".

o  ZDMRD0 - M8203 Static Diagnostic #1

This  diagnostic  is  equivalent  to   the   PDP-11

diagnostic CZDMRD0.  It performs basic tests of the

line unit.  It assumes that the  microprocessor  is

in  working  order,  since it is dependent on it to

access the M8203;  the M8203 is not accessible from

the UNIBUS.

KSDMR.BWR - Beware file for KS DMR (non-)support      Page 9

The microprocessor Run switch (SW7  E28)  can  also

cause test failures if set incorrectly;  however in

this case, the default is correct.

o  ZDMSD0 - M8203 Static Diagnostic #2

This  diagnostic  is  equivalent  to   the   PDP-11

diagnostic  CZDMSD0.  It performs extended tests of

the line unit.  It should be run after ZDMRD0  runs

successfully.

This diagnostic  is  most  useful  if  you  install

turnaround  connectors  someplace;   the diagnostic

allows for 4 places to put  them.   One  should  be

suitable for your site.

The diagnostics asks  for  the  contents  of  three

M8203  registers  that  are  derrived  from  switch

settings.  If you change the SW  P-table,  you  can

get  test 11 to print them for you.  Of course, you

still get to check that what it prints  corresponds

to what you intended to set.

o  ZDMIC0 - DMR11 Functional Tests

This  diagnostic  is  equivalent  to   the   PDP-11

diagnostic CZDMIC0.  It performs functional testing

of the DMR11 subsystem, and should be run after the

static diagnostics.

Test 2 will print out the  revision  level  of  the

DMR11 CROMs.

Test 10 will generally fail even with a good DMR11.

It  depends  on  CPU timing.  The expected error is

"ERR 19 M8207 PROGRAM TIMER OUT OF RANGE".

7.5  Loading and Starting Diagnostics

The DMR11 diagnostics are started  in  the  same  way  as  a

monitor is.

Type Control/Backslash (\) to get the KS10> prompt.  Then BT

to  load  BOOT.   Type  DSKB:diagnostic[6,6]<CR> to load and

start DDT11.  When you get the "Input: " prompt,  enter  the

magic  string  "/UBA:3/LA36/PATCH/GO".   Typing  errors  may

cause alarming failures;  if they do, start over.  When  you

get  a  prompt  of  a  blank  line,  type 200<ESC>G to start

execution.  From this point on, the dialog is  the  same  as

that for the PDP-11 diagnostic.

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 10

For  example,  the  following  shows  loading  and  starting

ZDMRD0:

^\

KS10>BT

KS10>USR MOD

BOOT V1(4)

BOOT>DSKB:ZDMRD0[6,6]

[Loading from DSKB:ZDMRD0[6,6]]

DDT11 5(55)-3

Input:  /UBA:3/LA36/PATCH/GO

200$G

beginning execution @pc = 200

DIAG. RUN-TIME SERVICES-D-0

M8203 STATIC LOGIC TESTS - PART 1 OF 2

INIT IS M8203

50 HZ (L) N ?

LSI (L) N ?

DR>STA/PASS:1/FLAGS:PNT

# UNITS (D) ? 1

UNIT 0

DEVICE CSR ADDRESS : (O) 164000 ?

DEVICE VECTOR ADDRESS : (O) 610 ?

DEVICE PRIORITY LEVEL : (O) 5 ?

M8207 RUN SWITCH (E28 SW7) - TYPE 0 IF OFF, 1 IF

ON : (O) 1 ? 1

TST:001

TST:002

...

TST:042

CZDMR EOP   1

0 CUMULATIVE ERRORS

DR>

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 11

8.0  SOFTWARE INSTALLATION

8.1  Rebuild MONGEN

Building a monitor with DMR support requires that MONGEN  be

re-assembled with FTUNSUPPORTED turned on.

8.2  HDWGEN differences

When FTUNSUPPORTED is turned on,  the  dialogue  differs  in

many   places   from   the  docunentation  in  the  Software

Installation  Guide.   Only  the  DMR-specific  changes  are

discussed here.

Following the section on KDPs, a new section  on  DMR  lines

appears.

#  DMR11  Lines(0,0-8)[Number  of  DMR11  synchronous   line

controllers.]:

Reply with the number of DMR11s installed in your 2020.

If a DMR is not found at run time, a message is issued,

but the system will function without it.

The following questions are repeated for  each  DMR  in  the

configuration:

DMRn line type(ANF10,DECNET,USER,IBM):

Reply with a one of the listed  keywords.   Your  reply

indicates  which of the protocols listed is allowed use

of this DMR.  USER allows you to OPEN the  DMRn  device

and run your own protocol.  IBM has never been tried.

DMRn switched line(Y,N)[

Does DMRn control a switched (dial-up) line]:

Reply "Y" or "N" as appropriate.   Switched  lines  are

sensitive  to  DSR glitches.  Non-switched lines ignore

them.  If you answer "N", the remaining  questions  are

skipped, as they only apply to switched lines.

DMRn half duplex(Y,N)[

Does DMRn control a half duplex line]:

Reply "Y" or "N" as appropriate.  "Y" causes the DMR to

be  initialized in half-duplex mode.  This setting must

be consistent at both ends of the  physical  link.   If

you  answer  "N", the remaining question is skipped, as

it only applies to half duplex lines.

Is DMRn secondary(Y,N)[

Exactly one end of the link should be a secondary station.

Secondary status controls link startup only]:

Half duplex DDCMP requires that  the  start  timers  on

each  end  of  a  physical  link  differ  in  order  to

guarantee that a  START  message  can  get  thru.   "Y"

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 12

causes  a  three second timer to be used.  "N" causes a

one second timer to be used.  DMC11s use a  one  second

timer.   The timer value must be DIFFERENT at the other

end of the physical link.

8.3  Changing The Number Of Buffers Assigned

Each DMR is assigned static buffer  space  in  COMMON.   Two

variables  in  NETPRM.MAC  determine  how  many  buffers are

available to each DMR, and how much memory is allocated.  In

general, the defaults should be adequate.  But for satellite

links, for example, you may want to increase them.

DR%NBF is  the  number  of  buffers  assigned  to  each  DMR

channel.   Each DMR has two channels (transmit and receive),

so each DMR uses 2*DR%NBF buffers.  DMRPPL is the number  of

pages  to  be  allocated  per  DMR  line.   These  pages are

permanently allocated in COMMON, and are used as DMR scratch

and buffer space.  The exact formulae for allocation of this

space may be found in NETPRM  and  DMRSER  (D8RINT.MAC).   A

reasonable  rule  of  thumb for the current software is that

increasing DR%NBF by one costs about  700(8)  words  of  DMR

buffer space.

If increasing DR%NBF will make the maximum message size  too

small, DMRPPL should be increased as well.  For small values

of DR%NBF, there is a one  for  one  correspondence  between

increasing DR%NBF and DMRPPL.

8.4  Monitor Build

The  rest  of  the  monitor  assembly  and  build   proceeds

normally.  You should assemble D8RINT.MAC in some convenient

place (after the universal files) , and  include  D8RINT.REL

in the LINK command string someplace after MONBTS and before

TOPS10.REL.

8.5  Considerations For Half-Duplex ANF-10 Links

Half duplex links  will  work  with  no  further  effort  to

another  KS  system.   Use  of half duplex links to a PDP-11

based ANF node requires the following (unsupported) patch:

In your C.P11 file, add the following symbol definitions for

each half duplex DMC/DMR line:

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 13

LnHDX=1         ;Makes the line half duplex

LnSEC=x         ;1 makes line "secondary" (3 sec timer)

"n" is the synchronous line number of the line you  want  to

change.  Half duplex will only work with DMC/DMR lines.

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 14

File 1) BLKK:DNCDMC.P11[7,7023]         created: 1555 27-May-82

File 2) DSKP:DNCDMC.702[30,5742,TL]     created: 1540 13-Jul-82

1)5     DMCLB0: .WORD   0                       ;ADDRESS OF FIRST DMC-11 LINE

****

2)5             LSXHDX=B3                       ;LINE IS TO BE RUN IN HDX MODE

2)              LSXSEC=B4                       ;LINE IS A SECONDARY STATION

2)      DMCLB0: .WORD   0                       ;ADDRESS OF FIRST DMC-11 LINE

**************

1)11            BIT     #LSXMAI,LB.STX(J)       ;IF MAINT MODE,

1)              BNE     10$                     ; THE REST IS DONE DIFFERENTLY.

****

2)11            BIT     #LSXHDX,LB.STX(J)       ;WANT TO RUN HDX MODE?

2)              BEQ     5$                      ;NO, NORMAL (FDX) MODE

2)              BIS     #MC.HD,R2               ;YES, SET THE BIT

2)              BIT     #LSXSEC,LB.STX(J)       ;WANT TO USE LONG START TIMER?

2)              BEQ     5$                      ;NO, WE ARE MASTER STATION

2)              BIS     #MC.SEC,R2              ;YES, SET SECONDARY BIT

2)      5$:     BIT     #LSXMAI,LB.STX(J)       ;IF MAINT MODE,

2)              BNE     10$                     ; THE REST IS DONE DIFFERENTLY.

**************

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 15

File 1) BLKK:DNLBLK.P11[7,7023]         created: 1558 27-May-82

File 2) DSKP:DNLBLK.702[30,5742,TL]     created: 1601 15-Jul-82

1)1             .WORD   0                                       ;LB.ST2

1)      .IF NE,LBTOT

****

2)1             .WORD   0                                       ;LB.STX

2)      .IF NE,LBTOT

**************

1)1     .ENDM   X

****

2)1     .IF DF LB.STX

2)          .IF GT DMCN

2)              .IIF NDF,L'Q'HDX,L'Q'HDX=0

2)              .IIF NE,L'Q'HDX,L'Q'HDX=1

2)              .IIF NDF,L'Q'SEC,L'Q'SEC=0

2)              .IIF NE,L'Q'SEC,L'Q'SEC=1

2)              .=.-<LB.SIZ-LB.STX>

2)              .WORD <L'Q'HDX*LSXHDX>+<L'Q'SEC*LSXSEC>

2)              .=<.-2>+<LB.SIZ-LB.STX>

2)          .ENDC

2)      .ENDC

2)      .ENDM   X

**************

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 16

File 1) BLKK:DNDCMP.P11[7,7023]         created: 0856 07-Jul-82

File 2) DSKP:DNDCMP.702[30,5742,TL]     created: 1140 16-Jul-82

1)1             MOV     LB.SLA(J),R3            ;GET THE HARDWARE ACCESS ADDRESS FOR LINE

****

2)1     .IIF DF,LB.STX, CLR     LB.STX(J)       ;NOT DMC, DUPS WANT ST2 CLEAR

2)              MOV     LB.SLA(J),R3            ;GET THE HARDWARE ACCESS ADDRESS FOR LINE

**************

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 17

File 1) BLKK:DNCNFG.P11[7,7023]         created: 0900 07-Jul-82

File 2) DSKP:DNCNFG.P11[30,5742,TL]     created: 1220 18-Jul-82

1)22    X       ZER,0   ;FIRST WORD TO ZERO ON A RESET

1)      .IF NE FTDUP11!DMCN     ;AN EXTRA STATUS WORD FOR THE DUP-11 AND DMC11

****

2)22    .IF NE FTDUP11!DMCN     ;AN EXTRA STATUS WORD FOR THE DUP-11 AND DMC11

**************

1)22    .IF NE,FTDUP11

****

2)22    X       ZER,0   ;FIRST WORD TO ZERO ON A RESET

2)      .IF NE,FTDUP11

**************

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 18

9.0  REFERENCES

The following documents  may  be  consulted  for  additional

information:

o  DMR11 Synchronous Controller Technical Manual

Describes the detailed installation and programming

of   the   DMR11   hardware.    DEC   order  number

EK-DMR11-TM-002.

o  M8203 Line Unit Technical Manual

Describes the operation of the  line  unit  module.

Appendix B contains the technical data for Integral

Modem   (COAX)   cables.     DEC    order    number

EK-M8203-TM-002.

o  KS10-Based DECsystem-2020 Technical Manual

Describes  the  operation  of  the  KS10.    Power,

cabling,   and  Unibus  limitations  are  included.

Also, IO programming and module  utilization.   DEC

order number EK-0KS10-TM-002.

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 19

10.0  PROGRAMMING THE DMR:  DEVICE

The DMR:  device is programmed in a manner  similar  to  the

KDP  and  DTE  devices.   Since  these  devices  may  not be

documented  elsewhere,  this  section   will   provide   the

information necessary to program the DMR:  device.

Caution

The DMR:  device  is  provided

primarily   for  DEC  in-house

use.      The      information

presented  in  this section is

subject  to   change   without

notice.

The DMR:  device is programmed like  any  ordinary  buffered

mode device, with the following exceptions/notes:

10.1  Dynamic DDBs

The  DMR:   device  does  not  have  a  pre-allocated   DDB.

Instead,  the  DDB is built at OPEN time.  Thus, a RESOURCES

command will not show its availibility.  DMR device  support

is  automatically  included  when  DMRs  are  included  in a

MONGEN.

10.2  OPEN and the DMR:  device

There  is  one  DMR:   device  (DMRn:)  available  for  each

physical  DMR  built into the monitor.  A DMR:  DDB can only

be built by an OPEN UUO from a job which has POKE  (JP.POK),

JACCT,  or  [1,2]  privileges.   FILOP., and random commands

will return "no such device", as  will  OPEN  UUOs  from  an

unprivileged job.

10.3  Buffer Size

The DMR:  device will  allow  you  to  use  variable  length

buffers.   The default buffer size is constant for any given

monitor load,  but  varies  as  a  function  of  the  NETPRM

parameters DMRPPL and DR%NBF.  Thus, you should not count on

the default buffer size in any program, unless you obtain it

from a DEVSIZ UUO.

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 20

10.4  IO Modes

Only byte mode (IO.BYT) is  allowed  to  the  DMR:   device.

Byte  mode  has a default byte size of 8 bits.  Asynchronous

IO (UU.AIO set in the OPEN block) is  supported.   The  DMR:

device  supports simultaneous input and output;  no implicit

WAIT UUOs are done by the monitor  which  would  force  half

duplex operation.

10.5  Software Interrupts

The usual software interrupts for asynchronous IO  work  for

the  DMR:   device;   Input  Done, Output Done, Input Error,

Output Error.

When DDCMP  prototocol  comes  up  on  a  line,  an  ON-LINE

interrupt  is  generated.  When DDCMP goes down, an OFF-LINE

interrupt is generated.

10.6  File Status Bits

The following File Status (GETSTS) bits are defined for  the

DMR:  device:

10.6.1  IO.IMP - Set if you attempt to use  a  DMR  that  is

assigned to another protocol (DECnet, ANF10, IBMcomm).  Also

set if a maintenance message is received while normal  DDCMP

is running.

10.6.2  IO.DTE - Set if  you  attempt  IO  to  a  line  that

doesn't  have  DDCMP  running  yet.  (Normal mode only) This

error can occur in any mode if a switched line  hasn't  been

connected  yet.   In  any  case, IO.DTE is a soft error, and

should be retried after a suitable delay.

10.6.3  IO.DER - Set if protocol on the line is (went) down.

Cleared when protocol starts.  Set when protocol goes down.

KSDMR.BWR - Beware file for KS DMR (non-)support     Page 21

10.6.4  IO.BKT - Set if a message is received or transmitted

that  exceeds  the maximum message size allowed.  A received

message that is too large causes the DMR to  shut  down  the

line.

10.6.5  IO.MAI - If set, indicates that the line is  to  run

in  DDCMP  maintenance  mode.   If clear, indicates that the

line is to run in DDCMP normal mode.

10.6.6  IO.SRM (1B28) - Not currently set  by  the  monitor.

If  set,  means  that  a DDCMP START was received by the DMR

while in  maintenance  mode.   This  condition  can  not  be

dectected  by the DMR11.  But if set, this bit is treated as

an error condition.

10.6.7  IO.MRN (1B29) - Not currently set  by  the  monitor.

If  set,  means that a DDCMP MOP message was received by the

DMR while in normal  mode.   Currently,  this  condition  is

reported  by  IO.IMP.  But if set, this bit is treated as an

error condition.

10.7  Errors

Error bits are not cleared automatically by the monitor.  If

any  error  bit  is set, the monitor will refuse to initiate

further IO.  You can clear any of the error bits via OPEN or

SETSTS.