MAKVFU.DOC -- Changes from version 1(17) to version 2(33)

October 1979

THIS SOFTWARE IS FURNISHED UNDER A LICENSE AND MAY ONLY BE USED

OR COPIED IN ACCORDANCE WITH THE TERMS OF SUCH LICENSE.

COPYRIGHT (c) DIGITAL EQUIPMENT CORPORATION 1976,1977,1979,1986.

ALL RIGHTS RESERVED.

MAKVFU.DOC                                                      Page 2

MAKVFU.DOC -- Changes from version 1(17) to version 2(33)

October, 1979

1.0  SUMMARY

This version of  MAKVFU  adds  several  new  commands,  adds  explicit

support for the LP05 and LP14 printers, and fixes a number of bugs.

2.0  EXTERNAL CHANGES

Three new commands have been added to MAKVFU.  The TYPE  command  sets

defaults  for  a  specific  line printer type.  The CLEAR command will

clear all settings for a specific VFU channel.  The NEW  command  will

clear the entire VFU.  ALL three commands are explicitly documented in

the MAKVFU.HLP file.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

None.

5.0  INTERNAL CHANGES

Edit

[18] - lp100 h.w. change - maximum external line no. (mxlinx) =143.

not 144. therefore, max. internal line no. (maxlin) =142.

[19] - lp100 h.w. change - line 1 of channel 1 must be set. changed

.lines and .botom code

[20] - fixed p 144 bug.

[21] - fixed c 1 l 143 bug.   -- also added lp05 info.

[22] - lp100 h.w. change - channel 12 bottom must be defined for

lp05. therefore, maxchl increased to 12. also, .init code

changed to define channel 12 bottom. also, messages m11,12

added to .info.

[23] - inserted warning message in .lpi to warn against

attempting programmable changes to lpi for an lp05.

[24] - fixed getsix to allow 0's in filename

[25] - fixed getdec bug -- 8 will be error for getoct, net terminator

char.

[26] - corrected error msgs to correspond to manual

MAKVFU.DOC                                                      Page 3

[27] - fixed info to always output lp05, lp07 message

[30] - add zero command to clear a channel

and new to clear entire vfu

these are to facilitate non-standard vfu's

[31] - suppress lp05 warning in lpi 6 9 mar 1977

[32] - add type command

lp05 lpi illegal, set channel 12 bot

lp07

other

[33] - add lp14 type. same as lp05.

[End of MAKVFU.DOC]

[VFU1.DOC is appended as an integral part of MAKVFU.DOC]

VFU1.DOC                                                        Page 4

VFU1.DOC -- V1(23)

January 1977

1.0  SUMMARY

MAKVFU provides support for the programmable  Direct  Access  Vertical

Format  Unit (DAVFU) feature on the LP100/LP07 Line Printer subsystem.

This feature relieves the computer operator from  the  time  consuming

manual  installation  of the proper vertical format using a paper tape

reader.  It also reduces the risk of running  a  print  job  with  the

incorrect vertical format.

The MAKVFU program produces a binary VFU file.   This  file  describes

the   vertical   formats   for  8  separate  channels.   Each  channel

corresponds to a  particular  non-printable  ASCII  character.   These

characters are called vertical control codes, and are related to the 8

channels as follows:

Channel        Character           ASCII Value(octal)

1              Form Feed           14

2              Device Control 0    20

3              Device Control 1    21

4              Device Control 2    22

5              Device Control 3    23

6              Device Control 4    24

7              Vertical Tab        13

8              Line Feed           12

The MAKVFU commands permit the user to define  special  print  formats

for  each  of  the  8  channel/character  pairs.  These characters are

imbedded into text files sent  to  the  line  printer  to  format  the

printer output.

LPTSPL V102 (in GALAXY release 2)  will  automatically  load  a  DAVFU

based  on  a switch in the LPFORM.INI file.  The procedure for setting

this up will be documented in the  GALAXY  operator's  manual.   Until

this  software  ia  available,  the  DAVFU  can  be loaded with PIP by

typing:

.R PIP

*LPTn:=vfufile/I

VFU1.DOC                                                        Page 5

In order for this to work however, the following patch must be applied

to the 6.03 lineprinter service routine (LPTSER):

change:

LPTREL:  MOVEI U,551000

to:

LPTREL:  MOVEI U,541000

(i.e.  $Q-10000)

This is the first  release  of  MAKVFU.   This  release  has  no  know

deficiencies.   It  was developed under 6.03 and tested under 6.01 (as

of 7/9/76).

MAKVFU is documented in the following publications:

LP100 Software Installation Guide

MAKVFU.HLP

MAKVFU.DOC

2.0  EXTERNAL CHANGES

Not applicable for a new release.   See  LP100  Software  Installation

Guide for a full description of the user interface.

3.0  KNOWN BUGS AND DEFICIENCIES

There are no known deficiencies.

4.0  INSTALLATION INSTRUCTIONS

See chapter 2 of the LP100 Software Installation Guide for a  complete

description of the installation instructions.

5.0  INTERNAL CHANGES

Not applicable for a new release.

[End of VFU1.DOC]