CHAPTER 1

FLOPPY DISKETTES (RXA:)

Diskettes  are  block-addressable  devices,  each  of  which

contains 77 tracks of 26 sectors each.  No file structure is

defined by the TOPS-10 monitor, although it is possible  for

a user-mode program to define a file structure.

Diskettes  may  be  recorded  in  either  single  or  double

density.   Single  and  double density may not be mixed on a

single diskette.  Single density diskettes record 64  16-bit

words  per  sector.   Double  density  diskettes  record 128

16-bit  words  per  sector.   The  diskette's   density   is

determined  by  the  monitor during each IN, INPUT, OUT, and

OUTPUT  UUO,  and  the  file  status  (IO.DEN)  is   updated

accordingly.   This  allows  your  program  to determine the

density  of  a  diskette  by  simply  reading  a  block  and

examining the resultant file status.

Only 32 bits of each 36-bit  data  word  in  your  program's

buffer  are  read or written.  Bits 0, 1, 18, and 19 are not

transferred and are indeterminate.  Bits 2  through  17  are

transferred first.

The monitor provides two  basic  addressing  modes  for  the

diskette.   Your  program specifies which addressing mode is

used for each IO operation in the file status word.

In logical addressing mode,  several  physical  sectors  are

grouped to form a logical block, which is transmitted with a

single IN, INPUT, OUT, or OUTPUT UUO.   Logical  blocks  are

numbered sequentially from zero.  Physical track zero is not

accessible in logical addressing mode because it is reserved

for  potential  ANSI  standardization.  The physical sectors

comprising each logical block are not contiguous;  they  are

assigned   in   such   a  manner  as  to  optimize  hardware

performance.  The monitor provides  two  logical  addressing

modes;  one (Logical-11) is compatible with Digital's PDP-11

17-September-1984 15:17:21

1-1

FLOPPY DISKETTES (RXA:)

software, the other (Logical-8) is compatible with Digital's

PDP-8 software.  When in Logical-11 mode, 4 (single density)

or 2 (double density) sectors are  grouped  to  form  a  128

(36-bit)   word   logical   block.    The   sector-to-sector

interleave factor is 2;  the track-to-track skew is 6.  When

in  Logical-8  mode,  3 physical sectors (of either density)

are grouped to form a 96 (single  density)  or  192  (double

density) 36-bit word block.  The sector-to-sector interleave

factor is 3;  the track-to-track skew is 0.  Physical tracks

74  thru  77 are not available when in Logical-8 mode;  they

are reserved for use by diagnostics.

In physical addressing mode, a single sector  of  32  or  64

36-bit  words  (depending  on  the  diskette's  density)  is

transmitted by each IN, INPUT,  OUT,  or  OUTPUT  UUO.   All

sectors,  including  track zero are accessible.  The sectors

are sequentially numbered starting with zero.

Accessible tracks and buffer size

as a function of IO mode

Buffer size      Contents

Density Addressing Tracks   36-Bit / 8-Bit   Data words

------- ---------- ------ ------------------ ----------

Single  Physical     77   32 words/128 bytes  64 16-bit

Single  Logical-8    74   96 words/384 bytes 256 12 bit

Single  Logical-11   76  128 words/512 bytes 256 16-bit

Double  Physical     77   64 words/256 bytes 128 16-bit

Double  Logical-8    74  192 words/768 bytes 512 12-bit

Double  Logical-11   76  128 words/512 bytes 256 16-bit

Table 1

Note that, except in Logical-11 mode, incorrectly specifying

the density of a diskette may cause an unintentional address

check due to the differing buffer sizes.

Up to two  RX20  controllers  are  supported  per-CPU.   The

controller  names  are  RXA  and  RXB.   Each controller can

support up to two RX02 drives.   Thus,  a  system  with  the

maximum  number  of controllers and drives would include the

device names RXA0, RXA1, RXB0, and RXB1.

The diskette hardware provides the ability for  each  sector

to  be  marked  "deleted" via use of a special "deleted data

mark" which the hardware can  create  and  detect.   TOPS-10

allows  your program to create and detect deleted data marks

by manipulating the diskette's file status word (IO.PAR).  A

sector  which  is flagged with a deleted data mark may still

1-2

FLOPPY DISKETTES (RXA:)

contain valid data;  the presence of  a  deleted  data  mark

does  not  alter the capacity of a sector.  Every OUTPUT UUO

executed with IO.PAR set in the file status word will  cause

a  deleted  data  mark  to  be written with the data;  every

OUTPUT UUO executed with IO.PAR clear  in  the  file  status

word  will  write only the data, and will erase any previous

deleted data mark.  No Digital operating system makes use of

deleted data marks.  However, some other vendors use them to

signify that a sector is available for use.  Programs  which

write  diskettes for interchange with these vendors will use

deleted data marks as prescribed by  the  vendor.   Programs

which  write  diskettes for interchange with Digital systems

should not write deleted  data  marks.   Note  that  use  of

deleted   data  marks  as  a  storage  allocation  technique

requires your program to read a potentially large number  of

sectors  until  a  deleted  data  mark  is encountered, then

rewrite the sector with  new  data.   In  contrast,  Digital

systems  use  a bit table or directory structure to allocate

storage.  This approach allows a sector to be allocated  for

new  data  by  simply  checking  the  bit table or directory

(usually a few sectors in size) without an  exhaustive  scan

of the disk.

1-3

FLOPPY DISKETTES (RXA:)

A floppy diskette may be operated  only  in  buffered  image

data  mode  (.IOIMG).  The data mode must be specified by an

OPEN UUO, and preserved during any subsequent  SETSTS  UUOs.

The  file  status bits (which are read by GETSTS, and set by

OPEN and SETSTS UUOs) provide additional  control  over  the

diskette.  The following bits are defined for diskettes:

IO.DER  Device error.   Set  when  an  error  occurs

during   an   IO   operation  which  is  not

attributable to media.

IO.DTE  Data error.  Set when  a  media-attributable

error,  such  as  a CRC (data parity) error,

occurs during an IO operation.

IO.BKT  Block number too  large.   Set  when  IO  is

attempted  to  a  block  number which is too

large for the selected device/density.

IO.WPS  (Same  as  IO.BOT)  Encodes  the   type   of

interleaving  desired  when using logical IO

mode.   When  set,  specifies   that   PDP-8

compatible  mode  is  desired.   When clear,

specifies that  PDP-11  compatible  mode  is

desired.   It  is illegal to set IO.WPS when

using physical IO mode.

IO.PAR  Set by the monitor when a deleted data  mark

is  encountered  during  an input operation.

Cleared by the monitor when a  deleted  data

mark  is  not  encountered  during  an input

operation.  Set by your program prior to  an

output  operation  to  cause  a deleted data

mark  to  be  written   with   each   sector

transferred.

IO.DEN  This two-bit field encodes the  density  and

addressing  mode  for the diskette.  Setting

the  high  order  bit   specifies   physical

addressing  mode.   Clearing  the high order

bit  specifies  logical   addressing   mode.

Setting  the  low order bit specifies double

density mode.  Clearing the  low  order  bit

specifies single density mode.

IO.SIM  When set, specifies that  2-sided  media  is

present in the drive.  When clear, specifies

that 1-sided media  is  present.   Like  the

density  bit of IO.DEN, IO.SIM is updated by

the monitor during each transfer.  IO.SIM is

also used to determine the maximum number of

blocks which the device can store.  (2-Sided

media    has    twice    the    number    of

1-4

FLOPPY DISKETTES (RXA:)

blocks/diskette  for  any  given   density.)

Digital  hardware does not currently support

2-sided media.

1-5

FLOPPY DISKETTES (RXA:)

Floppy diskettes respond to the following UUOs.   Except  as

noted,  the  calling  sequence  is the same as for any other

device.

DEVSIZ     Returns the default buffer  size  and  number  of

buffers   for   a   given  IO  mode.   To  ensure

compatibility with future  releases,  always  use

DEVSIZ  when allocating your own buffers.  (Don't

"hardwire" buffer sizes.)

FILOP.     IO functions perform the same  functions  as  the

individual UUOs listed here.

OPEN       Assigns a floppy diskette to a channel.  See  the

file  status bits (above).  Note that the initial

file  status  bits  specified  by  an  OPEN   UUO

determine the size of buffers built by subsequent

INBUF, OUTBUF, IN, INPUT, OUT,  or  OUTPUT  UUOs.

As  a result, if you plan to use more than one IO

mode while a channel is OPEN, it  is  recommended

that  you  set  the initial file status to the IO

mode wich requires the largest buffer size.  This

will ensure that the largest possible buffers are

built by the monitor, and will prevent  later  IO

from     over-writing     the    buffers'    end.

Alternatively, you  can  build  your  own  buffer

rings.

GETSTS     Reads the current file status word.

IN,INPUT   Reads one block or sector from the diskette.

OUT,OUTPUT Writes one block or sector to the diskette.

SETSTS     Sets the current file status word.

USETI      Specifies which block or sector  is  to  be  read

next.    If   USETI   is   not  used,  the  first

block/sector read  is  zero.   Subsequent  IN  or

INPUT  UUOs  will  read  the  next  consecutively

numbered block/sector.

USETO      Specifies which block or sector is to be  written

next.    If   USETO   is   not  used,  the  first

block/sector written is zero.  Subsequent OUT  or

OUTPUT  UUOs  will  write  the next consecutively

numbered block/sector.

CLOSE      flushes the buffer rings normally.

RELEAS     Performs a CLOSE (if necessary), then disconnects

the channel from the device.

1-6

FLOPPY DISKETTES (RXA:)

In addition to the UUOs listed above, the  following  DEVOP.

functions also control diskettes:

I.      .DFRXS (=1)     Read extended status

NOTE

This DEVOP.  function provides

access     to    a    hardware

maintenance function which may

be useful to the programmer in

limited circumstances.  For  a

description  of  the  returned

status  words,  see  the  RX02

Technical  Manual  (SDC  order

number EK-0RX02-TM-001).

Calling sequence:

MOVE    AC,[XWD 4,addr]

DEVOP.  AC,

error return

Normal return

addr:   EXP     .DFRXS          ;function code

/ EXP     UDX

<  SIXBIT  /devnam/        ;device specification

\ EXP     channo

BLOCK   2               ;buffer for returned hardware status

;see the RX02 Technical Manual

Returns the following values in AC:

Normal return:  IO status for device (GETSTS)

Error return:   Reason for failure; one of:

DFACS%  Address check (The argument list is not in user memory)

DFPRV%  Device active (Operation illegal when device is active)

DFNLR%  Value error (Format code was not SINGLE or DOUBLE)

DFNIA%  Not initialized (IO.DER or IO.DTE set in file status)

DFDOL%  Device offline

1-7

FLOPPY DISKETTES (RXA:)

II.     .DFFMT (=11)    Format a diskette

Calling sequence:

MOVE    AC,[XWD 3,addr]

DEVOP.  AC,

error return

Normal return

addr:   EXP     .DFFMT          ;function code

/ EXP     UDX

<  SIXBIT  /devnam/        ;device specification

\ EXP     channo

/ SIXBIT  /SINGLE/        ;to format the diskette in single density

\ SIXBIT  /DOUBLE/        ;to format the diskette in double density

Returns the following values in AC:

Normal return   IO status for device (GETSTS)

Error return    Reason for failure.  See .DFRXS for error codes.

Diskettes  purchased  from  Digital  are  pre-formatted  for

either  single  density (RX01) or double density (RX02) use.

This DEVOP.  allows  you  to  format  diskettes  from  other

vendors,  and  allows  you  to  reformat  a  double  density

diskette for single density use, and  viceversa.   Diskettes

purchased  for  single  density  use  may  not be usable for

double-density applications.  After formatting  a  diskette,

we  strongly  recommend reading the entire disk, and mapping

any errors.  A format operation takes about  15  seconds  to

complete,  and  destroys  any  data  previously on the disk.

Should a power failure, hardware  malfunction,  or  operator

action  interrupt  a format operation, a diskette may result

which contains address marks  of  both  densities.   Such  a

diskette  is  illegal, may result in improper operation, and

should be reformatted correctly.

1-8

FLOPPY DISKETTES (RXA:)

III.    .DFHCW (1002/2002) Read/Set the error counter

Calling sequence:

Read:   MOVE    AC,[XWD 2,addr]

DEVOP.  AC,

error return

Normal return   ;error counter stored in AC

addr:   EXP     .DFHCW          ;function code

/ EXP     UDX

<  SIXBIT  /devnam/        ;device specification

\ EXP     channo

Set:    MOVE    AC,[XWD 3,addr]

DEVOP.  AC,

error return

Normal return   ;AC is not changed

addr:   EXP     .DFHCW+.DFSET   ;function code

/ EXP     UDX

<  SIXBIT  /devnam/        ;device specification

\ EXP     channo

EXP new value           ;(usually zero)

Error returns:  AC contains an error code.

DFACS%  Address check (argument list not in memory)

DFPRV%  No privileges (set)

DFNLR%  Value out of range (argument list too small)

DFNXD%  Nonexistant device

Allows any program to read, and privileged programs  to  set

the  error  statistics word maintained by the monitor.  This

word has the format:

_0___________________17_18___________________35_

|Number of hard errors | Number of soft errors !

------------------------------------------------

The monitor increments the number of soft  errors  once  for

each  IO  operation  which  did  not  succeed  on  the first

attempt, but did succeed during  monitor-initiated  retries.

The  monitor  increments  the number of hard errors once for

each IO operation  which  was  not  successful,  even  after

retrys.   A  given  error  is either a hard error, or a soft

error;  no error is counted twice.

1-9

FLOPPY DISKETTES (RXA:)

The error counter may be of use to a program which wishes to

move  data  from  a  sector which has a large number of soft

errors to a new sector, before the original  sector  becomes

unreadable.  A program may also wish to inform the user when

a diskette should be replaced.

If a nonprivileged program wishes to use  error  information

based  only on its use of a diskette, it must read the error

counter  when  it  starts  operation,  and  again  when   it

finishes.   The  difference  between the two values reflects

that program's use.  Privileged programs  may  simply  clear

the error counter when they start.  The distributed software

never resets the error counter, and the monitor makes no use

of  the statistics accumulated.  However, some installations

may reset the error counter on a MOUNT command, and log  the

value  on  DISMOUNT  in  order  to  maintain  a record of an

individual floppy's performance.

1-10