COPYRIGHT (c) DIGITAL EQUIPMENT CORPORATION 1979,1981,1984,1986.  ALL RIGHTS RESERVED.

THIS SOFTWARE IS FURNISHED UNDER A LICENSE AND MAY BE USED AND COPIED

ONLY  IN  ACCORDANCE  WITH  THE  TERMS  OF  SUCH LICENSE AND WITH THE

INCLUSION OF THE ABOVE COPYRIGHT NOTICE.  THIS SOFTWARE OR ANY  OTHER

COPIES THEREOF MAY NOT BE PROVIDED OR OTHERWISE MADE AVAILABLE TO ANY

OTHER PERSON.  NO TITLE TO AND OWNERSHIP OF THE  SOFTWARE  IS  HEREBY

TRANSFERRED.

THE INFORMATION IN THIS SOFTWARE IS SUBJECT TO CHANGE WITHOUT  NOTICE

AND  SHOULD  NOT  BE  CONSTRUED  AS A COMMITMENT BY DIGITAL EQUIPMENT

CORPORATION.

DIGITAL ASSUMES NO RESPONSIBILITY FOR THE USE OR RELIABILITY  OF  ITS

SOFTWARE ON EQUIPMENT WHICH IS NOT SUPPLIED BY DIGITAL.

Page 2

Table of Contents

1.0     ABSTRACT . . . . . . . . . . . . . . . . . . . . . . 3

2.0     INSTALLATION INSTRUCTIONS  . . . . . . . . . . . . . 3

2.1       INSTALLATION ON TOPS10 . . . . . . . . . . . . . . 4

2.2       INSTALLATION ON TOPS20 . . . . . . . . . . . . . . 4

3.0     AN OVERVIEW OF TGHA  . . . . . . . . . . . . . . . . 4

3.1       RUNNING TGHA . . . . . . . . . . . . . . . . . . . 4

3.2       MAKING LIST FILES  . . . . . . . . . . . . . . . . 4

3.3       TGHA FILE REQUIREMENTS . . . . . . . . . . . . . . 5

3.4       INITIAL SYSTEM START UP  . . . . . . . . . . . . . 6

3.5       MF20 ERRORS  . . . . . . . . . . . . . . . . . . . 6

3.6       KNOWN ERROR DETERMINATION  . . . . . . . . . . . . 6

3.7       CORRECTIVE ACTION  . . . . . . . . . . . . . . . . 7

3.8       PARITY ERRORS  . . . . . . . . . . . . . . . . . . 7

4.0     HISTORY FILE . . . . . . . . . . . . . . . . . . . . 8

4.1       DIRECTORY PAGE . . . . . . . . . . . . . . . . . . 8

4.2       GROUP PAGE . . . . . . . . . . . . . . . . . . . . 8

4.3       STORAGE MODULE PAGE  . . . . . . . . . . . . . . . 9

4.4       UNUSED STORAGE MODULES . . . . . . . . . . . . .  10

5.0     TRACE FILE . . . . . . . . . . . . . . . . . . . .  10

5.1       SPARE BIT SWAP . . . . . . . . . . . . . . . . .  10

5.2       PARITY ERROR   . . . . . . . . . . . . . . . . .  11

5.3       SERIOUS MF20 HARDWARE FAILURES . . . . . . . . .  11

5.4       USE OF SPARE BIT BY KLI  . . . . . . . . . . . .  11

5.5       KLI DETECTED BAD BLOCK . . . . . . . . . . . . .  11

5.6       MISSING DATABASE . . . . . . . . . . . . . . . .  12

5.6.1       RECOVERY PROCEDURE . . . . . . . . . . . . . .  12

5.7       SOFTWARE STATE WORD  . . . . . . . . . . . . . .  12

6.0     SPEAR  . . . . . . . . . . . . . . . . . . . . . .  13

6.1       NEW KNOWN ERROR: . . . . . . . . . . . . . . . .  13

6.2       SERIOUS MF20 HARDWARE FAILURE  . . . . . . . . .  13

6.3       USE OF SPARE BIT BY KLI  . . . . . . . . . . . .  13

6.4       KLI DETECTED BAD BLOCK . . . . . . . . . . . . .  14

6.5       MISSING DATABASE . . . . . . . . . . . . . . . .  14

6.6       SOFTWARE STATE WORD  . . . . . . . . . . . . . .  14

7.0     MF20 MEMORY CONTROLLER SBUS DIAGNOSTIC FUNCTIONS .  15

8.0     MF20 ARRAY BOARD . . . . . . . . . . . . . . . . .  22

9.0     MG20 ARRAY BOARD (M8570) . . . . . . . . . . . . .  23

10.0    MF20 ERRORS  . . . . . . . . . . . . . . . . . . .  24

10.1      CORRECTION CODE (ECC) CALCULATION  . . . . . . .  24

10.2      DETECTION AND CORRECTION . . . . . . . . . . . .  26

10.3      DOUBLE BIT ERROR SYNDROMES . . . . . . . . . . .  27

11.0    KL10 I/O INSTRUCTIONS RELATING TO MEMORY . . . . .  29

Page 3

1.0  ABSTRACT

TGHA - THE GREAT HEURISTIC ALGORITHM

TGHA IS THE MOS MEMORY ANALYZER PROGRAM THAT  IS  RUN  BY  THE  MONITOR,

WHENEVER   A   CORRECTABLE   ERROR  OCCURS  IN  AN  MF20.   IT  HAS  THE

RESPONSIBILITY FOR KEEPING THE MF20 MEMORIES IN A VIABLE CONDITION USING

THE  MF20  MAINTAINENCE FEATURES.  WHEN RUN BY THE MONITOR, TGHA OBTAINS

DATA ABOUT THE MF20 ERROR FROM THE MONITOR.  TGHA CAN ALSO BE RUN  BY  A

USER  WITH AT LEAST MAINTAINANCE PRIVILEGES.  IN THIS MODE, TGHA IS USED

TO CREATE ASCII FILES (BIT TO TEXT) FROM TGHA'S DATA BASE.   HISTORY.LST

CONTAINS  INFORMATION ABOUT THE HARDWARE CONFIGURATION, THE STATE OF THE

CONTROLLER CORRECTIVE LOGIC, AND A LIST  OF  LOGGED  ERRORS.   TRACE.LST

CONTAINS  ENTRIES  TRACKING  THE  CORRECTIVE ACTION TAKEN BY TGHA.  TGHA

4(0) CAN BE RUN WITH EITHER OR BOTH MF20  (16K)  AND  MG20  (64K)  ARRAY

MODULES  IN  THE MEMORY SYSTEM.  HOWEVER, BOTH TYPES CANNOT BE INSTALLED

WITHIN THE SAME CONTROLLER

1.  TGHA SOURCES WILL NOT BE AVAILABLE TO CUSTOMERS.

2.  THIS  DOCUMENT  APPLIES  TO  BOTH  TOPS10  AND  TOPS20   UNLESS

OTHERWISE SPECIFIED.

3.  IF THE MEMORY BOOT PORTION OF KLI DOES NOT CONFIGURE A BLOCK OF

MF20 MEMORY DUE TO MULTIPLE BIT ERRORS, TGHA WILL INDICATE WHAT

MEMORY AND HOW MUCH MEMORY WAS  NOT  CONFIGURED.   THE  MESSAGE

WILL  APPEAR  ON  THE  CTY,  IN  THE  TGHA  TRACE  FILE, AND IN

ERROR.SYS.

4.  IF THE MEMORY BOOT PORTION OF KLI FINDS  A  PARITY  ERROR,  KLI

WILL  USE  THE  SPARE  BIT AND CONFIGURE THE BLOCK.  TGHA, UPON

FINDING THAT A SPARE BIT WAS SET INTO USE BY  KLI,  WILL  NEVER

USE  THAT SPARE BIT FOR ANY OTHER PURPOSE.  UPON FINDING SUCH A

CONDITION, TGHA WILL OUTPUT AN APPROPRIATE MESSAGE ON THE  CTY,

IN THE TRACE FILE, AND IN ERROR.SYS.

5.  IF A PARITY ERROR EVER OCCURS DURING SWAPPING  IN  OF  A  SPARE

BIT,  THE  SPARE BIT WILL THEN BE FLAGGED AS IF KLI HAD SET THE

SPARE BIT INTO USE  (SEE  PREVIOUS  PARAGRAPH).   SIMILAR  CTY,

TRACE FILE AND SPEAR ENTRIES WILL BE MADE.

2.0  INSTALLATION INSTRUCTIONS

TGHA DATA AND TRACE FILE NAMES ARE TGHA.DAT AND  TGHA.TRA  RESPECTIVELY.

THE OLD FILES TGHAV2 OR TGHAV3 DAT/TRA SHOULD BE RENAMED TO PRESERVE THE

DATABASE.

Page 4

2.1  INSTALLATION ON TOPS10

FOR TGHA TO RUN PROPERLY ON TOPS10, THE FOLLOWING ENTRY MUST BE MADE  IN

OPR.ATO AFTER DAEMON IS STARTED::

1.  :SLOG

2.  :DEF TGHA=

3.  R TGHA

SINCE TGHA MAKES ENTRIES IN ERROR.SYS VIA DAEMON, DAEMON MUST BE RUNNING

BEFORE TGHA IS STARTED.  TGHA.EXE MUST RESIDE IN THE [1,4] AREA:

2.2  INSTALLATION ON TOPS20

TGHA.EXE MUST RESIDE IN PS:<SYSTEM> AREA:

3.0  AN OVERVIEW OF TGHA

3.1  RUNNING TGHA

1.  THE MONITOR RUNS TGHA AT INITIAL SYSTEM START UP

2.  THE MONITOR RUNS TGHA IN RESPONSE TO MF20 ERRORS

3.  FIELD SERVICE CAN RUN TGHA TO OBTAIN LIST FILES

3.2  MAKING LIST FILES

************************************************************************

NOTE:

IN ORDER TO RUN TGHA  MANUALLY,  THE  USER  MUST  HAVE  THE  MINIMUM  OF

MAINTAINENCE CAPABILITIES AND BE ENABLED.  THIS IS REQUIRED BECAUSE TGHA

LOOKS AT  THE  MF20  MEMORY  TO  DETERMINE  THE  CURRENT  CONFIGURATION.

************************************************************************

TGHA CAN ALSO BE RUN BY FIELD SERVICE PERSONNEL.   IN  THIS  MODE,  TGHA

KNOWS  THAT  IT  IS  NOT  THE  SYSTEM  TGHA.   THEREFORE,  TGHA DOES NOT

AUTOMATICALLY LOOK FOR MF20 ERRORS.

TGHA WILL RESPOND WITH A PROMPT OF:

TGHA>

IF "HELP"<CR> IS TYPED, THE FOLLOWING TEXT WILL BE PRINTED

Page 5

TGHA HELP - COMMAND     PURPOSE

EXIT            EXIT FROM TGHA.

HELP            TYPE THIS TEXT.

HISTORY         DUMP HISTORY FILE.

TRACE           DUMP TRACE FILE.

TGHA>

THE HISTORY AND TRACE DUMP FILES WILL BE CREATED IN THE AREA FROM  WHICH

TGHA  HAS  BEEN RUN.  THEY WILL BE HISTRY.LST FOR TOPS10 AND HISTORY.LST

FOR TOPS20 AND TRACE.LST RESPECTIVELY.  THE HISTORY AND TRACE DUMP FILES

WILL BE CREATED IN THE AREA FROM WHICH TGHA HAS BEEN RUN.

THESE FILES ARE LISTABLE VERSIONS OF THE TGHA  DATABASE  AND  THE  TRACE

FILE.   ALTHOUGH  TGHA.TRA  MAY  LOOK  READABLE, REMEMBER THAT IT IS THE

EQUIVALENT OF A RING BUFFER.  IE.  THE OLDEST DATA IS OVER WRITTEN  WHEN

THE  FILE  BECOMES  LARGE  ENOUGH.   THIS  PREVENTS  THE TRACE FILE FROM

BECOMING USELESSLY LARGE.  TRACE.LST  IS  IN  CHRONOLOGICAL  ORDER,  THE

OLDEST ENTRIES FIRST.

WHEN TGHA IS RUN BY FIELD SERVICE, THE ONLY FUNCTIONS AVAILABLE ARE  THE

DUMPING  OF  THE  LIST FILES.  NO CHANGES IN THE MEMORY CONFIGURATION OR

THE USE OF THE SPARE BITS CAN BE DONE BY TGHA IN USER MODE.

3.3  TGHA FILE REQUIREMENTS

THE TGHA FILE SYSTEM FOR BOTH TOPS10 AND TOPS20 INCLUDES  THE  FOLLOWING

FILES:

FILE            PURPOSE                 CREATION MECHANISM

----            -------                 -------------------

TGHA.EXE        TGHA EXECUTION FILE     MONITOR TAPE

TGHA.HLP        TGHA HELP FILE          MONITOR TAPE

TGHA.DOC        TGHA OVERVIEW FILE      MONITOR TAPE

TGHA.DAT        TGHA HISTORY/DATA FILE  TGHA.EXE WHEN FIRST RUN

TGHA.TRA        TGHA TRACE FILE         TGHA.EXE WHEN FIRST RUN

TGHA.BAD        BAD COPY OF TGHA.DAT    TGHA.EXE WHEN THE DATABASE GETS

CONFUSED

WHEN THE MONITOR FIRST STARTS UP ON A SYSTEM RELOAD, IT LOOKS  FOR  TGHA

IN  THE APPROPRIATE AREA.  IF IT IS NOT THERE, TGHA CANNOT BE RUN BY THE

MONITOR.  IF TGHA.EXE IS NOT FOUND, THE  MONITOR  WILL  THROW  AWAY  THE

ERROR  DATA  THAT  MIGHT  HAVE  BEEN  COLLECTED  BY  TGHA.  TGHA.DAT AND

TGHA.TRA WILL BE CREATED BY TGHA WHEN NECESSARY.

TGHA.DAT WILL NOT GROW AS ERRORS ARE INCURRED.  TGHA WILL PURGE OLD DATA

AS  ITS  DATABASES  FILL UP.  THE ONLY WAY THAT TGHA.DAT WILL GROW IS IF

NEW HARDWARE IS BROUGHT ON LINE (IE.  A STORAGE MODULE IS SWAPPED  OR  A

NEW  CONTROLLER  IS  ADDED  TO  THE  SYSTEM).   TGHA REQUIRES ALL OF THE

HISTORY DATA THAT IT HAS COLLECTED TO REMAIN INTACT SO THAT IT CAN  MAKE

CORRECT DECISIONS ABOUT CORRECTIVE ACTION.

Page 6

***TGHA.DAT SHOULD  NOT  BE  DELETED.   IT  CONTAINS  IMPORTANT  HISTORY

INFORMATION.

3.4  INITIAL SYSTEM START UP

AT INITIAL SYSTEM START UP, THE MONITOR RUNS TGHA IN START UP MODE.   IN

THIS  MODE, TGHA WILL FIRST ENABLE SINGLE BIT ERROR REPORTING THROUGHOUT

MF20 MEMORY.  TGHA WILL THEN EITHER BUILD THE HISTORY FILE  IF  IT  DOES

NOT  ALREADY  EXIST,  OR  VERIFY  THAT  IT  KNOWS  ABOUT ALL OF THE MF20

HARDWARE THAT IS ON LINE.  IF THE  HISTORY  FILE  EXISTS  AND  NEW  MF20

HARDWARE APPEARS, TGHA WILL ADD THIS NEW HARDWARE TO ITS HISTORY FILE.

ONCE THE INITIAL START UP INITIALIZATION IS COMPLETE,  TGHA  THEN  LOOKS

FOR ANY MF20 ERRORS THAT HAVE OCCURED SINCE THE INITIAL SYSTEM START UP.

3.5  MF20 ERRORS

FOR TOPS10, WHEN AN MF20 CORRECTABLE ERROR OCCURS, THE MONITOR WAKES  UP

THE SYSTEM TGHA.  FOR TOPS20, WHEN AN MF20 CORRECTABLE ERROR OCCURS, THE

MONITOR CALLS TGHA WITH A JOB NUMBER OF 0.  TGHA THEN GETS THE DATA  FOR

THE  MF20 ERROR FROM THE MONITOR.  THIS IS CALLED A CHRONOLOGICAL ERROR.

THEY ARE STORED IN ORDER OF OCCURRANCE IN THE HISTORY  DATABASE.   THERE

IS A SEPARATE CHRONOLOGICAL ERROR LIST FOR EACH MODULE.

ONLY WHEN THE CHRONOLOGICAL ERROR LIST IS FULL  DOES  TGHA  GO  OFF  AND

ATTEMPT TO RESOLVE A KNOWN ERROR FROM THIS LIST.

3.6  KNOWN ERROR DETERMINATION

KNOWN ERROR DETERMINATION IS DONE STATISTICALLY USING  THE  CHI  SQUARED

GOODNESS OF FIT FORMULA.  STARTING WITH THE FIRST CHRONOLOGICAL ERROR IN

THE STORAGE MODULE, THE FORMULA IS USED TO TEST THE DISTRIBUTION OF  THE

OTHER  ERRORS  IN THE CHRONOLOGICAL ERROR LIST FOR EACH TYPE OF POSSIBLE

HARDWARE ERROR.

FOR INSTANCE, THE FIRST  HARDWARE  FAILURE  CONSIDERED  IS  A  FULL  MUX

FAILURE.   A  TABLE  IS BUILT BY SCANNING ALL OF THE OTHER ERRORS IN THE

STORAGE MODULE AND TALLYING ONLY THOSE ERRORS THAT ARE  COVERED  BY  THE

SPECIFIC  TYPE  OF HARDWARE ERROR BEING CONSIDERED, IN THIS CASE, A FULL

MUX FAILURE.  THE TABLE FOR A FULL MUX FAILURE IS DISTRIBUTED  BY  BLOCK

AND  SUBBLOCK NUMBER.  THIS TRANSLATES INTO A DISTRIBUTION BY CHIP SINCE

THERE ARE 16  CHIPS  INVOLVED  IN  A  FULL  MUX  FAILURE.   IF  THE  THE

DISTRIBUTION  IS  EVEN  ENOUGH,  THE  CHI  2'D GOODNESS OF FIT TEST WILL

SUCCEED, AND A KNOWN ERROR IS DECLARED.

THIS CONTINUES UNTIL EITHER THE HARDWARE ERROR TYPE IS FOUND OR  ALL  OF

THE ERROR TYPES HAVE BEEN TRIED.

THE LAST TYPE OF HARDWARE ERROR CONSIDERED IS A CELL  ERROR.   IF  THERE

ARE MORE THAN A MINIMUM NUMBER OF ERRORS OF THE SAME CELL (TYPICALLY 5),

THEN A CELL ERROR IS DECLARED.

Page 7

AFTER AN ERROR TYPE HAS BEEN FOUND, ALL OF THE CHRONOLOGICAL ERRORS THAT

ARE  INFLUENCED  BY  THE  KNOWN  ERROR  TYPE  ARE  ELIMINATED  FROM  THE

CHRONOLOGICAL ERROR LIST.  THIS PROCEDURE CONTINUES FOR  EACH  REMAINING

ERROR IN THE CHRONOLOGICAL ERROR LIST.

AFTER THE KNOWN ERROR  ROUTINE  HAS  BEEN  RUN,  THE  CORRECTIVE  ACTION

ROUTINE IS CALLED.

3.7  CORRECTIVE ACTION

THE GOAL OF THE CORRECTIVE ACTION ROUTINES IS TO DETERMINE  THE  OPTIMUM

CORRECTIVE ACTION GIVEN THE CURRENT KNOWN ERRORS.  THIS IS DONE ON A PER

GROUP BASIS.

FIRST, THE WORST ERROR IN THE GROUP IS FOUND.  THIS IS  THE  ERROR  THAT

AFFECT  THE MOST AMOUNT OF MEMORY.  ONCE THE WORST ERROR HAS BEEN FOUND,

THE APPLICABLE SPARE BITS ARE USED TO COVER IT.  IF THERE IS  MORE  THAN

ONE  KNOWN  ERROR  IN  THE GROUP, A SCAN IS DONE TO SET ALL ICE (INHIBIT

CORRECTABLE ERROR REPORTING) BITS WITHIN THE SCOPE OF THE  WORST  ERROR.

THIS  PROCEDURE  IS  REPEATED  UNTIL ALL KNOWN ERRORS FOR THE GROUP HAVE

BEEN ANALYZED.

NOTE:   THERE IS ONE EXCEPTION TO THE PREVIOUS PROCEDURE.  IF THE MEMORY

BOOT  PORTION OF KLI HAS (DURING THE DBE SCAN - DOUBLE BIT ERROR) USED A

SPARE BIT TO HANDLE A DOUBLE BIT ERROR, TGHA WILL NOT CHANGE THE USE  OF

THAT  SPECIFIC  SPARE  BIT.  IF TGHA WERE TO ATTEMPT TO USE SUCH A SPARE

BIT FOR ANOTHER PURPOSE, THE  POSSIBILITY  OF  PARITY  ERRORS  RESULTING

WOULD BE TOO GREAT TO RISK.

ALSO, IF A PARITY ERROR OCCURS DURING THE SETTING OF A SPARE  BIT,  TGHA

WILL SET THE SAME CONSIDERATIONS ON THAT SPARE BIT AS IF KLI HAD SET IT.

ON THE OCCURRENCE OF  THE  PREVIOUS  CONDITIONS,  APPROPRIATE  ERROR.SYS

ENTRIES WILL BE MADE.

3.8  PARITY ERRORS

PARITY ERRORS ARE HANDLED LIKE PARITY ERRORS ALWAYS HAVE BEEN.  IE.   IF

THE  MONITOR  SUCCESSFULLY  CONTINUES  AFTER  THE  PARITY ERROR, IT WILL

ATTEMPT TO TAKE THE PAGE OF MEMORY WITH THE PARITY ERROR  OFFLINE.   THE

MONITOR  THEN  RUNS  TGHA  IF  THE  PAGE  WAS SUCCESSFULLY REMOVED.  THE

MONITOR MAY NOT BE ABLE TO TAKE THE PAGE WITH THE PARITY ERROR OFF  LINE

IF  ,FOR INSTANCE, THE PAGE WAS PART OF THE RESIDENT MONITOR.  TGHA WILL

ENTER PARITY ERRORS IN ITS TRACE FILE, NOT THE  HISTORY  FILE  DATABASE.

TGHA  WILL  MAKE  AN  ENTRY  IN  THE TRACE FILE AND ERROR.SYS INDICATING

PHYSICAL ADDRESS, BLOCK OF MEMORY, AND THE ORIENTATION OF THE 4  STORAGE

MODULES CONTAINING THE ERROR.

Page 8

4.0  HISTORY FILE

THE HISTORY FILE IS MADE UP OF 3 DIFFERENT TYPES OF PAGES.  THEY ARE THE

DIRECTORY  PAGE,  GROUP  PAGES,  AND STORAGE MODULE PAGES.  THE SEQUENCE

STARTS OUT WITH ONE DIRECTORY PAGE.  THIS  PAGE  CONTAINS  CONFIGURATION

INFORMATION  ABOUT  ALL OF THE MF20'S ON THE SYSTEM.  THE NEXT PAGE IS A

GROUP PAGE FOLLOWED BY FOUR STORAGE MODULE  PAGES.   THE  PATTERN  OF  A

GROUP  AND  FOUR  SUCCEEDING  STORAGE  MODULE PAGES IS REPEATED FOR EACH

GROUP CURRENTLY ON LINE IN MF20'S.   A  STORAGE  MODULE  PAGE  FOR  EACH

STORAGE MODULE THAT HAS GONE AWAY IS THEN LISTED.

4.1  DIRECTORY PAGE

THE FIRST PAGE OF THE HISTORY FILE CONTAINS THE CURRENT CONFIGURATION OF

THE MF20 MEMORY, INCLUDING THE LOCATION OF EACH STORAGE MODULE BY SERIAL

NUMBER.  NOTE THAT THE SERIAL NUMBER IN THE HISTORY FILE IS IN THE  SAME

FORMAT AS THE STICKER ON THE STORAGE MODULE ITSELF.

THE LOCATION OF THE STORAGE MODULES IN MF20 MEMORY IS DOCUMENTED IN  THE

HISTORY  LIST  FILE  FOR ARCHIVAL AND FIELD SERVICE REASONS.  THE ERRORS

LOGGED IN THE HISTORY FILE ARE DEPENDENT UPON  THE  ORIENTATION  OF  THE

STORAGE  MODULES.   IF THEY ARE MOVED, THE ERROR CORRECTIONS IN THE MF20

MAY HAVE DIFFERENT CHARACTERISTICS.  IF FIELD SERVICE WISHES TO  REPLACE

A  STORAGE  MODULE,  THE  EXACT  LOCATION  BY SERIAL NUMBER IS THEREFORE

DOCUMENTED.

4.2  GROUP PAGE

THE GROUP PAGE CONTAINS INFORMATION SPECIFIC TO THE CONFIGURATION OF THE

GROUP.   THE  LAST  TIME THE GROUP WAS USED IN THIS CONFIGURATION MAY BE

USEFUL IF GROUPS OF STORAGE MODULES ARE REMOVED AND PUT  BACK  INTO  THE

MEMORY  LATER.   THE TIME OF THE LAST CHANGE TO THE GROUP IS USEFUL WHEN

DETERMINING WHERE THE MOST RECENT ERRORS HAVE OCCURED IN THE MF20.

LET IT BE NOTED HERE THAT  THE  CHRONOLOGICAL  LIST  OF  ERRORS  IN  THE

HISTORY  FILE AND THE ENTRIES IN THE TRACE FILE ARE TAGGED WITH THE DATE

AND TIME.  THIS IS REQUIRED  WHEN  CORRELATING  CORRECTIVE  ACTION  WITH

MF20.

THE GROUP PAGE ALSO CONTAINS A MAP OF  THE  CURRENT  STATE  OF  THE  BIT

SUBSTITUTION  RAM.   THE  BIT  SUBSTITUTION  RAM  IS  USED TO DIRECT BIT

REPLACEMENT WITH THE SPARE BIT RAMS.

THE VALUE OF THE SPARE BIT RAM CONTAINS 4 FIELDS THAT HAVE THE FOLLOWING

MEANING:

FIELD BITS              MEANING

400             EITHER KLI USED THIS SPARE BIT TO COVER A PARITY

ERROR OR A PARITY OCCURED DURING A BIT SWAP.

THE 400 BIT IS IN THE TGHA DATABASE ONLY, AND IS

NOT USED IN THE ACTUAL BITSWAP PROCESS.

374             THE OCTAL VALUE OF THE BIT BEING REPLACED *

Page 9

2               SET - CORRECTABLE ERROR REPORTING IS DISABLED

CLEARED - CORRECTABLE ERROR REPORTING IS ENABLED

1               BIT SUB RAM VALUE PARITY BIT

*       TO FIND THE VALUE OF THE BIT BEING REPLACED, THIS FIELD MUST

BE SHIFTED TO THE RIGHT BY 2 PLACES, AND THE DECIMAL

EQUIVALENT VALUE OF THE OCTAL DIGITS IS THE WORD BIT BEING

REPLACED.

BIT SUBSTITUTION RAM EXAMPLES:

-----------------------------

VALUE           MEANING

256             - THE SPARE BIT IS NOT IN USE.  THE VALUE OF

THE BIT TO BE SWAPPED POINTS TO THE SPARE BIT.

THE SPARE BIT IS IN DECIMAL BIT POSITION 43

- CORRECTABLE ERROR REPORTING IS DISABLED FOR

THIS BIT SUB RAM ADDRESS

255             - SPARE BIT IS NOT IN USE (SAME AS PREVIOUS EXAMPLE)

- CORRECTABLE ERROR REPORTING IS ENABLED

652             - THE 400 BIT ON INDICATES THAT THIS USE OF

THIS SPARE BIT PREVENTS A PARITY ERROR

FROM OCCURING.  TGHA WILL NOT CHANGE THIS

VALUE.  THE BITSUB RAM DATA REQUIRES ODD

PARITY.

- BIT 42 OF THE MF20 DATA PATH IS BEING SWAPPED

(THE PARITY BIT).

THE ACTUAL SPARE BIT ADDRESS IN SBUS DIAG FUNCTION 7  IS  7  BITS  WIDE.

THE  ADDRESS  IN THE GROUP PAGE ACCOUNTS FOR ONLY THE LOWER 5 BITS.  THE

HIGH ORDER 2 BITS IS THE GROUP NUMBER WITHIN THE CONTROLLER.  THE  GROUP

FIELD  IS DETERMINED FROM THE GROUP POSITION IN THE MF20 CONTROLLER, AND

NOT THE GROUP DATABASE.  THEREFORE THE GROUP PORTION  OF  THE  FIELD  IS

NOT,  AND  INDEED CANNOT BE KEPT WITHIN THE SPARE BIT RAM ADDRESS TABLE.

THIS PORTION OF THE SBUS DIAG FUNCTION IS FILLED IN AT EXECUTION TIME.

4.3  STORAGE MODULE PAGE

A STORAGE MODULE PAGE CONTAINS INFORMATION RELATING  SPECIFICLY  TO  THE

STORAGE  MODULE.   THE  SERIAL  NUMBER APPEARS IN THE SAME FORMAT AS THE

STICKER ON THE MODULE ITSELF.  THIS IS CRITICAL FOR FIELD RETURNS, WHICH

REQUIRE  THAT THIS PAGE BE ATTACHED TO THE STORAGE MODULE IF IT IS TO BE

RETURNED TO THE FACTORY FOR REPAIRS.  THE LAST TIME THE MODULE WAS  USED

CAN  BE  USEFUL  IN  TRACKING  MODULE  SWAPS  MADE IN THE MF20 AND FIELD

RETURNS.

THE KNOWN ERROR LIST CONTAINS THE ERROR INFORMATION RELATED TO THE KNOWN

ERRORS  RESOLVED  FROM  CHRONOLOGICAL  ERRORS  BY  TGHA.   MOST  OF  THE

INFORMATION IN EACH ENTRY IS  USED  BY  TGHA  TO  DETERMINE  OVERLAP  OF

ERRORS.   THIS  OVERLAP IS IMPORTANT WHEN CONSIDERING THE OPTIMUM USE OF

THE SPARE BITS.

Page 10

THE STORAGE MODULE  CHIP  LOCATION  IS  INCLUDED  TO  FACILITATE  MODULE

REPAIR.

THE TIME OF RESOLUTION OF THE ERROR MAY GIVE AN INDICATION OF  THE  RATE

OF DETERIORATION OF FAILURE OF THE MODULE.

THE CURRENT CHRONOLOGICAL ERROR LIST FOR THE STORAGE MODULE WILL INCLUDE

ANY ERRORS THAT HAVE NOT BEEN RESOLVED INTO KNOWN ERRORS YET.  THERE MAY

BE INCLUDED IN THIS LIST, ANY SOFT ERRORS THAT HAVE OCCURED.  THESE SOFT

ERRORS  WILL  BE  REMOVED FROM THE LIST SOMETIME AFTER A MINIMUM DEFAULT

(ABOUT 1 WEEK).

4.4  UNUSED STORAGE MODULES

ANY STORAGE MODULES THAT ARE IN THE  TGHA  HISTORY  FILE,  YET  ARE  NOT

PRESENTLY  IN USE THEN FOLLOW IN A SIMILAR MANNER.  THE INCLUSION OF THE

UNUSED STORAGE MODULES IN THE HISTORY FILE IS REQUIRED FOR THE CASE WHEN

FIELD  SERVICE  HAS  REPLACED A STORAGE MODULE, RELOADED THE SYSTEM, AND

WANTS A COPY OF THE HISTORY FILE FOR THAT STORAGE  MODULE  SO  THAT  THE

ERROR  DATA  CAN BE INCLUDED WITH THE MODULE WHEN THE MODULE IS RETURNED

FOR REPAIRS.

SINCE THESE STORAGE MODULES ARE NO LONGER PART OF ANY CONFIGURED  GROUP,

NO GROUP INFORMATION IS GIVEN.

5.0  TRACE FILE

THE TRACE FILE CONTAINS DATED ENTRIES INDICATING WHAT CORRECTIVE  ACTION

HAS  BEEN  TAKEN BY TGHA.  IF THE TGHA SOFTWARE RUNS INTO CONFUSION OR A

DATABASE GETS FULL, ENTRIES INTO THE TRACE FILE WILL  ALSO  REFLECT  THE

DIFFICULTY.   SERIOUS  MF20  HARDWARE ERRORS WILL ALSO BE ENTERED IN THE

TRACE FILE.

5.1  SPARE BIT SWAP

11-Oct-82 15:50:12       - IN CONTROLLER 10, GROUP 0, VECTOR 3,THE SPARE BIT IS GOING TO BE CHANGED TO 124 (OCTAL).

ONCE THE SPARE BIT SWAP HAS COMPLETED, THE RESULTS OF THE BITSWAP ARE INDICATED.

11-Oct-82 15:50:12     - BIT SUBSTITUTION - EVERYTHING COMPLETED NORMALLY

WHEN CORRECTABLE ERROR REPORTING IS DISABLED FOR A SPARE BIT SUBSTITUTION RAM ADDRESS, THE FOLLOWING ENTRY IS MADE:

11-Oct-82 15:50:13       - IN CONTROLLER 10, GROUP 0, VECTOR 3, THE ICE BIT IS GOING TO BE SET.

Page 11

5.2  PARITY ERROR

**********

11-OCT-82 19:20:02

* PARITY ERROR AT ADDRESS 1001475, BLOCK 0

* STORAGE MODULE SERIAL NUMBERS BY FIELD: 0 = 7460209 1 = 7460108 2 = 7460117 3 = 7460200

**********

5.3  SERIOUS MF20 HARDWARE FAILURES

************************************************************************

*11-OCT-82 19:20:02

* TGHA HAS TEMPORARILY CORRECTED A SERIOUS MOS MEMORY FAILURE.

*** CALL FIELD SERVICE TO REPORT THIS CONDITION ***

************************************************************************

THIS MESSAGE ALSO APPEARS ON THE  CTY.   ALTHOUGH  THE  ERROR  HAS  BEEN

CORRECTED USING THE SPARE BITS, THE PROBABILITY OF A PARITY ERROR DUE TO

FURTHER HARDWARE DEGRADATION HAS RISEN TO AN UNCOMFORTABLE  STATE.   THE

MF20  DIAGNOSTICS  SHOULD  BE  RUN  AND  THE  OFFENDING  STORAGE  MODULE

REPLACED.

5.4  USE OF SPARE BIT BY KLI

*****

5-AUG-79 13:20:29

THE MEMORY BOOT IN KLI HAS USED THE SPARE BIT TO PREVENT A PARITY ERROR.

THIS CONDITION SHOULD BE CORRECTED AS SOON AS POSSIBLE.

CONTROLLER      GROUP   BLOCK   WORD (BITS 33-35)

10              0       1       5

*****

5.5  KLI DETECTED BAD BLOCK

IF KLI HAS NOT CONFIGURED A BLOCK OF MF20 MEMORY  BECAUSE  OF  AN  ERROR

THAT  CANNOT  BE  REPAIRED  USING  THE  SPARE BIT, THE BLOCK WILL NOT BE

CONFIGURED.  THIS MESSAGE ALSO APPEARS ON THE CTY.   AN  ENTRY  WILL  BE

MADE IN THE TRACE FILE AS FOLLOWS:

************************************************************************

*5-AUG-79 13:40:51

* THE FOLLOWING BLOCKS ARE MARKED AS BAD

* AND ARE NOT ON LINE:

*  CONTROLLER   GROUP   BLOCK

*       10      1       3

*       10      2       1

* THIS CONSISTS OF 128K OF MEMORY THAT IS OFF LINE.

*** CALL FIELD SERVICE TO REPORT THIS CONDITION ***

************************************************************************

Page 12

5.6  MISSING DATABASE

IF TGHA FINDS THAT THERE IS NO DATABASE AND THAT AN MF20 CONTROLLER  WAS

LAST INITIALIZED BY TGHA, TGHA WILL OUTPUT THE FOLLOWING MESSAGE:

*****

15-AUG-1979 15:10:20

* MF20 CONTROLLER 10 WAS LAST INITIALIZED BY TGHA AND

* TGHA HAS LOST ITS DATABASE.  REFER TO TRACE OR SPEAR

* DOCUMENTATION IN TGHA.DOC FOR FURTHER DETAILS AND RECOVERY INSTRUCTIONS.

*****

IN THIS SITUATION, TGHA CANNOT DETERMINE WHETHER ANY SPARE BITS  IN  USE

WERE SET BY THE MEMORY BOOT IN KLI OR TGHA.  THIS INFORMATION WAS STORED

IN THE TGHA DATABASE, WHICH  WENT  AWAY  FOR  SOME  REASON.   TGHA  MUST

THEREFORE  ASSUME  THAT ALL SPARE BITS IN USE WERE SET BY KLI TO PREVENT

PARITY ERRORS.  THIS MEANS THAT THESE SPARE BITS WILL NOT  BE  AVAILABLE

FOR CORRECTION OF FURTHER HARDWARE FAILURES IN THE MF20 MEMORY.  THIS IS

NOT AN IDEAL SITUATION FOR TGHA TO BE IN, AND WILL NOT RECTIFY ITS  SELF

AUTOMATICALLY.   ALTHOUGH THIS SITUATION IS NOT IMMEDIATELY CRITICAL, IT

DEGRADES THE  PERFORMANCE  OF  TGHA.   THE  RECOVERY  PROCEDURE  CAN  BE

DEFERRED  TO  A  CONVENIENT TIME, LIKE A FIELD SERVICE PM OR A SCHEDULED

SYSTEM RELOAD.

5.6.1  RECOVERY PROCEDURE

1.  DELETE TGHA.DAT FROM PS:<SYSTEM>

2.  BRING THE SYSTEM DOWN

3.  RECONFIGURE THE MEMORY SYSTEM BY USING THE  'FORCE'  OPTION  IN

THE MEMORY CONFIGURATION PORTION OF KLI.

4.  REBOOT THE SYSTEM

THIS IS THE -ONLY- TIME THAT THE TGHA DATABASE FILE (TGHA.DAT) SHOULD BE

EXPLICITELY  DELETED.   ALTHOUGH  THIS  DESTROYS  ALL OF THE KNOWN ERROR

HISTORY, THE KNOWN ERRORS WILL BE RESOLVED BY TGHA IN THE SAME MANNER AS

BEFORE.

THIS IS THE ONLY TIME THAT THE TGHA DATABASE FILE (TGHA.DAT)  SHOULD  BE

EXPLICITELY  DELETED.   ALTHOUGH  THIS  DESTROYS  ALL OF THE KNOWN ERROR

HISTORY, THE KNOWN ERRORS WILL BE RESOLVED BY TGHA IN THE SAME MANNER AS

BEFORE.   THE  PROBABILITY  OF  THIS SITUATION ARISIG IS VERY LOW.  THIS

DRASTIC ACTION IS EXPLAINED ONLY TO COVER THIS SITUATION.

5.7  SOFTWARE STATE WORD

THE SOFTWARE STATE OF THE MEMORY INDICATES WHETHER KLI OR TGHA  WAS  THE

LAST  PROGRAM  TO  MAKE  ANY  CHANGES  IN THE BIT SUB RAM.  ALTHOUGH THE

SOFTWARE BITS IN SBUS FUNCTION WERE DESIGNED FOR THIS PURPOSE, CONFLICTS

MADE  THEM  IMPRACTICAL.   THE  INFORMATION  IS FLAGGED IN A WORD IN THE

Page 13

BITSUB RAM INSTEAD OF USING THE SOFTWARE STATE BITS.  IF, WHEN TGHA GOES

TO  CHECK  THE  STATE OF THE SOFTWARE STATE WORD IN THE BITSUB RAM, TGHA

FINDS THAT THE DATA IS NOT OF THE FORMAT THAT EITHER KLI OR  TGHA  USES,

THE FOLLOWING MESSAGE WILL BE PUT OUT:

*****

17-AUG-1979 13:02:59

IN CONTROLLER 10 THE BITSUB RAM STATE WORD WAS GARBAGE.

ASSUMIMG A DBE SCAN WAS DONE LAST.  SEE TGHA.DOC FOR FURTHER DETAILS.

*****

THIS IS A VERY STRANGE STATE SINCE THE SPARE BIT SUB  RAM  ADDRESS  USED

FOR  THE  SOFTWARE  STATE  WORD  IS  NOT  IN  THE ADDRESS SPACE USED FOR

SWAPPING OF MEMORY BITS.  IT IS SUGGESTED THAT THE MF20  DIAGNOSTICS  BE

RUN AT THE NEXT CONVENIENT TIME.

6.0  SPEAR

ERROR.SYS ENTRIES MADE BY TGHA HAVE THE FOLLOWING FORMAT:

6.1  NEW KNOWN ERROR:

A NEW MF20 KNOWN ERROR HAS BEEN DECLARED.  DATA:  STORAGE MODULE  SERIAL

NUMBER:   838009  BLOCK:   1, SUBBLOCK:  0, BIT IN FIELD (10):  11, ROW:

71, COLUMN:  24, E NUMBER 192, ERROR TYPE:  CHIP

6.2  SERIOUS MF20 HARDWARE FAILURE

************************************************************************

*11-OCT-82 19:20:02

* TGHA HAS TEMPORARILY CORRECTED A SERIOUS MOS MEMORY FAILURE.

*** CALL FIELD SERVICE TO REPORT THIS CONDITION ***

************************************************************************

ALTHOUGH THE  ERROR  HAS  BEEN  CORRECTED  USING  THE  SPARE  BITS,  THE

PROBABILITY  OF  A  PARITY ERROR DUE TO FURTHER HARDWARE DEGRADATION HAS

RISEN TO AN UNCOMFORTABLE STATE.  THE MF20 DIAGNOSTICS SHOULD BE RUN AND

THE OFFENDING STORAGE MODULE REPLACED.

6.3  USE OF SPARE BIT BY KLI

*****

5-AUG-83 13:20:29

THE MEMORY BOOT IN KLI HAS USED THE SPARE BIT TO PREVENT A PARITY ERROR.

THIS CONDITION SHOULD BE CORRECTED AS SOON AS POSSIBLE.

CONTROLLER      GROUP   BLOCK   WORD (BITS 33-35)

10              0       1       5

*****

Page 14

6.4  KLI DETECTED BAD BLOCK

IF KLI HAS NOT CONFIGURED A BLOCK OF MF20 MEMORY  BECAUSE  OF  AN  ERROR

THAT  CANNOT  BE  REPAIRED  USING  THE  SPARE BIT, THE BLOCK WILL NOT BE

CONFIGURED.  AN ENTRY WILL BE MADE IN ERROR.SYS AS FOLLOWS:

************************************************************************

*5-AUG-83 13:40:51

* THE FOLLOWING BLOCKS ARE MARKED AS BAD

* AND ARE NOT ON LINE:

*  CONTROLLER   GROUP   BLOCK

*       10      1       3

*       10      2       1

* THIS CONSISTS OF 128K OF MEMORY THAT IS OFF LINE.

*** CALL FIELD SERVICE TO REPORT THIS CONDITION ***

**********************************************************************

6.5  MISSING DATABASE

IF TGHA FINDS THAT THERE IS NO DATABASE AND THAT AN MF20 CONTROLLER  WAS

LAST INITIALIZED BY TGHA, TGHA WILL OUTPUT THE FOLLOWING MESSAGE:

*****

15-AUG-1983 15:10:20

* MF20 CONTROLLER 10 WAS LAST INITIALIZED BY TGHA AND

* TGHA HAS LOST ITS DATABASE.  REFER TO TRACE OR SPEAR

* DOCUMENTATION IN TGHA.DOC FOR FURTHER DETAILS AND RECOVERY INSTRUCTIONS.

*****

FOR FURTHER EXPLANATION REFER TO MISSING DATABASE  SECTION  UNDER  TRACE

FILE ENTRIES.

6.6  SOFTWARE STATE WORD

THE SOFTWARE STATE OF THE MEMORY INDICATES WHETHER KLI OR TGHA  WAS  THE

LAST  PROGRAM  TO  MAKE  ANY  CHANGES  IN THE BIT SUB RAM.  ALTHOUGH THE

SOFTWARE BITS IN SBUS FUNCTION WERE DESIGNED FOR THIS PURPOSE, CONFLICTS

MADE  THEM  IMPRACTICAL.   THE  INFORMATION  IS FLAGGED IN A WORD IN THE

BITSUB RAM INSTEAD OF USING THE SOFTWARE STATE BITS.  IF, WHEN TGHA GOES

TO  CHECK  THE  STATE OF THE SOFTWARE STATE WORD IN THE BITSUB RAM, TGHA

FINDS THAT THE DATA IS NOT OF THE FORMAT THAT EITHER KLI OR  TGHA  USES,

THE FOLLOWING MESSAGE WILL BE PUT OUT:

*****

17-AUG-1983 13:02:59

IN CONTROLLER 10 THE BITSUB RAM STATE WORD WAS GARBAGE.

ASSUMIMG A DBE SCAN WAS DONE LAST.  SEE TGHA.DOC FOR FURTHER DETAILS.

*****

THIS IS A VERY STRANGE STATE SINCE THE SPARE BIT SUB  RAM  ADDRESS  USED

FOR  THE  SOFTWARE  STATE  WORD  IS  NOT  IN  THE ADDRESS SPACE USED FOR

SWAPPING OF MEMORY BITS.  IT IS SUGGESTED THAT THE MF20  DIAGNOSTICS  BE

RUN AT THE NEXT CONVENIENT TIME.

Page 15

7.0  MF20 MEMORY CONTROLLER SBUS DIAGNOSTIC FUNCTIONS

FUNCTION 00      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !CLEAR*                                                                       *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  ! 0-5 *     !     !     *     !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*                                                                             ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !     *     !     !     *     !     !     *     !     !     *     !  0  !  0  *  0  !  0  !  0  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*CONTR!CORR !INCMP* <PARITY ERRORS> *INTERLEAVE ! <LAST WORD REQUESTS > *<LAST TYPE>! <   ERROR ADDRESS     *

FROM MEM 6* ERR ! ERR !CYCLE*READ !WRITE! ADR *  1  !  1  ! RQ0 * RQ1 ! RQ2 ! RQ3 * RD  ! WR  ! 14  * 15  ! 16  ! 17  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                      ERROR   ADDRESS   (CONTINUED)                                      > *

FROM MEM 8* 18  ! 19  ! 20  * 21  ! 22  ! 23  * 24  ! 25  ! 26  * 27  ! 28  ! 29  * 30  ! 31  ! 32  * 33  ! 34  ! 35  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

FUNCTION 01      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !                                         *<GROUP LOOPBACK >*                 *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     !     !     *     !     !     *ENABL! GN2 ! GN1 *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*                                         ! <   STATUS    > !LD EN!           ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !     *     !     !     *     !DSABL! SF2 * SF1 !25-27!     *     !  0  !  0  *  0  !  0  !  1  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                               ! <MEM CONTROLLER TYPE> *  <LPBK GRP SEL> *                 *

FROM MEM 6*     !     !     *     !     !     *     !     !  0  *  1  !  0  !  1  *  0  !  1  !  2  *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                         ! <   STATUS    > !                                               *

FROM MEM 8*     !     !     *     !     !     *     !DSABL! SF2 * SF1 !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

SF2 & SF1 ARE SOFTWARE FLAGS DO NOT EFFECT THE HARDWARE, NOR DOES THE HARDWARE AFFECT THEM EXCEPT TO CLEAR THEM ON POWER-UP.  THEIR

COMBINED MEANINGS ARE: 0/CONTROLLER WAS JUST POWERED UP; 1/ALL RAMS BUT ADRESP ARE LOADED, CRUDE PATCHING IS DONE AND BITSUB RAM

SAYS WHICH BLOCKS MAY BE USED; 2/SAME AS 1 BUT CONTROLLER IS CONFIGURED; 3/SAME AS 2 EXCEPT THAT TGHA HAS FINISHED ITS WORK.

Page 16

FUNCTION 02      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !                       * <  SM PROM SELECT   > !<PROM BYTE>*                 *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     !     !     * GN2 ! GN1 ! FN2 * FN1 !  2  !  1  *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*                             ! < DIAG MIXER INPUT SELECT > !                 ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !     *     !     !  2  *  3  !  4  !  5  *  6  !     !     *     !  0  !  0  *  0  !  1  !  0  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                             !<ERROR WD#>! <      SELECTED DIAG DATA OR PROM DATA      > *                 *

FROM MEM 6*     !     !     *     !     !  2  *  1  !  0  !  1  *  2  !  3  !  4  *  5  !  6  !  7  *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*           ! <     MOS  ADDRESS  (SINGLE STEP ONLY)      > !<CTL RAM PAR ERR>!                             *

FROM MEM 8*     !     !  0  *  1  !  2  !  3  *  4  !  5  !  6  *  7  ! TIM ! SUB * ADR !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

NOTE: BITS 23-27 ARE ONLY USED DURING SINGLE STEP DIAGNOSTIC TESTS.  THE LEFTMOST 1 BIT SELECTS THE CORRESPONDING MIXER INPUT

FOR READING DIAGNOSTIC SIGNALS.  IF 23-27 ARE 0 THEN THE DATA RETURNED IS THE SELECTED SM PROM WORD.  NOTE ALSO THAT

BITS 26-27 ARE USED TO SELECT THE ADDRESS WHICH IS SENT BACK IN THE MOS ADDRESS FIELD.

FUNCTION 03      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !                 !FAST *     !<FIXED VAL LOGIC RAMS >!LD EN*LD EN!           *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     !     ! BUS *     !ACKN !D VLD*RDA34!RDA35! 10  *11-13!     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*           ! <    FIXED VALUE LOGIC RAM LOAD ADDRESS     > !                 ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !  0  *  1  !  2  !  3  *  4  !  5  !  6  *  7  !     !     *     !  0  !  0  *  0  !  1  !  1  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                               !FAST *     !<FIXED VAL LOGIC RAMS >!                       *

FROM MEM 6*     !     !     *     !     !     *     !     ! BUS *     !ACKN !D VLD*RDA34!RDA35!     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                                                                                           *

FROM MEM 8*     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

NOTES:  THE ACKN RAM (BIT 10) USES ADR BITS 1-7 ONLY.

THE FAST-BUS BIT IS DISCONNECTED INTERNALLY AND IS LEFT IN ONLY FOR DIAGNOSTIC COMPATABILITY.

Page 17

FUNCTION 04      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !PORT *                 * <        SINGLE    STEP    CLOCK    CONTROL         *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !LPBK *     !     !     *A COM!B COM!D VLD*     !     !SSCLK*SSMOD!           *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*   SS CLK CTL  > *REFR !LD EN!     * <  REFRESH INTERVAL (GATED CLOCK/16)  > ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !RNOW *ALLOW!24-30!     *  1  !  2  !  3  *  4  !  5  !  6  *  7  !  0  !  0  *  1  !  0  !  0  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                                     * <      SS  CLOCK  CONTROL  ECHO       > !           *

FROM MEM 6*     !     !     *     !     !     *     !     !     *A COM!B COM!     *     !     !     *SSMOD!     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                 *REFR !     ! <       REFRESH   INTERVAL   COUNTER        > !                             *

FROM MEM 8*     !     !     *ALLOW!     !  0  *  1  !  2  !  3  *  4  !  5  !  6  *  7  !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

NOTE: IF PORT LOOPBACK IS SET (BIT 5) THE CONTROLLER WILL JUST ECHO ANYTHING SENT OUT BY AN SBUS DIAG AS LONG AS THE PROPER

CONTROLLER NUMBER IS GIVEN.  ONLY AN SBUS RESET CAN CLEAR THIS CONDITION.

FUNCTION 05      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !     * <  SINGLE  STEP  SIMULATED  REQUEST  BITS   > !     *LD EN!           *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *ST A !ST B ! RQ0 * RQ1 ! RQ2 ! RQ3 * RD  ! WR  !     * SS  !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*           ! <         SINGLE STEP REQUEST SIMULATED ADDRESS BITS          > ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     ! 14  * 15  ! 16  ! 17  * 18  ! 19  ! 20  * 21  !     ! 34  * 35  !  0  !  0  *  1  !  0  !  1  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                                                                                           *

FROM MEM 6*     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                   * <   SS  MOS  RAS    > !                                               *

FROM MEM 8*     !     !     *     !     !     *  0  !  1  !  2  *  3  !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

Page 18

FUNCTION 06      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !           ! <     DIAGNOSTIC DATA TO SY OR WP BOARD     > *LD EN!           *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     ! 32  ! 16  *  8  !  4  !  2  *  1  !MISC !MISC *07-14!     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*                                         !<ECC DIAG SUBFCN>!                 ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !     *     !     !     *     !  4  !  2  *  1  !     !     *     !  0  !  0  *  1  !  1  !  0  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                         ! <    DIAGNOSTIC DATA FROM SY OR WP BOARD    > *                 *

FROM MEM 6*     !     !     *     !     !     *     ! 32  ! 16  *  8  !  4  !  2  *  1  !MISC !MISC *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                   * <   SS  MOS  CAS    > !                                               *

FROM MEM 8*     !     !     *     !     !     *  0  !  1  !  2  *  3  !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

NOTE: SBDIAG FCN 6 DOES DIFFERENT THINGS DEPENDING ON THE SUBFUNCTION CODE IN BITS 25-27:

6.0     READ THE ECC REGISTER ON THE SYN BOARD.

6.1     READ THE SYNDROME BUFFER ON THE SYN BOARD.  THIS FUNCTION IS ALSO NORMALLY USED BY MONITOR.

6.2     SEL DIAG BITS 07-13 IN PLACE OF MOS BITS 36-42, FORCE ZEROS ON 00-35, RUN THRU A CORRECTION PASS, AND RETURN 00-35.

*** PLEASE NOTE THAT FCN 6.2 RETURNS THE ENTIRE WORD, NOT JUST BITS 7-14. ***

6.3     UNUSED

6.4     WR ECC COMPLEMENT REG IF BIT 15 SET, THEN READ IT BACK.

6.5     WR ECC COMPLEMENT REG IF BIT 15 SET, THEN ENABLE IT TO BE SENT TO MEM IN PLACE OF D36-42 ON NEXT WR TO MEM.

6.6     READ D36-43 MIXER.

6.7     ENABLE LATCHING OF D36-43 MIXER AFTER NEXT WRITE.

7     8     9    10    11    12    13    14                    7     8     9    10    11    12    13    14

+-----+-----*-----+-----+-----*-----+-----+-----*              +-----+-----*-----+-----+-----*-----+-----+-----*

SUBFUNCTION ! <  MEM BITS 36-43 LATCHED ON LAST RD ERROR  > *  SUBFUNCTION ! <  ECC BITS TO BE COMPLEMENTED ON WR  > ! PAR *

6.0     !ECC32!ECC16*ECC8 !ECC4 !ECC2 *ECC1 !E PAR!SPARE*   6.4        !  32 !  16 *  8  !  4  !  2  *  1  !E PAR! CTL *

+-----+-----*-----+-----+-----*-----+-----+-----*              +-----+-----*-----+-----+-----*-----+-----+-----*

+-----+-----*-----+-----+-----*-----+-----+-----*              +-----+-----*-----+-----+-----*-----+-----+-----*

SUBFUNCTION ! <SYNDROME LATCHED ON LAST RD ERR> !<ERR TYPE >*  SUBFUNCTION ! <BITS 36-43 AS WRITTEN TO MEM AFTER FCN 6.7 > *

6.1     !  32 !     16 *  8  !  4  !  2  *  1  ! CORR! DBL *   6.6     !ECC32!ECC16*ECC8 !ECC4 !ECC2 *ECC1 !E PAR!SPARE*

+-----+-----*-----+-----+-----*-----+-----+-----*              +-----+-----*-----+-----+-----*-----+-----+-----*

Page 19

FUNCTION 07      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !           ! <  BIT SUBSTITUTION RAM DATA (BIT NUMBERS)  > *LD EN!           *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     ! 32  ! 16  *  8  !  4  !  2  *  1  ! ICE ! PAR *07-14!     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*                 * <  BIT SUBSTITUTION RAM LOAD ADDRESS  > !                 ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !     * GN2 ! GN1 ! BN2 * BN1 ! 33  ! 34  * 35  !     !     *     !  0  !  0  *  1  !  1  !  1  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                         ! <      BIT SUBSTITUTION RAM DATA ECHO       > *                 *

FROM MEM 6*     !     !     *     !     !     *     ! 32  ! 16  *  8  !  4  !  2  *  1  ! ICE ! PAR *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                   * <  SS  MOS  WR  EN  > !                                               *

FROM MEM 8*     !     !     *     !     !     *  0  !  1  !  2  *  3  !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

"ICE" MEANS IGNORE CORRECTABLE ERROR (IE DON'T SET CORR ERR FLAG)

FUNCTION 10      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !           ! <  VOLTAGE MARGINS  > ! < V MARGIN ENABLES  > *LD EN!           *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     !12.60!5.25 *-2.10!-5.46! 12  *  5  ! -2  !-5.2 *07-14!     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*                                               !CORR * CLR !                 ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     !     *     !     !     *     !     ! DIS *DCBAD!     !     *     !  0  !  1  *  0  !  0  !  0  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                         ! <  VOLTAGE MARGINS  > ! < V MARGINS ENABLED > *                 *

FROM MEM 6*     !     !     *     !     !     *     !12.60!5.25 *-2.10!-5.46! 12  *  5  ! -2  !-5.2 *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                               !CORR * DC  !                                               *

FROM MEM 8*     !     !     *     !     !     *     !     ! DIS * BAD !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

NOTE: THE VOLTAGES GIVEN ABOVE APPLY FOR ONES IN BITS 7-10 WHERE ENABLED BY BITS 11-14.  THE VOLTAGES CORRESPONDING TO ZEROS

IN BITS 7-10 ARE 11.40, 4.75, -1.90, AND -4.94.  MORE THAN ONE MARGIN MAY BE SET CONCURRENTLY.

Page 20

FUNCTION 11      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !                                               ! <   TIMING RAM LOAD ADR...  *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     !     !     *     !     !     *     !  0  !  1  *  2  !  3  !  4  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*  ...ADR > !LD EN* <         TIMING   RAM   DATA         > !                 ! <     FUNCTION NUMBER     > *

TO MEM   4*  5  !  6  !21-30* RAS ! CAS ! PAR *WR EN!2ND A!D RDY*B CLR!     !     *     !  0  !  1  *  0  !  0  !  1  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                                                                                           *

FROM MEM 6*     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                 * <     TIMING   RAM   DATA   ECHO      > !     ! 4K  *-DSEL!                             *

FROM MEM 8*     !     !     * RAS ! CAS ! PAR *WR EN!2ND A!D RDY*B CLR!     ! EN  *CYCEN!     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

FUNCTION 12      00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E)    1* <    CONTROLLER NUMBER    > !                 ! <    ADDRESS  RESPONSE  RAM  DATA     > *LD EN!           *

TO MEM   2*  0  !  1  !  ?  *  ?  !  ?  !     *     !     ! PAR *TYPE ! GN2 ! GN1 * BN2 ! BN1 !DESEL*08-14!     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E)    3*           ! <     ADDRESS RESPONSE RAM LOAD ADDRESS     > !                 ! <     FUNCTION NUMBER     > *

TO MEM   4*     !     ! 14  * 15  ! 16  ! 17  * 18  ! 19  ! 20  * 21  !     !     *     !  0  !  1  *  0  !  1  !  0  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

LH(E+1)  5*                                               ! <    ADDRESS  RESPONSE  RAM  ECHO     > *                 *

FROM MEM 6*     !     !     *     !     !     *     !     ! PAR *TYPE ! GN2 ! GN1 * BN2 ! BN1 !DESEL*     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

RH(E+1)  7*                                                                                                           *

FROM MEM 8*     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

NOTE: "DESEL" (BIT 14) IS "BOX DESELECTED ON 1".

Page 21

*** MF20 STORAGE ORGANIZATION ***

MF20 STORAGE IS COMPOSED OF GROUPS, FIELDS, BLOCKS, & SUBBLOCKS.                  KL10 BIT NUMBER VS. "FIELD" LOCATION

A GROUP IS A SET OF FOUR STORAGE BOARDS.  A FIELD IS ONE BOARD (11 BITS)                ----------------------------------------

WIDE.  4 FIELDS USED IN PARALLEL GIVE A 44-BIT WORD: 36 DATA BITS, 6 ECC

(ERROR CHECK & CORRECT) BITS, ONE 43-BIT PARITY BIT, AND ONE SPARE BIT.                 KL10      BIT           BIT IN  SYNDROME

A BIT'S GROUP AND FIELD NUMBERS UNIQUELY PICK THE BIT'S STORAGE BOARD.                   BIT      USE   FIELD   FIELD   (NOTE 1)

A GROUP HAS 1 TO 4 BLOCKS (USUALLY 4) NUMBERED FROM 0.  A BLOCK'S SIZE                  ----    ------  -----   ------  --------

IS 4 TIMES RAM SIZE.  ITS START ADDRESS IS PROGRAMMABLE (SBDIAG FCN 12).                 00      DATA     0       02       014

EACH BLOCK HAS 4 SUBBLOCKS.  SUBBLOCKS ARE THE BASIC MF20 STORAGE UNIT,                  01      DATA     1       02       024

ARE 1 WORD (44 BITS) WIDE, AND ARE SELECTED BY ADDRESS 34-35.  EVERY 4TH                 02      DATA     0       03       030

WORD OF A BLOCK'S MEMORY BELONGS TO A GIVEN SUBBLOCK.  THERE IS 1 SPARE                  03      DATA     1       03       034

BIT RAM PER SUBBLOCK.  DURING A MEMORY CYCLE ALL OF THE BLOCK'S FIELDS                   04      DATA     0       04       044

AND SUBBLOCKS ARE ALWAYS CYCLED, WITH THE 4 XBUS REQUEST BITS SPECIFYING                 05      DATA     1       04       050

WHICH SUBBLOCK'S WORDS ARE ACTUALLY TO BE USED.                                          06      DATA     0       05       054

07      DATA     1       05       060

TABLE OF SLOT NUMBERS VS. GROUP, FIELD, AND BIT.                             08      DATA     0       06       064

--------------------------------------------------------                         09      DATA     1       06       070

10      DATA     0       07       074

SLOT    GROUP   FIELD                 BITS                                       11      DATA     1       07       104

----    -----   -----   --------------------------------                         12      DATA     0       08       110

8       2       0     36,38,00,02,04,06,08,10,12,14,16                         13      DATA     1       08       114

9       1       0                   SAME                                       14      DATA     0       09       120

10       0       0                   SAME                                       15      DATA     1       09       124

11       2       1     37,39,01,03,05,07,09,11,13,15,17                         16      DATA     0       10       130

12       1       1                   SAME                                       17      DATA     1       10       134

13       0       1                   SAME                                       18      DATA     2       02       140

14       2       2     40,42,18,20,22,24,26,28,30,32,34                         19      DATA     3       02       144

15       1       2                   SAME                                       20      DATA     2       03       150

16       0       2                   SAME                                       21      DATA     3       03       154

17       2       3     41,43,19,21,23,25,27,29,31,33,35                         22      DATA     2       04       160

18       1       3                   SAME                                       23      DATA     3       04       164

19       0       3                   SAME                                       24      DATA     2       05       170

25      DATA     3       05       174

------------------------------------------------------------------------                 26      DATA     2       06       204

27      DATA     3       06       210

CALCULATING A FAILED RAM "E" NUMBER                                      28      DATA     2       07       214

-----------------------------------                                      29      DATA     3       07       220

30      DATA     2       08       224

TO CALCULATE THE "E" NUMBER OF A RAM ON AN M8579 BOARD YOU NEED                  31      DATA     3       08       230

THE BLOCK NUMBER (BN), WORD NUMBER (WN), AND BIT-IN-FIELD NUMBER (BIFN).                 32      DATA     2       09       234

USE ERROR ADDRESS BITS 14-21 IN AN SBDIAG FUNCTION 12 TO READ THE                        33      DATA     3       09       240

ADDRESS RESPONSE RAM.  BITS 12-13 OF THE SBDIAG ECHO ARE BN.  ERROR                      34      DATA     2       10       244

ADDRESS BITS 34-35 ARE WN.  BIFN CAN BE DERIVED FROM THE TABLE ON THE                    35      DATA     3       10       250

RIGHT USING EITHER THE SYNDROME (FROM FUNCTION 6.1) OR KNOWN ERROR DATA                  36     ECC32     0       00       200

PATTERN.  USE BIFN TO SELECT A NUMBER FROM THE TABLE BELOW AND PLUG THAT                 37     ECC16     1       00       100

NUMBER INTO THE FORMULA TO GET THE ACTUAL "E" NUMBER.                                    38      ECC8     0       01       040

39      ECC4     1       01       020

BIFN:     0   1   2   3   4   5    6    7    8    9   10                         40      ECC2     2       00       010

TABLE:   19. 38. 57. 76. 96. 116. 149. 167. 188. 204. 224.                       41      ECC1     3       00       004

42     ECCPAR    2       01       000

FORMULA: E# = TABLE(BIFN) - (4 * WN) - BN                                        43     SPARE     3       01    (NOTE 2)

NOTE 1: THE SYNDROME IS A SIX BIT NUMBER AND IS SHOWN HERE OCTALLY GROUPED AS IT WOULD APPEAR IN AN SBDIAG FUNCTION 6.1 ECHO

IN BITS 07-12.  NOTE THAT THE 43-BIT PARITY BIT IS NOT INCLUDED IN THIS LIST.

NOTE 2: THE SPARE BIT HAS NO ASSOCIATED SYNDROME.  IF THE SPARE BIT HAPPENS TO BE REPLACING THE BIT WHICH YOUR SYNDROME

POINTS TO, THEN THE ERROR IS IN THE SPARE BIT AND NOT THE BIT WHICH THE SYNDROME POINTS TO.

Page 22

8.0  MF20 ARRAY BOARD

MF20 ARRAY BOARD PROM DATA.                           M8579 MF20 STORAGE ARRAY BOARD E NUMBER LAYOUT

EVERY MF20 STORAGE ARRAY BOARD HAS ASSOCIATED WITH IT A SERIAL      THE M8579 STORAGE BOARDS DO NOT HAVE ANY E NUMBERS

NUMBER AND OTHER RELATED DATA.  THERE IS ONE 32-BIT PROM "WORD" PER         ETCHED ON THEM, MAKING IT DIFFICULT TO LOCATE A

BOARD WHICH IS READ USING FUNCTION 2.  WHICH WORD YOU GET DEPENDS UPON      SPECIFIC DIP.  USE THE MAP BELOW TO LOCATE MOS RAM

BITS 9-12.  FOUR SBDIAGS ARE NECESSARY TO READ THE ENTIRE WORD BY           DIPS OR DC008 MUX DIPS.  EXAMPLE: THE E NUMBER FOR

VARYING THE BYTE NUMBER IN BITS 13 & 14.  THE LAYOUT OF A PROM WORD IS      BLOCK (BLK) 2, SUBBLOCK (SBLK) 1, BIT 18 WOULD BE

AS FOLLOWS:                                                                 51.  USE BLK & SBLK TO FIND THE ROW.  USING THE BIT

NUMBER, LOOK IN THE LOWER TABLE TO FIND THE COLUMN.

PROM BYTE:  0       1       2       3                               THE MAP IS ORIENTED AS IF YOU HELD THE BOARD WITH

PROM BIT:   01234567012345670123456701234567                        THE COMPONENT SIDE UP AND THE CONNECTOR EDGE TOWARDS

CONTENT:    YYYYWWWWWW###########PPABNNSSMMM                        YOU.  NOTE THAT THE ROW OF MUX CHIPS IS STAGGERED.

YYYY    IS THE 4 BIT YEAR NUMBER WHICH IS THE BCD EQUIVALENT OF THE         BLK SBLK       MOS RAM AND MUX CHIP E NUMBERS

LAST DIGIT OF THE CALENDAR YEAR.                                    --------------------------------------------------

WWWWWW  IS THE 6 BIT WEEK NUMBER.                                            0   0    224 204 188 167 149 116  96  76  57  38  19

1   0    223 203 187 166 148 115  95  75  56  37  18

##...## IS AN 11 BIT SERIAL NUMBER WHICH IS RESTARTED FROM 1 EACH WEEK.      2   0    222 202 186 165 147 114  94  74  55  36  17

3   0    221 201 185 164 146 113  93  73  54  35  16

PP      IS A MOS RAM POPULATION CODE:  00 - MOS RAM BLOCK 0 EXISTS           0   1    220 200 184 163 145 112  92  72  53  34  15

01 - BLOCKS 0 & 1 EXIST               1   1    219 199 183 162 144 111  91  71  52  33  14

10 - BLOCKS 0, 1, & 2 EXIST           2   1    218 198 182 161 143 110  90  70  51  32  13

11 - BOARD IS FULLY POPULATED         3   1    217 197 181 160 142 109  89  69  50  31  12

0   2    216 196 180 159 141 108  88  68  49  30  11

A       PARITY BIT SUCH THAT BYTES 0-2 OF THE WORD HAVE EVEN PARITY.         1   2    215 195 179 158 140 107  87  67  48  29  10

2   2    214 194 178 157 139 106  86  66  47  28   9

B       ANOTHER PARITY BIT SUCH THAT BYTE 3 HAS ODD PARITY.                  3   2    213 193 177 156 138 105  85  65  46  27   8

0   3    212 192 176 155 137 104  84  64  45  26   7

NN      IS THE 2 BIT "TIMING NUMBER".  THIS NUMBER DETERMINES WHICH          1   3    211 191 175 154 136 103  83  63  44  25   6

DISTINCT TIMING IS TO BE USED WITH THE RAMS ON THIS BOARD.           2   3    210 190 174 153 135 102  82  62  43  24   5

3   3    209 189 173 152 134 101  81  61  42  23   4

SS      IS THE MOS RAM SIZE CODE:  11 - INDICATES 4K RAMS                     MUX ==> 207 205 171 169 131  98  79  59  40  20   2

10 - 16K RAMS

01 - 32K RAMS                              FIELD    KL10 BIT NUMBERS AS POSITIONED IN FIELD

00 - 64K RAMS                              -----   -------------------------------------------

0      16  14  12  10  08  06  04  02  00  38  36

MMM     IS THE 3 BIT MOS RAM MANUFACTURER CODE:                                 1      17  15  13  11  09  07  05  03  01  39  37

2      34  32  30  28  26  24  22  20  18  42  40

000 - MOSTEK            100 - MOTOROLA                                  3      35  33  31  29  27  25  23  21  19  43  41

001 - INTEL             101 - T.I.

010 - FUJITSU           110 - OTHER (DEC, DEC PART NUMBER, ETC.)

011 - HITACHI           111 - UNASSIGNED                              =================================================

THERE ARE ACTUALLY 8 PROM WORDS PER ARRAY BOARD, OF WHICH ONLY

ONE IS ALLOWED OUT BY THE STATE OF THE 3 JUMPERS.  IF NO JUMPER IS CUT          CONTROL BOARD SLOT NUMBERS & BASIC FUNCTIONS

THEN WORD 0 IS SELECTED, IF ALL ARE CUT THEN WORD 7 IS SELECTED.  THESE      ---------------------------------------------------

JUMPERS ARE CUT IN RELATION TO THE MANUFACTURER OF THE MOS RAMS ON THE

BOARD.  NOTE ALSO THAT THE SERIAL NUMBER IS ALSO PRINTED ON THE BOARD IN     NUMBER NAME SLOT        FUNCTION

THE FORM:

YWW####                                      M8574  WRP   04  XBUS DATA/WR PATH/ECC GEN/SPARE OUT

M8576  CTL   05  XBUS & CYC CTL/WD & GRP SEL/PROM RD

**** PLEASE MAKE SURE WHEN AN ARRAY BOARD IS SENT IN FOR REPAIR THAT THE     M8575  SYN   06  SPARE IN/CHK/CORR/ADR 14-21/V MARG

**** ERROR TYPEOUT SHOWING THE SERIAL NUMBER IS ATTACHED TO THE BOARD.       M8577  ADT   07  MOS ADR/TIMING/REFRESH/BLK SEL/ERR

Page 23

9.0  MG20 ARRAY BOARD (M8570)

CALCULATING A FAILED RAM "E" NUMBER

-----------------------------------

TO CALCULATE THE "E" NUMBER OF A RAM ON AN M8570 BOARD YOU NEED

THE BLOCK NUMBER (BN), WORD NUMBER (WN), AND BIT-IN-FIELD NUMBER (BIFN).

USE ERROR ADDRESS BITS 14-21 IN AN SBDIAG FUNCTION 12 TO READ THE

ADDRESS RESPONSE RAM.  BITS 12-13 OF THE SBDIAG ECHO ARE BN.  ERROR

ADDRESS BITS 34-35 ARE WN.  BIFN CAN BE DERIVED FROM THE TABLE ON THE

RIGHT USING EITHER THE SYNDROME (FROM FUNCTION 6.1) OR KNOWN ERROR DATA

PATTERN.  USE BIFN TO SELECT A NUMBER FROM THE TABLE BELOW AND PLUG THAT

NUMBER INTO THE FORMULA TO GET THE ACTUAL "E" NUMBER.

BIFN:     0   1   2   3   4   5    6    7    8    9   10

TABLE:   19. 38. 57. 75. 92. 112. 146. 165. 183. 202. 221.

FORMULA: E# = TABLE(BIFN) - (4 * WN) - BN

M8570 MG20 STORAGE ARRAY BOARD E NUMBER LAYOUT

M8570 STORAGE BOARDS DO NOT HAVE ANY E NUMBERS ETCHED ON THEM, MAKING IT DIFFICULT TO LOCATE A SPECIFIC DIP.  USE THE MAP BELOW

TO LOCATE MOS RAM DIPS OR DC008 MUX DIPS.  EXAMPLE: THE E NUMBER FOR BLOCK (BLK) 2, SUBBLOCK (SBLK) 1, BIT 18 WOULD BE 51.  USE

BLK & SBLK TO FIND THE ROW.  USING THE BIT NUMBER, LOOK IN THE LOWER TABLE TO FIND THE COLUMN.  THE MAP IS ORIENTED AS IF YOU

HELD THE BOARD WITH THE COMPONENT SIDE UP AND THE CONNECTOR EDGE TOWARDS YOU.  NOTE THAT THE ROW OF MUX CHIPS IS STAGGERED.

BLK SBLK       MOS RAM AND MUX CHIP E NUMBERS

-----------------------------------------------------

0   0    221 202 183 165 146 112  92  75  57  38  19

1   0    220 201 182 164 145 111  91  74  56  37  18

2   0    219 200 181 163 144 110  90  73  55  36  17

3   0    218 199 180 162 143 109  89  72  54  35  16

0   1    217 198 179 161 142 108  88  71  53  34  15

1   1    216 197 178 160 141 107  87  70  52  33  14

2   1    215 196 177 159 140 106  86  69  51  32  13

3   1    214 195 176 158 139 105  85  68  50  31  12

0   2    213 194 175 157 138 104  84  67  49  30  11

1   2    212 193 174 156 137 103  83  66  48  29  10

2   2    211 192 173 155 136 102  82  65  47  28   9

3   2    210 191 172 154 135 101  81  64  46  27   8

0   3    209 190 171 153 134 100  80  63  45  26   7

1   3    208 189 170 152 133  99  79  62  44  25   6

2   3    207 188 169 151 132  98  78  61  43  24   5

3   3    206 187 168 150 131  97  77  60  42  23   4

MUX ==>  204 203 185 148 128 114  95  58  40  20   2

FIELD      KL10 BIT NUMBERS AS POSITIONED IN FIELD

-----    -------------------------------------------

0       16  14  12  10  08  06  04  02  00  38  36

1       17  15  13  11  09  07  05  03  01  39  37

2       34  32  30  28  26  24  22  20  18  42  40

3       35  33  31  29  27  25  23  21  19  43  41

====================================================

Page 24

10.0  MF20 ERRORS

10.1  CORRECTION CODE (ECC) CALCULATION

CALCULATION OF THE ECC IS BASED UPON BIT CHANGES FROM A DATA WORD OF ALL ZEROS.  THE ECC FOR ALL ZEROS IS DEFINED TO BE

11 111 11 (OR 376 AS IT WOULD APPEAR IN BITS 07-13 OF AN SBDIAG FUNC 6.0).  TO CALCULATE THE ECC  FOR ANY GIVEN WORD.  TAKE

EACH "1" BIT FROM THE DATA WORD, GET THE SYNDROME FOR THAT BIT FROM THE TABLE BELOW, AND XOR IT INTO THE PREVIOUS ECC.  INITIALLY

THE ECC IS THE ECC FOR A DATA WORD OF ZERO.

IF THERE ARE MORE ONES THAN ZEROS IN THE WORD THEN DO THE SAME AS ABOVE ONLY APPLY THE SYNDROMES FOR EACH "0" BIT AND USE

THE INITIAL ECC VALUE OF 10 101 11 (256 OCTAL).  NOTE THAT THE FOLLOWING RELATION ALWAYS HOLDS:

ECC(X)=120.XOR.ECC(.NOT.X)

WHICH MEANS THAT THE ECC FOR THE COMPLEMENT OF A WORD DIFFERS FROM THE ECC OF THE WORD BY 120 OCTAL.

*** SYNDROME & 43-BIT PARITY TABLE ***

BIT    FCN 6    SYN SYN  SYN SYN SYN  SYN 43B

#     OCTAL    32  16    8   4   2    1  PAR

---    -----    --- ---  --- --- ---  --- ---

00     016      0   0    0   0   1    1   1            EXAMPLE: CALCULATE THE ECC & 43-BIT PARITY FOR A DATA WORD OF 3

01     026      0   0    0   1   0    1   1

02     032      0   0    0   1   1    0   1                 376 IS ECC FOR ZERO

03     034      0   0    0   1   1    1   0                 250 IS SYNDROME & 43B PAR FOR BIT 35

04     046      0   0    1   0   0    1   1                 244 IS SYNDROME & 43B PAR FOR BIT 34

05     052      0   0    1   0   1    0   1                 --- .XOR. ALL OF THE ABOVE

06     054      0   0    1   0   1    1   0                 362 IS THE ECC FOR 3

07     062      0   0    1   1   0    0   1

08     064      0   0    1   1   0    1   0

09     070      0   0    1   1   1    0   0

10     076      0   0    1   1   1    1   1

11     106      0   1    0   0   0    1   1            ANOTHER METHOD FOR COMPUTING THE ECC IS AS FOLLOWS: FOR EACH OF THE 7

12     112      0   1    0   0   1    0   1            ECC BITS TAKE THE DATA WORD, .AND. IT WITH THE APPROPRIATE MASK BELOW,

13     114      0   1    0   0   1    1   0            AND COMPUTE THE RESULT'S ODD PARITY.  THAT BIT IS ONE OF THE ECC BITS.

14     122      0   1    0   1   0    0   1

15     124      0   1    0   1   0    1   0            000000 001777   ECC 32

16     130      0   1    0   1   1    0   0            000177 776000   ECC 16

17     136      0   1    0   1   1    1   1            037600 776007   ECC 8

18     142      0   1    1   0   0    0   1            343617 036170   ECC 4

19     144      0   1    1   0   0    1   0            554663 146631   ECC 2

20     150      0   1    1   0   1    0   0            665325 253252   ECC 1

21     156      0   1    1   0   1    1   1            732351 455514   43B PAR

22     160      0   1    1   1   0    0   0

23     166      0   1    1   1   0    1   1

24     172      0   1    1   1   1    0   1

25     174      0   1    1   1   1    1   0

26     206      1   0    0   0   0    1   1

27     212      1   0    0   0   1    0   1            BITS 03, 09, 10, 13, 15, 25, AND 32 (040624 002010) FORM A SET WHICH IS

28     214      1   0    0   0   1    1   0            ONE OF MANY POSSIBLE "INDEPENDENT" BIT SETS.  BY VARYING THE VALUES OF

29     222      1   0    0   1   0    0   1            THESE 7 BITS ONE CAN GENERATE ALL 128 ECC CODES, THOUGH NOT IN ANY

30     224      1   0    0   1   0    1   0            SPECIAL ORDER.  ADDING BIT 35 ALLOWS PLAYING WITH THE SPARE BIT BUT DOES

31     230      1   0    0   1   1    0   0            NOT ALTER THE INDEPENDENCE OF THIS BIT SET.

32     236      1   0    0   1   1    1   1

33     242      1   0    1   0   0    0   1

34     244      1   0    1   0   0    1   0            NOTE THAT THE OCTAL NUMBER 200,,2217 (33555599 DECIMAL) IS A SMALL,

35     250      1   0    1   0   1    0   0            RELATIVELY EASY TO REMEMBER NUMBER WHICH PRODUCES AN ALL ZEROS ECC.

Page 25

--  --   --  --  --   --  --

10  15   18  19  20   20  20  <== SUM OF SYNDROME ONE BITS

Page 26

10.2  DETECTION AND CORRECTION

HOW MF20 ERROR CORRECTION WORKS.                                     AN EXAMPLE OF ERROR CORRECTION.

--------------------------------                                     -------------------------------

WHEN A WRITE IS DONE THE MF20 CALCULATES AN ECC BASED UPON THE          WRITE A 3 TO MEMORY, ASSUME THAT BIT 27 IS STUCK

DATA IT GOT FROM THE CPU.  THE DATA AND THE ECC ARE THEN PUT IN MEMORY.         HIGH IN MEMORY.  AS SHOWN ON THE PREVIOUS PAGE

WHEN A READ IS DONE THE MF20 GETS THE DATA AND THE OLD ECC FROM MEMORY.         THE ECC FOR 3 IS 362, THEREFORE:

IT THEN COMPUTES A NEW ECC ON THE DATA AND XOR'S THE OLD AND NEW ECC'S                  3<362> GOES TO MEMORY.

WHICH RESULTS IN A SYNDROME.  OBVIOUSLY IF THERE WAS NO ERROR IN MEMORY         ON THE READ BIT 27 IS STUCK HIGH SO THAT WE SEE:

THE OLD AND NEW ECC'S WILL BE THE SAME, AND THE SYNDROME WILL BE ZERO                   403<362> FROM MEMORY.

SINCE XOR'ING EQUAL VALUES PRODUCES 0.                                          WE CALCULATE THE NEW ECC:

374 IS ECC FOR 0 (LESS 43B PAR BIT)

NOW, WHEN THERE IS AN ERROR IN ONE BIT THE NEW SYNDROME WILL                    250 IS SYNDROME FOR BIT 35

DIFFER FROM WHAT IT WOULD HAVE BEEN BY A VALUE EQUAL TO THE SYNDROME OF                 244 IS SYNDROME FOR BIT 34

THE BAD BIT.  THUS XOR'ING THE NEW SYNDROME WITH THE OLD ONE FROM MEMORY                210 IS SYNDROME FOR BIT 27

WILL RESULT IN A SYNDROME EQUAL TO THAT OF THE BAD BIT.  THE SYNDROME                   --- XOR THE WHOLE MESS TOGETHER

MAPPED TO A BIT WHICH, IN TURN, IS USED TO COMPLEMENT (IE CORRECT) THE                  170 IS THE NEW ECC.

BAD BIT.  SEE EXAMPLE ON RIGHT AT TOP.                                          WE NOW CALCULATE THE SYNDROME:

360 IS THE OLD ECC (LESS 43B PAR BIT)

WHEN ONE BIT IS BAD ON A READ THEN, OBVIOUSLY, THE PARITY OF THE                170 IS THE NEW ECC

43-BIT WORD FROM MEMORY WILL BE BAD.  BAD PARITY IS WHAT CAUSES THE MF20                --- XOR

TO RECOGNIZE A CORRECTABLE ERROR.  THE MF20 THEN GOES OFF AND COMPUTES                  210 IS THE SYNDROME

THE SYNDROME.  IF THE SYNDROME IS THAT OF ONE OF THE ECC BITS THEN THE          THE HARDWARE MAPS THE SYNDROME TO THE PROPER BIT

MAPPED CORRECTION BIT GOES NOWHERE SINCE WE DON'T SEND THE ECC BACK TO          AND CHANGES THE BIT:

THE CPU.                                                                                403 IS DATA FROM MEMORY

400 IS THE MAPPED BIT

IF THERE IS NO PARITY ERROR AND THE SYNDROME IS ZERO THEN THERE                 --- XOR

IS NO CORRECTION NECESSARY.  HOWEVER A DOUBLE BIT (UNCORRECTABLE) ERROR                   3 IS WHAT GOES BACK TO THE CPU.

GIVES GOOD PARITY (SINCE 2 BITS ARE BAD) AND A NONZERO SYNDROME.  THIS          HOCUS-POCUS-PRESTO-CHANGO, THE ERROR IS GONE!

IS HOW AN MF20 RECOGNIZES AN DOUBLE BIT ERROR.  WHEN A DOUBLE BIT ERROR

IS DETECTED THE MF20 SHIPS THE DATA, AS IS, TO THE CPU WHILE FORCING BAD      --------------------------------------------------

XBUS PARITY.  SEE THE TABLE ON THE RIGHT AT THE BOTTOM FOR A CONCISE

LIST OF ERROR CONDITIONS.                                                       SYNDROME  PARITY  MEANING & ACTION TAKEN BY MF20

--------  ------  ------------------------------

ZERO      OK    EVERY THING IS OK.  TAKE DATA

AS IT IS.

ZERO      BAD   43-BIT PARITY BIT IS BAD.

USE THE DATA AS IT IS.

1,2,4,8,    BAD   AN ECC BIT IS BAD.  USE THE

16,32             DATA AS IT IS.

NOT 0 OR    BAD   A DATA BIT IS BAD.  CORRECT IT

1,2, ETC           AND SEND BACK CORRECTED WORD.

NOT 0       OK    THIS IS A NON-CORRECTABLE

ERROR.  PUT DATA ON XBUS AS

IT IS PLUS BAD PARITY.

Page 27

10.3  DOUBLE BIT ERROR SYNDROMES

THIS IS THE TABLE OF SYNDROMES VS. ALL POSSIBLE BIT PAIRS WHICH COULD CAUSE A DOUBLE

BIT ERROR WITH THAT SYNDROME.  THE SYNDROME IS THE OCTAL NUMBER ON THE LEFT.  REMEMBER THAT

IT IS ONLY 6 BITS BUT IS SHOWN ALIGNED AS IT IS IN AN SBUS DIAG FUNCTION 6.1.  ALL THE BIT

PAIRS WHICH WOULD RESULT IN A GIVEN SYNDROME ARE ON THE LINE TO THE RIGHT OF THE SYNDROME.

THESE PAIRS ARE DECIMAL KL10 BIT NUMBERS WITH THE LOWER NUMBER OF THE PAIR TO THE LEFT OF THE

COMMA.  THE PAIRS ARE IN ORDER BY THE LOWER NUMBER.  THE TABLE CONTINUES ON THE NEXT PAGE.

SYN   BIT PAIRS WHICH COULD CAUSE A DBE WITH THE GIVEN SYNDROME

---   ---------------------------------------------------------

374   15,35 16,34 17,33 18,32 19,31 20,30 21,29 22,28 23,27 24,26 25,36

370   14,35 16,33 17,34 18,31 19,32 20,29 21,30 22,27 23,28 24,36 25,26

364   14,34 15,33 17,35 18,30 19,29 20,32 21,31 22,26 23,36 24,28 25,27

360   14,33 15,34 16,35 18,29 19,30 20,31 21,32 22,36 23,26 24,27 25,28

354   11,35 12,34 13,33 18,28 19,27 20,26 21,36 22,32 23,31 24,30 25,29

350   12,33 13,34 18,27 19,28 20,36 21,26 22,31 23,32 24,29 25,30 35,37

344   11,33 13,35 18,26 19,36 20,28 21,27 22,30 23,29 24,32 25,31 34,37

340   11,34 12,35 18,36 19,26 20,27 21,28 22,29 23,30 24,31 25,32 33,37

334   11,31 12,30 13,29 14,28 15,27 16,26 17,36 23,35 24,34 25,33 32,37

330   11,32 12,29 13,30 14,27 15,28 16,36 17,26 22,35 24,33 25,34 31,37

324   11,29 12,32 13,31 14,26 15,36 16,28 17,27 22,34 23,33 25,35 30,37

320   11,30 12,31 13,32 14,36 15,26 16,27 17,28 22,33 23,34 24,35 29,37

314   11,27 12,26 13,36 14,32 15,31 16,30 17,29 19,35 20,34 21,33 28,37

310   11,28 12,36 13,26 14,31 15,32 16,29 17,30 18,35 20,33 21,34 27,37

304   11,36 12,28 13,27 14,30 15,29 16,32 17,31 18,34 19,33 21,35 26,37

300   11,26 12,27 13,28 14,29 15,30 16,31 17,32 18,33 19,34 20,35 36,37

274   01,35 02,34 03,33 04,31 05,30 06,29 07,28 08,27 09,26 10,36 32,38

270   02,33 03,34 04,32 05,29 06,30 07,27 08,28 09,36 10,26 31,38 35,39

264   01,33 03,35 04,29 05,32 06,31 07,26 08,36 09,28 10,27 30,38 34,39

260   01,34 02,35 04,30 05,31 06,32 07,36 08,26 09,27 10,28 29,38 33,39

254   00,33 04,27 05,26 06,36 07,32 08,31 09,30 10,29 28,38 34,40 35,41

250   00,34 04,28 05,36 06,26 07,31 08,32 09,29 10,30 27,38 33,40 35,42

244   00,35 04,36 05,28 06,27 07,30 08,29 09,32 10,31 26,38 33,41 34,42

240   04,26 05,27 06,28 07,29 08,30 09,31 10,32 33,42 34,41 35,40 36,38

234   00,29 01,27 02,26 03,36 08,35 09,34 10,33 28,39 30,40 31,41 32,42

230   00,30 01,28 02,36 03,26 07,35 09,33 10,34 27,39 29,40 31,42 32,41

224   00,31 01,36 02,28 03,27 07,34 08,33 10,35 26,39 29,41 30,42 32,40

220   00,32 01,26 02,27 03,28 07,33 08,34 09,35 29,42 30,41 31,40 36,39

214   00,36 01,31 02,30 03,29 04,35 05,34 06,33 26,40 27,41 28,42 32,39

210   00,26 01,32 02,29 03,30 05,33 06,34 27,42 28,41 31,39 35,38 36,40

Page 28

204   00,27 01,29 02,32 03,31 04,33 06,35 26,42 28,40 30,39 34,38 36,41

200   00,28 01,30 02,31 03,32 04,34 05,35 26,41 27,40 29,39 33,38 36,42

174   00,22 01,20 02,19 03,18 04,16 05,15 06,14 07,13 08,12 09,11 10,37 17,38 21,39 23,40 24,41 25,42

170   00,23 01,21 02,18 03,19 04,17 05,14 06,15 07,12 08,13 09,37 10,11 16,38 20,39 22,40 24,42 25,41

164   00,24 01,18 02,21 03,20 04,14 05,17 06,16 07,11 08,37 09,13 10,12 15,38 19,39 22,41 23,42 25,40

160   00,25 01,19 02,20 03,21 04,15 05,16 06,17 07,37 08,11 09,12 10,13 14,38 18,39 22,42 23,41 24,40

154   00,18 01,24 02,23 03,22 04,12 05,11 06,37 07,17 08,16 09,15 10,14 13,38 19,40 20,41 21,42 25,39

150   00,19 01,25 02,22 03,23 04,13 05,37 06,11 07,16 08,17 09,14 10,15 12,38 18,40 20,42 21,41 24,39

144   00,20 01,22 02,25 03,24 04,37 05,13 06,12 07,15 08,14 09,17 10,16 11,38 18,41 19,42 21,40 23,39

140   00,21 01,23 02,24 03,25 04,11 05,12 06,13 07,14 08,15 09,16 10,17 18,42 19,41 20,40 22,39 37,38

134   00,14 01,12 02,11 03,37 04,24 05,23 06,22 07,21 08,20 09,19 10,18 13,39 15,40 16,41 17,42 25,38

130   00,15 01,13 02,37 03,11 04,25 05,22 06,23 07,20 08,21 09,18 10,19 12,39 14,40 16,42 17,41 24,38

124   00,16 01,37 02,13 03,12 04,22 05,25 06,24 07,19 08,18 09,21 10,20 11,39 14,41 15,42 17,40 23,38

120   00,17 01,11 02,12 03,13 04,23 05,24 06,25 07,18 08,19 09,20 10,21 14,42 15,41 16,40 22,38 37,39

114   00,37 01,16 02,15 03,14 04,20 05,19 06,18 07,25 08,24 09,23 10,22 11,40 12,41 13,42 17,39 21,38

110   00,11 01,17 02,14 03,15 04,21 05,18 06,19 07,24 08,25 09,22 10,23 12,42 13,41 16,39 20,38 37,40

104   00,12 01,14 02,17 03,16 04,18 05,21 06,20 07,23 08,22 09,25 10,24 11,42 13,40 15,39 19,38 37,41

100   00,13 01,15 02,16 03,17 04,19 05,20 06,21 07,22 08,23 09,24 10,25 11,41 12,40 14,39 18,38 37,42

074   00,07 01,05 02,04 03,38 06,39 08,40 09,41 10,42 11,24 12,23 13,22 14,21 15,20 16,19 17,18 25,37 30,35 31,34 32,33

070   00,08 01,06 02,38 03,04 05,39 07,40 09,42 10,41 11,25 12,22 13,23 14,20 15,21 16,18 17,19 24,37 29,35 31,33 32,34

064   00,09 01,38 02,06 03,05 04,39 07,41 08,42 10,40 11,22 12,25 13,24 14,19 15,18 16,21 17,20 23,37 29,34 30,33 32,35

060   00,10 01,04 02,05 03,06 07,42 08,41 09,40 11,23 12,24 13,25 14,18 15,19 16,20 17,21 22,37 29,33 30,34 31,35 38,39

054   00,38 01,09 02,08 03,07 04,40 05,41 06,42 10,39 11,20 12,19 13,18 14,25 15,24 16,23 17,22 21,37 26,35 27,34 28,33

050   00,04 01,10 02,07 03,08 05,42 06,41 09,39 11,21 12,18 13,19 14,24 15,25 16,22 17,23 20,37 27,33 28,34 35,36 38,40

044   00,05 01,07 02,10 03,09 04,42 06,40 08,39 11,18 12,21 13,20 14,23 15,22 16,25 17,24 19,37 26,33 28,35 34,36 38,41

040   00,06 01,08 02,09 03,10 04,41 05,40 07,39 11,19 12,20 13,21 14,22 15,23 16,24 17,25 18,37 26,34 27,35 33,36 38,42

034   00,39 01,40 02,41 03,42 04,09 05,08 06,07 10,38 11,16 12,15 13,14 17,37 18,25 19,24 20,23 21,22 26,31 27,30 28,29 32,36

030   00,01 02,42 03,41 04,10 05,07 06,08 09,38 11,17 12,14 13,15 16,37 18,24 19,25 20,22 21,23 26,32 27,29 28,30 31,36 39,40

024   00,02 01,42 03,40 04,07 05,10 06,09 08,38 11,14 12,17 13,16 15,37 18,23 19,22 20,25 21,24 26,29 27,32 28,31 30,36 39,41

020   00,03 01,41 02,40 04,08 05,09 06,10 07,38 11,15 12,16 13,17 14,37 18,22 19,23 20,24 21,25 26,30 27,31 28,32 29,36 39,42

014   00,42 01,02 03,39 04,05 06,38 07,10 08,09 11,12 13,37 14,17 15,16 18,21 19,20 22,25 23,24 26,27 28,36 29,32 30,31 34,35 40,41

010   00,41 01,03 02,39 04,06 05,38 07,09 08,10 11,13 12,37 14,16 15,17 18,20 19,21 22,24 23,25 26,28 27,36 29,31 30,32 33,35 40,42

004   00,40 01,39 02,03 04,38 05,06 07,08 09,10 11,37 12,13 14,15 16,17 18,19 20,21 22,23 24,25 26,36 27,28 29,30 31,32 33,34 41,42

000   A ZERO SYNDROME NEVER CAUSES A DOUBLE BIT ERROR.

Page 29

11.0  KL10 I/O INSTRUCTIONS RELATING TO MEMORY

00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

APRID           *-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

700000          *  <               MICROCODE OPTIONS               >  *  <            MICROCODE VERSION NUMBER           >  *

LH(E)   *KLPAG!XADDR!UNSTD*     !     !     *     !     !TRCKS*     !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

*  <       HARDWARE OPTIONS      >  *  <                     PROCESSOR SERIAL NUMBER                     >  *

RH(E)   *50 HZ!CACHE!CHANL* XADR!MSTR !     *     !     !     *     !     !     *     !     !     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

18    19    20    21    22    23    24    25    26    27    28    29    30    31    32    33    34    35

CONO APR,       *-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

700200          *     ! I/O !     SELECTED FLAGS    * <                SELECT FLAG                > !     *     PI LEVEL    *

E       *     !RESET!  EN * DIS ! CLR ! SET * SBUS! NXM ! IOPF*MBPAR!C DIR!ADR P*POWER!SWEEP!     *  4  !  2  !  1  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

CONI APR,       *-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

700240          *                                   * <               ENABLED FLAGS               > !                       *

LH(E)   *     !     !     *     !     !     * SBUS! NXM ! IOPF*MBPAR!C DIR!ADR P*POWER!SWEEP!     *     !     !     *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

*     !SWEEP!                       * SBUS!     ! I/O *  MB !CACHE! ADDR*POWER!SWEEP! INT *     PI LEVEL    *

RH(E)   *     ! BUSY!     *     !     !     * ERR ! NXM ! PGF * PAR ! DIR ! PAR * FAIL! DONE! REQ *  4  !  2  !  1  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

00/18 01/19 02/20 03/21 04/22 05/23 06/24 07/25 08/26 09/27 10/28 11/29 12/30 13/31 14/32 15/33 16/34 17/35

RDERA           *-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

700400          *  WORD NO  !SWEEP* CHAN!  DATA SRC *WRITE!                                         !    PHYSICAL ADDRESS   *

LH(E)   *     !     ! REF * REF !     !     * REF ! JUNK! JUNK*     !     !     *     !     !  14 *  15 !  16 !  17 *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

* <                                    PHYSICAL ADDRESS OF FIRST WORD OF TRANSFER                         > *

RH(E)   *  18 !  19 !  20 *  21 !  22 !  23 *  24 !  25 !  26 *  27 !  28 !  29 *  30 !  31 !  32 *  33 !  34 !  35 *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

DATA SRC        WRITE = 0       WRITE = 1

00       MEMORY (READ, RPW)  CHANNEL (STORE STATUS)

01                           CHANNEL (DATA STORE)

10                           EBOX STORE FROM AR

11       READ FROM CACHE     WRITEBACK FROM CACHE

(PG REFILL OR CHAN READ)

18    19    20    21    22    23    24    25    26    27    28    29    30    31    32    33    34    35

CONO PI,        *-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

700600          *  WRITE EVEN PAR *     ! DROP!CLEAR* REQ ! TURN CHAN *  TURN SYS ! <           SELECT CHANNEL            > *

E       * ADDR! DATA! DIR *     ! INT ! SYS * INT !  ON ! OFF * OFF !  ON !  1  *  2  !  3  !  4  *  5  !  6  !  7  *

*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*-----+-----+-----*

[END OF TGHA.DOC]