WTBOOT.DOC -- Changes from V6(11) to V7(12)

June 1979

COPYRIGHT (C) 1971,1975,1979 BY

DIGITAL EQUIPMENT CORPORATION, MAYNARD, MASS.

THIS SOFTWARE IS FURNISHED UNDER A LICENSE AND MAY BE USED AND  COPIED

ONLY  IN  ACCORDANCE  WITH  THE  TERMS  OF  SUCH  LICENSE AND WITH THE

INCLUSION OF THE ABOVE COPYRIGHT NOTICE.  THIS SOFTWARE OR  ANY  OTHER

COPIES  THEREOF MAY NOT BE PROVIDED OR OTHERWISE MADE AVAILABLE TO ANY

OTHER PERSON.  NO TITLE TO AND OWNERSHIP OF  THE  SOFTWARE  IS  HEREBY

TRANSFERRED.

THE INFORMATION IN THIS SOFTWARE IS SUBJECT TO CHANGE  WITHOUT  NOTICE

AND  SHOULD  NOT  BE  CONSTRUED  AS  A COMMITMENT BY DIGITAL EQUIPMENT

CORPORATION.

DIGITAL ASSUMES NO RESPONSIBILITY FOR THE USE OR  RELIABILITY  OF  ITS

SOFTWARE ON EQUIPMENT WHICH IS NOT SUPPLIED BY DIGITAL.

WTB7.DOC                                                        Page 2

WTBOOT.DOC -- Changes from V6(11) to V7(12)

June 1979

1.0  SUMMARY

This  version  of  WTBOOT  incorporates  full  support  for  the  KS10

processor.

2.0  EXTERNAL CHANGES

If WTBOOT is assembled for the KS10 system using the FTKS10 switch and

for  dialogue  mode  (STDWTB=0), it will ask the appropriate questions

for that system type.  The  answer  to  the  "which  BOOTS  to  write"

question  may  be  answered with "RH" for the RH11 version.  "SKIP" or

"NONE" are also valid answers.  No read-in loader  question  is  asked

for  the  KS10.   If  dialogue  mode  is  not  enabled  (STDWTB=1, the

default), RH11 BOOTS will be written on all available packs.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

None.

5.0  INTERNAL CHANGES

A new feature test switch, FTKS10, has been added which if  turned  on

will  generate  a  WTBOOT  that  needs  the RH11 BOOTS.  In this case,

WTBOOT must be loaded with relocatable BOOTS that is assembled for the

KS10.

[End of WTB7.DOC]

[WTB6.DOC is appended as an integral part of WTB7.DOC]

WTB6.DOC                                                        Page 3

WTBOOT.DOC -- Changes from V5 to V6(11)

April 1977

1.0  SUMMARY

The purpose of the release of V6 of WTBOOT is  to  support  RH20'S  on

KL10'S.   If  the  feature test switch FTKL10 is off (default is off),

assembly and loading of WTBOOT with BOOTS.REL produces a WTBOOT  which

is functionally identical to WTBOOT V5.

2.0  EXTERNAL CHANGES

The distributed version of WTBOOT will write a  BOOTS  on  disk  packs

which  is  functionally equivalent to the one written on disk packs by

V5.  Version 6 has been modified so as not to write a readin loader on

block 0 of a disk pack if it is being run on a KL10 unless it is asked

to do so.  In dialogue mode V6 of WTBOOT  differs  slightly  from  V5.

The  "readin  loader"  question  now  accepts the answer of "none", in

which case it will not write block 0 of the disk pack.  If the feature

test  switch  FTKL10  is set to 0 this is the only change from V5.  If

FTKL10 is set to 1, WTBOOT must be loaded with both  RH10/RP04  BOOTS,

and RH20/RP04 BOOTS.  In non-dialogue mode this version of WTBOOT will

write RH10 BOOTS on packs which are on RH10'S, and RH20 BOOTS on packs

which  are  on  RH20'S.   In dialogue mode the answer to which kind of

BOOTS is to be written can be answered "R2", in  which  case  it  will

write RH20 BOOTS, or "RP" in which case it will write RH10 BOOTS.  The

distributed versions of WTBOOT, WABOOT, and  WIBOOT  are  functionally

identical  to  those  distributed  with V5.  WLBOOT has been assembled

with FTKL10=1 and non-dialogue mode, so that it will write RH10  BOOTS

or RH20 BOOTS on the disk packs.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

None.

WTB6.DOC                                                        Page 4

5.0  INTERNAL CHANGES

A feature test switch, FTKL10, has  been  added  to  WTBOOT  which  if

turned on (defaults off) allows the selection of RH10 or RH20 BOOTS.

[End of WTB6.DOC]

[WTB5.DOC is appended as an integral part of WTB6.DOC]

WTB5.DOC                                                        Page 5

WTBOOT.DOC -- Changes from V4 to V5

September 1976

1.0  SUMMARY

The purpose of the release of version 5 of WTBOOT is  to  support  the

RP06's.

2.0  EXTERNAL CHANGES

Fix to let readin-loader understand RP06 disk drives.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

None.

5.0  INTERNAL CHANGES

None.

[End of WTB5.DOC]

[WTB4.DOC is appended as an integral part of WTB5.DOC]

WTB4.DOC                                                        Page 6

WTBOOT.DOC -- Changes from V3 to V4

July 1975

1.0  SUMMARY

The purpose of the release of  version  4  of  WTBOOT  is  to  support

version  20  of BOOTS (making the conversion from monitors in SAV file

format to monitors in EXE file format as painless  as  possible).   If

the  feature  test  switch  STDWTB is on (default is on), assembly and

loading  of  WTBOOT  with  BOOTS.REL  produces  a  WTBOOT   which   is

functionally identical to WTBOOT version 3.

2.0  EXTERNAL CHANGES

The distributed version of WTBOOT.SAV will write a BOOTS on disk packs

which  is  functionally equivalent to the one written on disk packs by

version 3.  Namely, a version of BOOTS which will read and write  .SAV

format  files from or to fixed head disks and RP02's or RP03's.  Also,

included in this distribution are two programs  called  WABOOT  (which

writes  a  version  of  BOOTS  which  will  work on both KA10 and KI10

processors) and WIBOOT (which writes a version  of  BOOTS  which  will

only work on KI10 processors but which is capable of dumping more than

256K) which will at the operators option carry  on  a  dialogue  which

allows the operator to request 1 of 3 things to selectively be written

on blocks 4-7 of each disk pack mounted on the system or to  bypass  a

disk  pack  not writing anything on blocks 4-7.  The options which are

available include writing a version of BOOTS which can read/write  EXE

format files from/to RP02/RP03 packs, writing a version of BOOTS which

can read/write EXE format files from/to RP04 packs or  writing  zeroes

in  blocks  4-7  of  the  pack (the monitor will ignore this pack when

attempting to read in  BOOTS  on  a  407  restart.   In  addition,  in

dialogue  mode  will also ask if the read-in BOOTS bootstrap should be

written in 18 bit format or in 22 bit format on block 0 of  each  RS04

or  RP04 in the system.  Finally, this distribution includes a program

called WLBOOT which will write a version of BOOTS which only works  on

the KL10 or all the RP04 packs mounted on the system.

Caution:  The version of BOOTS which will be written  onto  block  4-7

will only read/write EXE format files, i.e., it will not read or write

old format SAV files, and it will only be able to read from  or  write

to  the one type of controller which it was configured for, i.e., RP10

-RP02/RP03 drives, or RH10 - RP04 drives.   In  the  normal  case,  an

installation  will  have  one  type of controller or the other but not

both so normally, the version of BOOTS which supports that  controller

would  be  written  on  blocks  4-7  of  all  packs.   However,  if an

installation has both RP10s with RP02/RP03 drives and RH10s with  RP04

drives,  it  is  recommended that the version of BOOTS which reads and

writes on the packs where the monitor is normally kept and crashes are

normally  dumped  be written on blocks 4-7 of all packs.  For example,

if the installation has both RP03 and RP04 packs but  the  monitor  is

normally  dumped  from  DSKA and crashes are normally dumped onto DSKA

WTB4.DOC                                                        Page 7

which is a 2 pack RP04 file structure, it  is  recommended  that  RP04

BOOTS  be  written  on blocks 4-7 of all the disk packs on the system.

If the operator chooses not to enter the dialogue  mode,  WTBOOT  will

write  a  default  version of BOOTS onto blocks 4-7 of every disk pack

mounted on the system.  The default is to  write  RP02/RP03  BOOTS  on

RP02/RP03 packs and RP04 BOOTS on RP04 packs.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTIONS

Select the appropriate .SAV file from the four which  are  distributed

(WTBOOT, WABOOT, WIBOOT, or WLBOOT) and install it on SYS.

5.0  INTERNAL CHANGES

A feature test switch STDWTB (defaults off) has been added  to  WTBOOT

which  if  turned  off,  enables dialogue mode and allows WTBOOT to be

loaded with both an RP02/RP03 and an RP04 BOOTS.REL.

[End of WTB4.DOC]

[WTB3.DOC is appended as an integral part of WTB4.DOC]

WTB3.DOC                                                        Page 8

WTBOOT.DOC -- Changes from version 2(1) to 3(2)

January 1975

1.0  SUMMARY

WTBOOT version 3 now supports RP04's.  It will write an HRI  (hardware

read-in)  loader  on  block  0  of  an RP04 which will read BOOTS from

blocks 4-7 of the disk into the highest two pages of  the  processor's

physical memory.  (This is the same operation which TOPS10 performs on

a 407 restart.

Note that if the RH10 to which the RP04's are connected  is  connected

to  a  DF10C,  then  the mode select switch in the DF10C can be set to

either 18 bit mode (KA mode) or 22 bit mode (KI mode).  To  accomodate

both  cases,  version  3 of WTBOOT writes the read-in loader in 18-bit

format on even-numbered drives (0,2,4,6),  but  in  22-bit  format  on

odd-numbered  drives  (1,3,5,7).  Therefore, if the DF10C is set to KA

mode, the read-in unit switch in the RH10  should  point  at  an  even

numbered unit for HRI to succeed.  Similarly, if the DF10C is switched

to KI mode, an odd numbered unit should be  selected  by  the  read-in

unit switch in the RH10.

2.0  EXTERNAL CHANGES

RP04's are now supported.  See warnings in the summary above.

3.0  KNOWN BUGS AND DEFICIENCIES

None.

4.0  INSTALLATION INSTRUCTION

Version 17 of BOOTS is required and the  paramater  FTFHD==0  must  be

defined   in  BOOTS  to  achieve  RP04  support.   See  BTS17.DOC  and

WTBOOT.CTL.

5.0  INTERNAL CHANGES

None beyond those required for RP04 support.

[End of WTB3.DOC]

[WTBOOT.RNO is appended as an integral part of WTB3.DOC]

WTBOOT.RNO                                                      Page 9

WTBOOT.RNO -- (Originally part of 5.03 release)

May 1971

WTBOOT writes BOOTS, a disk bootstrap  loader  program,  on  blocks  4

through 7 of all disk packs in the system.  The user runs WTBOOT once,

the first time a 5.03 monitor is built.  It is necessary to run WTBOOT

again  only  if  the  disks are reformatted.  BOOTS must be written on

blocks 4 through 7 of each disk  pack  in  order  for  the  new  crash

procedure  to  work.  Refer to the crash procedure instructions in the

CRASH.RNO file.

To run WTBOOT the user logs in to [1,2] and types

.R WTBOOT

to which the system responds

EXIT

when the procedure is completed.

NOTE:  WTBOOT writes BOOTS only on disk packs that are on-line, in the

user's  file structure, and not write-locked.  For more information on

BOOTS, or on loading BOOTS with a paper tape, refer  to  the  document

entitled BOOTS, in the PDP-10 Software Notebook.

[End of WTBOOT.RNO]