-*- Text -*-				This is the file AI:KSHACK;BUILD DOC

Building a new KS10 ITS from scratch

1.  Inventory

Your KS10 ITS distribution kit includes:

1.  A KS10 boot tape containing the Salvager (labeled "Salvager").

2.  A KS10 boot tape containing DSKDMP (labeled "DSKDMP").

3.  A DUMP tape containing the minimal files needed to bring ITS up for

the first time (labeled "MINSYS").

4.  Four DUMP tapes containing sources and binaries for ITS itself, and

various programs including EMACS and MacLisp (Labeled "Distribution

tape #...").

5.  Hardcopy of the files:

KSHACK;BUILD DOC

KSHACK;CONSOL >

KSHACK;DSKDMP ORDER

KSHACK;AINOTE >

KSHACK;KSDEFS >

SYSDOC;DUMP FORMAT

.INFO.;DUMP INFO

2.  General explanations

This document contains step-by-step instructions for building a new ITS

system for a KS10.  This section introduces the major players.  The

following sections describe the steps in detail.

We use "$" to indicate the altmode/escape character (ascii code 33 octal).

Control characters are indicated by a proceeding "^", so "^Z" indicates the

character Control-Z (ascii code 32 octal).  "<cr>" is used to indicate the

carriage return character (ascii code 15 octal).

In this document we use a trailing "." to indicate a decimal number.  All

other numbers are in octal.  (Thus "105" and "69." both denote the same

number.)  However, when these instructions tell you to "type a decimal

number" it only means that the sequence of digits you type will be

interpreted as a decimal number.  It does not mean that you need to employ

any special notation to indicate the radix (such as a trailing decimal

point), in fact, you must not.

2.1.  The 8080 front end

The 8080 front end processor serves as a front console for the KS10.  It

can be used to start and stop the machine, examine and deposit memory, load

and debug both machine language and microcode, as well as boot the KS10

from tape and disk.  The 8080 also controls the system console.

Under normal operations the 8080 simply passes characters between the

processor and the system console, however typing ^\ causes the 8080 to

enter command mode.  When the machine is initially powered on, the 8080 is

already in command mode.  Command mode is indicated by the prompt:

KS10>

The 8080 will leave command mode if you type ^Z.  Relevant commands are:

MT	Loads the machine from a KS10 boot tape.  (Boot tapes are explained

below.)

BT	Loads the machine from the front end filesystem on the disk.  (Part

of the process described in this document is how one installs a

front end filesystem.)

ST nn	Starts the machine at address nn.

HA	Halts the machine.

A wallchart of the most useful 8080 front end commands can be found in the

file KSHACK;CONSOL >, there are many many more documented in the DEC

manuals.

2.2.  Tape formats

There are two different tape formats used on the tapes in your distribution

kit.

A "KS10 boot tape" is used to load a standalone program into the KS10

directly from tape.  The MT command to the 8080 front end causes it to

load a KS10 boot tape.  Such a tape starts with several records of

microcode that the 8080 loads into the KS10 micromachine.  This is followed

by a bootstrap program, called "MTBOOT", that the 8080 loads into main

memory and starts.  MTBOOT reads an Exec DDT and some other program into

memory from the rest of the tape and starts DDT.

[Footnote:  We do not use the DEC standard microcode.  The most important

differences are that the ITS microcode supports ITS-style paging and has a

different set of I/O instructions.  The file KSHACK;KSDEFS > documents the

ITS microcode.]

A "DUMP tape" contains files (and links) in the format produced by the ITS

DUMP program.  It can be used to load files into the filesystem either

using the TRAN routine in the standalone Salvager or using the DUMP program

running under timesharing.  The file SYSDOC;DUMP FORMAT describes the

format of DUMP tapes.

The two kinds of tape are not in the slightest way compatible with

each other.

2.3.  Exec DDT

There are perhaps more versions of DDT in the world than there are species

of insect.  The standalone DDT used by ITS (sometimes called "Exec DDT") is

incompatible with every other DDT, including the DDT used on ITS under

timesharing.  It's not documented anywhere, but the usual commands ($G, /,

return, etc.) work in the usual ways.

Exec DDT has very primitive rubout processing.  Typing a rubout/delete will

cause it to completely abort all unprocessed characters.

When Exec DDT is loaded its start address is 774000.  Even under

timesharing, Exec DDT is always kept at that location.  Thus you can

usually get to DDT by starting the KS10 at that address (using the ST

command to the 8080 front end).

2.4.  The Salvager

The Salvager is a program that runs standalone and performs various

filesystem maintenance operations.  It knows how to format disk packs and

how to copy them, how to build filesystems, how to detect damage in a

filesystem, and it contains tools that enable a hacker to repair a damaged

filesystem.  Documentation for the Salvager can be found in its source.

The Salvager used on the KS10 is a rewrite of the older salvager still used

for the KA10 and the KL10, its source can be found in the file

KSHACK;NSALV >.

Routines in the Salvager are run by simply using DDT to start them.  Thus

to run the Salvager routine "CHKR", one types (to Exec DDT):

CHKR$G

Relevant routines in the Salvager are:

MARK	Formats a disk pack.

TRAN	Reads files from a DUMP tape and writes them into the filesystem.

CHKR	Checks the filesystem for problems.

FESET	Sets the pointer on disk used by the 8080 front end to boot the

machine.

Examples of using all of these routines appear in the following sections.

Salvager routines prompt for and process input in a variety of

idiosyncratic and inconsistent ways.  Sometimes numbers are read in

decimal, sometimes they are read in octal.  Sometimes (for example when

asking for the number of a disk drive) only a single digit is read, and any

additional input you accidentally type, such as a carriage return, will be

taken as a response to the following command.  In the instructions that

follow, I have tried to carefully specify exactly what you must type, and

how it will be interpreted.  You should be equally careful about following

these instructions.

When a Salvager routine is finished it will generally type:

DDT

and return you to Exec DDT.  Serious errors will also cause Salvager

routines to print an error message and return to DDT.

Whenever the Salvager is waiting for input, typing ^Z will abort and return

immediately to DDT.  Remember this.  It will come in useful the first time

you make a error while typing to a Salvager routine.  (The Salvager's

rubout processing is generally pretty poor.)

A copy of the Salvager is also merged in with a dumped out ITS.  This is so

that before ITS starts running it can call the Salvager to check out the

filesystem (performing the same checks as CHKR).

2.5.  DSKDMP

DSKDMP is a sort of miniature disk operating system.  It is used to load

programs from the ITS filesystem into memory and start them, and to dump

memory into the filesystem.  The DSKDMP program itself is kept on disk in

the extra cylinders that are beyond those used by the filesystem.  DSKDMP

also uses this area of the disk for swapping space.

A short DSKDMP bootstrap program is always kept in the last 64. words of

memory, from 777700 through 777777, even when timesharing is running.  This

bootstrap knows how to find DSKDMP on disk and swap it in, swapping out

whatever is currently in main memory.  Thus once DSKDMP is installed you

can usually (unless something trashes those 64. words) start it up by

starting the KS10 at address 777700 (using the ST command to the 8080

front end).

The $U command in Exec DDT will check to see if the DSKDMP bootstrap is

present, and will jump to it if it is.  This is the most common way of

starting DSKDMP.

When the 8080 front end filesystem has been built as described below, the

BT command to the 8080 (or pressing the BOOT switch) will load the DSKDMP

bootstrap into memory and start it.  This makes it possible to get a DSKDMP

no matter what main memory contains (for example, after powering on the

machine).

DSKDMP commands are either simply the name of a file to load and start, or

a single letter followed by an altmode.  Many single letter commands are

then followed by a filename argument.  Relevant DSKDMP commands are:

F$	Lists the current directory.

L$file	Loads the file into memory without starting it.

T$file	Loads the file into memory assuming Exec DDT is already loaded.

Gives the file's symbols to DDT and starts DDT.

M$file	Merges the file into memory.

Does not load symbols or start anything.

D$file	Dumps memory into the named file.

Examples of using all of these commands appear in the following sections.

DSKDMP has an unusual convention for reading filenames: if only a single

name is given, then that is taken to be the second name of the file, and

the first name is defaulted to "@".  Thus when giving DSKDMP a filename

".;ITS" refers to the file .;@ ITS.

DSKDMP commands that read and write files sometimes take a few seconds to

run.  So that you will know when DSKDMP is done executing a command, DSKDMP

will not type a linefeed until after the command is finished.

DSKDMP has very primitive rubout processing.  Typing a rubout/delete will

simply cause it to start over and prompt for a new command.

DSKDMP error messages are somewhat terse.  A wallchart of DSKDMP commands

and error messages can be found in the file KSHACK;DSKDMP ORDER.

3.  Formatting the disk

Mount the Salvager boot tape and give the MT command to the 8080.  (Type ^\

to get the 8080 into command mode if necessary.)  This will load the

Salvager and Exec DDT into the KS10 and start DDT.  MTBOOT announces

itself:

ITS MTBOOT.176

If something goes wrong while reading the tape, MTBOOT will try and tell

you about it, otherwise when the tape stops spinning, MTBOOT is finished

and has started Exec DDT.  (A common error here is to forget to put the

tape drive on line.)

Mount a disk pack on a drive.  Understand that you are about to lose any

information you had on that pack.  Check that the drive is not write

protected.  Just to be safe, you should probably write protect any other

drives you happen to have.

Run the MARK routine the in Salvager:

MARK$G

The Salvager will begin by asking you:

Format pack on unit #

You should respond by typing the number of the drive in question.  Do not

type a carriage return after typing the number, the Salvager is only waiting

for you to type a single digit.

The Salvager now asks:

Are you sure you want to format the pack on drive # 0 (Y or N)

You should respond by typing "y" if this really is what you want to do.  Do

not type a carriage return, the Salvager is only waiting for you to type a

single character.  It will then ask you:

Pack no ?

The disk packs in an ITS filesystem are numbered.  Pack numbers can be

anywhere from 0 to 31., and are customarily written in decimal.  With this

question the Salvager is asking you what number you want this pack to be.

Your Salvager has been assembled to believe that pack number 0 is your

primary disk pack (you may have other secondary packs someday).  Since this

is in fact your primary pack you are trying to build, answer by typing "0"

followed by a carriage return.  The Salvager will type something like:

Pack 0., Drive #0 is serial #2184.

Begin formatting 815. cylinders....

and will start formatting the pack.  This will take several minutes (about

10. for an RP06), go make yourself a cup of tea while you wait.  When the

formatting is finished the Salvager will type:

Hardware formatting complete.

Verify pack? (Y or N)

The Salvager is offering to check that the pack is OK by reading back

everything it has just written.  Unless you like to live very dangerously,

you should answer "y".  Do not type a carriage return, the Salvager is only

waiting for you to type a single character.

There are likely to be ECC correctable errors detected during this

verification phase.  For each such error the Salvager will tell you where

the error occurred by typing something like:

Disk transfer error on unit 0.

ECC Corrected error: Unit #0 Cylinder 1264 Head 12 Sector 20

After verification is finished (it takes just as long as the writing

phase), the Salvager will count the number of ECC correctable errors that

occurred:

7 ECC-corrected errors during verification.

An absurdly large number of ECC errors, or some other error occurring during

verification should make you start over with a better disk pack.

The Salvager now asks you for the amount of swap space you want to allocate

on this pack for ITS to use:

Swapping Alloc?

This number (given in octal), is the number of blocks that ITS should avoid

using for file storage at the front of the pack.  This assures that there

will always be some empty space on the disk for swapping.  Since ITS will

use any free block on the disk for swapping, not just those in the swapping

area, this is really only the minimum amount of swapping space.  However,

since the first 500. blocks of the disk are already committed for storing

directories and the "HOM blocks" (part of the front end filesystem), you

will actually get 500. blocks less guaranteed swapping space than the

number you type here.  If you find this all confusing, just type

"3000<cr>".  This gives you a minimum of 1036. blocks of swapping space.

Now the Salvager asks:

Pack #0.  ID?

The Salvager wants you to tell it the name of the physical disk pack.  This

is whatever is actually written on the pack's physical label (probably a

little piece of paper glued to the cover).  (This "pack ID" is different

from the pack number, because in the future you might copy the contents of

this pack to a fresh disk pack.  That new pack would still be pack #0 in

the filesystem, but would have something different written on its label.

The pack number is the name of the information written on the pack, and the

pack ID as the name of the physical pack itself.)  You should type a string

of up to six characters followed by a carriage return.

The MARK routine is now finished formatting the pack, and it returns you to

DDT after typing:

DDT

4.  Loading the minimal files

The filesystem you just built is completely empty, so the next step is to

load a minimal set of files into the filesystem so that you can bring ITS

up for the first time.

Mount the minimal filesystem tape (the one marked "MINSYS") on the tape

drive, and run the TRAN routine in the Salvager:

TRAN$G

The Salvager asks:

onto unit #

You should respond by typing the number of the drive where your ITS pack is

mounted.  As before, you should not type a carriage return after the unit

number, since the salvager is only waiting for you to type a single digit.

As usual, the Salvager wants to make sure you know what you are doing, so

it asks:

Copy from unit #5 onto unit #0, OK (Y or N)

By "unit #5" it means "the tape drive".  This does not mean that it is

going to address the tape drive as drive number 5.  The Salvager always

uses tape drive number 0.  The digit "5" has absolutely no significance,

"unit #5" is just the Salvager's name for the tape drive.  You should

respond by typing a "y" if the target unit number is correct.

The Salvager now types:

Tape #1

.;@ DDT

This means that header on the tape you have mounted identifies this tape as

being tape #1, and that the first file on the tape is the file .;@ DDT,

which the Salvager is now loading.  When the file has been loaded, the

Salvager types "OK" and proceeds to the next file:

.;@ DDT OK

.;ITS BIN

This process should continue for about 60. files until the end of the tape,

at which point the Salvager types:

EOT

DDT

and returns you to DDT.

5.  Checking your new filesystem

At this point you might want to satisfy yourself that there really is a

good filesystem out there.  The CHKR routine in the Salvager is the

principal means for checking the consistency of the filesystem.  To run it

type:

CHKR$G

Since this routine is the Salvager's most common function, the Salvager

starts by announcing itself:

Salvager 215

before asking you:

Active unit numbers?

To answer this question the Salvager wants you to type a sequence of

digits, each representing the number of an active disk drive in your

filesystem.  Since at this point you only have a single drive, you should

just type its number, followed by a carriage return.  The Salvager will now

inform you about which packs are mounted on the drives you have specified:

Unit #0. ID is DX001, Pack #0

The unit number, pack ID, and pack number should all look familiar to you.

Now the Salvager asks:

Use MFD from unit:

The ITS filesystem keeps duplicate copies of all directories on all packs

for safety.  Here the Salvager is asking you which copy of the Master File

Directory (the catalog of all the user directories) you regard as

authoritative.  If you were recovering from some accident to a multi-pack

filesystem, this would be a useful question.  Since you only have a single

drive, just type the drive number again (and don't type a carriage return).

Now the Salvager asks:

Read all blocks of all files? (Y or N)

The Salvager is offering to test the readability of every file in your

filesystem.  When you have more files in the filesystem, this will be quite

a slow operation, but at this point it won't delay things too much, so you

might as well type "y".

Get user dirs from unit:

Now the Salvager wonders which drive has the pack you regard as

authoritative for the user directories.  Again, since you only have a

single pack, you can only answer by typing the number of its drive (and no

carriage return).

Now the Salvager starts working.  It is highly unlikely that there will be

any problem detected.  There is a possibility of some ECC correctable

errors during this process, but these aren't fatal.  If all is well the

Salvager simply returns you to DDT after typing (as usual):

DDT

6.  Installing DSKDMP

Now that you have a filesystem you need to install DSKDMP on your pack.

Start by mounting the DSKDMP boot tape and giving the MT command to the

8080.  (Type ^\ to get the 8080 into command mode if necessary.)  This will

load an Exec DDT and a program for installing DSKDMP into the machine.  As

usual MTBOOT types:

ITS MTBOOT.176

when it starts running, and leaves you in DDT after it has read the tape.

To go ahead and install a DSKDMP on disk, you have to actually run the

program that was just loaded, so type:

$G

This writes a copy of DSKDMP on your disk, installs the DSKDMP bootstrap in

memory, and starts DSKDMP running.  DSKDMP announces itself:

DSKDMP

To check that all is well, you can start by listing the directory named "."

where non-timesharing binary programs, like ITS itself, are generally

stored.  To do this, use the F$ command to DSKDMP:

F$

DSKDMP should show you a listing like this:

#00 RAM    RAM

#00 BT     BIN

#00 DSKDMP BIN

#00 SALV   BIN

#00 ITS    BIN

#00 @      DDT

where "#00" is the pack number of your single pack.  We will see what each

of these files is for in the following sections.

7.  Dumping an ITS

Now it is time to dump out an executable binary for ITS.  Start by using

the L$ command to DSKDMP to load (but not start) a copy of Exec DDT by

typing:

L$DDT<cr>

This loads the file .;@ DDT into your machine.  Now use the T$ command to

load .;ITS BIN into memory, give its symbols to Exec DDT, and start DDT, by

typing:

T$ITS BIN<cr>

Since DSKDMP has started DDT running, you can take this opportunity to use

DDT to patch ITS in any way that might be appropriate.  It's is unlikely

that you know of any such patches under the circumstances, so use the $U

command to DDT to return to DSKDMP by typing:

$U

which should cause DSKDMP to return and announce itself:

DSKDMP

Now use the M$ command to DSKDMP to merge the file .;SALV BIN into the

programs already in memory by typing:

M$SALV BIN<cr>

Finally, dump the whole mess out by typing:

D$ITS<cr>

which will dump memory into the file .;@ ITS.  If you give the F$ command

now by typing:

F$

DSKDMP should show you a directory like this:

#00 @      ITS

#00 RAM    RAM

#00 BT     BIN

#00 DSKDMP BIN

#00 SALV   BIN

#00 ITS    BIN

#00 @      DDT

8.  Bringing up ITS for the first time

Now you are ready to run timesharing for the first time.  The installation

procedure is not yet finished, because you have not yet built a front end

filesystem.  However the program for manipulating the front end filesystem

only runs under timesharing.

You should still be talking to DSKDMP after having dumped out an executable

ITS binary.  To load that binary, and start it in Exec DDT, you need only

type the name of the file to DSKDMP:

ITS<cr>

After DSKDMP is finished (indicated by typing a linefeed), tell DDT to

start ITS by typing:

$G

ITS starts by calling the Salvager, which starts by announcing itself:

Salvager 215

The Salvager will not ask you any questions this time.  It will make some

reasonable assumptions about what you want done, and will return to ITS

after it is finished.  (It exits to DDT if there are problems with the

filesystem.  "GO$G" will start ITS without running the Salvager, should you

be in foolish desperation.)  ITS now initializes itself.  ITS will discover

that it does not know the time, so it will complain:

THE KS-10 CLOCK HAS BEEN RESET, IF THE TIME CANNOT

BE DETERMINED FROM THE NETWORK, YOU MAY HAVE TO :PDSET

YEAR, PDTIME OFFSET, NOT ON DISK.

PLEASE SETLOC APPROPRIATELY.

One of the things you will do when timesharing starts is learn how to set

the time.

When ITS finally starts running the system console will type:

DX ITS 1631 SYSTEM JOB USING THIS CONSOLE.

LOGIN TARAKA 0

TARAKA DMPCPY .      _DMPCP OUTPUT WRITE

TARAKA DMPCPY .      @      ITS    DELRNM

The first line tells you what version of ITS is running, what machine (DX)

it was assembled for, and that this terminal is currently functioning as

the system console.  The second line tells you that someone with user name

"TARAKA" has logged in.  TARAKA is the system demon charged with starting

all other system demons.  One of the demons TARAKA will start is named

DMPCPY.  The next two lines show the job named "TARAKA DMPCPY" copying the

file .;@ ITS.  (ITS logs all file operations in system directories on the

system console.)  It is beyond the scope of this document to explain why

DMPCPY is copying this file.

Now you are ready to log in.  Type ^Z.  After a moment ITS should respond:

DX ITS.1631. DDT.1513.

TTY 0

You're all alone, Fair share = 75%

(ITS does not know the date, so messages cannot be reviewed right now.)

8.1.  Setting the time using PDSET

You are now talking to timesharing DDT, a program you will get to know much

better as you learn more about using ITS as a user.  At this point all you

need to know is that you run a program by typing a ":", the name of the

program, and then a carriage return.  You want to begin by telling the

system what the time is.  So run the PDSET program by typing:

:PDSET<cr>

You will then see:

(Please Log In)

___002 PDSET IOTLSR

PDSET.113

Please don't use this program unless you know how.

You are certain to break something if you happen to hit the wrong key.

Type Control-Z to exit, or ? for a reminder of the commands.

The reminder to log in (first line) was typed by timesharing DDT, which

wants to discourage you from running programs before you tell the system

who you are.  For the moment you can ignore it since your machine doesn't

have a reasonable database of user names yet anyway.

The second line was typed by ITS on the system console to advertise the

fact that the job named "___002 PDSET" is going into User I/O mode (see the

processor manual if you don't know what that is).  This message isn't

actually part of your output at all, its only because you have chosen to

log in from the system console that causes you to see this message.

"___002 PDSET" is the name of the job that timesharing DDT has created for

you to run the PDSET program in.

The rest of this output is coming from the PDSET program itself.  It tries

to scare you away from using it because normally you don't want people

randomly playing with the system's clock.  At this point you can type "?"

to see a list of the available commands if you like.  All you need to type

is:

yymmddD

where the current year is 19yy, and this is the dd'th day of the mm'th

month.  Then type:

hhmmssT

where the current time is hh:mm:ss in 24-hour time.  And then type:

!.

(That's right, an exclamation point followed by a period.)  The instant

you type the "." will be taken to be the time you have specified with the D

and T commands.  At this point the system will type on the system console

something like:

114562/ 0       773322,,1757    ___002

114561/ 0       3702    ___002

IT IS NOW  9:05:06 PM EDT, TUESDAY, APR 29,1986

These messages indicate that PDSET has persuaded ITS about the current date

and time.  To exit PDSET type:

Q

and you will be back at timesharing DDT.

8.2.  Building a front end filesystem using KSFEDR

Now run the KSFEDR program to build the front end filesystem.  To DDT type:

:KSFEDR<cr>

DDT will remind you:

(Please Log In)

and then run KSFEDR, which will start by looking for the front end

filesystem.  It won't find one, of course, so it will say:

ERROR:  File not found:  PK0000: .; .FEFS. PK0000

!

You should cause KSFEDR to create a fresh front end filesystem by typing:

create<cr>

ITS will now type on the system console something like:

___002 KSFEDR .      .FEFS. PK0000 WRITE  21:06:01

which indicates that the job ___002 KSFEDR has written a file named

.;.FEFS. PK0000, at the indicated time.  (All messages on the system

console include the time of day once the system knows what that is.)

KSFEDR now informs you of what it has done:

FE filesystem DSK: .; .FEFS. PK0000 on pack #0.

Directory address: 6000000004

!

Remember that directory address!  You will need to remember that number

later so that you can give it to the FESET routine in the Salvager and

complete the process of installing the front end filesystem.  Make sure you

get it right, it tends to have a lot of zeros in the middle which makes it

hard to remember.

Now you need to put two files in the front end filesystem.  Tell KSFEDR

that you want to write a file by typing:

write<cr>

It responds:

Are you sure you want to scribble in the FE filesystem?

You type:

yes<cr>

It asks:

Which file?

You say:

ram<cr>

because you want to write the microcode file.  It says:

Input from (Default DSK: USERS3; RAM RAM):

which is almost right, you actually want the file DSK: .; RAM RAM, so you

type

.;<cr>

and the microcode is written.  Next you want to write the DSKDMP bootstrap,

so again you say:

write<cr>

It responds:

Are you sure you want to scribble in the FE filesystem?

Again you type:

yes<cr>

When it asks:

Which file?

This time you say:

bt<cr>

And it offers

Input from (Default DSK: USERS3; BT BIN):

which is almost right, you actually want DSK: .; BT BIN, so you type

.;<cr>

and the bootstrap is in place.  Get out of KSFEDR and back to timesharing

DDT by typing

quit<cr>

8.3.  Bringing ITS down using LOCK

Now you are done using timesharing, so it is time to take ITS down again.

Run the LOCK program by typing:

:LOCK<cr>

LOCK responds (after DDT asks you to log in again):

LOCK.154

_

Say you want the system to go down in 5 minutes (you can't ask for it to

happen faster) by typing:

5kill

LOCK asks:

DO YOU REALLY WANT THE SYSTEM TO GO DOWN?

since you do, you type "y".  ITS now types on the system console:

___002 LOCK   SYS    DOWN   MAIL   WRITE  21:07:36

because your LOCK job is writing the file SYS;DOWN MAIL to contain an

explanation of why the system is going down.  LOCK will then demand:

PLEASE ENTER A BRIEF MESSAGE TO USERS, ENDED BY ^C

Since you are the only user, you hardly need to be considerate, so you

might as well immediately type a ^C.  All kinds of things now start

happening:

Timesharing DDT types on your terminal:

DX ITS going down in 4:59

because it thinks you might want to know about this.  ITS types on the

system console:

CULPRIT = ___002 LOCK   21:07:40

to tattle about who took the system down.  Then, because nobody is logged

in, ITS decides it is OK to go down immediately.  (DDT kept telling you to

log in, but you wouldn't listen!)  It types:

DX ITS 1631 NOT IN OPERATION 21:07:41

Then, the system demon named "PFTHMG DRAGON", who performs various

accounting functions and therefore has special dispensation to run after

the system is officially down, finishes up:

PFTHMG DRAGON CHANNA _DRGN_ TIMES  WRITE  21:07:42

PFTHMG DRAGON CHANNA LOGOUT TIMES  DELRNM 21:07:42

And finally ITS types:

SHUTDOWN COMPLETE

PI LEVEL 7 BUG

YOU ARE NOW IN DDT.

BUGPC/   CAIA COFFI4+1   $Q-2/   JRST COFFI7

and returns you to Exec DDT.  Don't worry about the use of the word "BUG" in

this message, there is no problem.

9.  Installing the front end filesystem

Now you want to run the FESET routine in the salvager.  You could use the

Salvager boot tape to get back to the salvager, but now that you have

DSKDMP in place it is more convenient to use it to create an executable

Salvager.  (The Salvager merged in with the ITS binary currently in memory

doesn't have any of its symbols loaded into DDT, so it's kind of

inconvenient to use.)  Start by returning to DSKDMP by typing:

$U

DSKDMP announces itself:

DSKDMP

Tell it:

L$DDT<cr>

to get a fresh copy of Exec DDT, then

T$SALV BIN<cr>

to load the file .;SALV BIN, give its symbols to DDT and start DDT.  Now

you can run the FESET routine by typing:

FESET$G

The Salvager will ask you:

Set FE filesystem directory pointer on the pack on unit #

to which you should respond by typing the number of the drive on which your

ITS pack is mounted.  Remember not to type a carriage return, as the

Salvager is only waiting for a single digit.  The Salvager now asks:

Directory address:

You should now type the directory address that KSFEDR told you about above.

The FESET routine now returns to DDT.

Now return to DSKDMP by typing:

$U

Since the Salvager currently in memory is a useful tool to have around, you

might as well save it as .;@ SALV.  Type:

D$SALV<cr>

10.  Bringing up ITS as usual

Now you should have a bootable ITS disk pack.  To test that it really

works, you can try booting it from scratch.  First type ^\ to get the 8080

into command mode.  Then tell it to boot from the front end filesystem on

disk by giving the BT command:

BT<cr>

After a few moments DSKDMP should come up and announce itself in the usual

way.  Tell DSKDMP to load the file .;@ ITS, and start it in Exec DDT by

simply typing that file's name:

ITS<cr>

When DSKDMP indicates it is finished by typing a linefeed, tell DDT to

start ITS running by typing:

$G

And ITS should come up.  (You will need to run PDSET again, because the BT

command halted the machine.)  The file KSHACK;AINOTE > is the notice we

keep posted on the front of AI to explain to users about crash dump

procedures, etc., you might want to go and read it next.

11.  Loading the rest of the filesystem using DUMP

Now that ITS is running, you will want to load in system sources and

binaries from your distribution tapes.  Type ^Z to get a timesharing DDT,

and run PDSET to set the time as you did before.  Mount the first tape, and

start up the DUMP program by typing (to timesharing DDT):

:DUMP<cr>

DUMP will announce itself:

DUMP  .399

_

The file .INFO.;DUMP INFO documents the commands accepted by the dump

program.  To load all the files (and links) from a distribution tape into

your filesystem tell DUMP:

LOAD MERGE NOASK CRDIR<cr>

("MERGE" tells DUMP that it should be careful about writing over any file

that already exists in your filesystem.  "NOASK" tells DUMP that when faced

with such a situation, it should always leave the existing file alone.

"CRDIR" tells DUMP to create any directories that are needed.)  DUMP will

want to know which files you want to load from the tape, so it will prompt

you:

FILE=

You respond by typing:

*;* *<cr>

Which means "all the files".  Now DUMP will go ahead and start loading.  It

will type something like:

TAPE NO     50 CREATION DATE  860601

After DUMP has read the entire tape, it will prompt you again with:

_

Then you can use the command:

UNLOAD<cr>

to rewind and unload the tape.  Then mount the next tape and start over

with the LOAD command again.

Note that the entire set of sources and binarys will probably not fit on a

single RM02 or RM03 disk pack.  If you have an RM02 or RM03 system, you

will have to be more selective about what you load.

If you don't want ITS to type out a line on the system console every time

DUMP loads a file into a system directory, patch the system as follows:

MNGDIR/ JRST POPJ1

12.  Getting started using ITS

Congratulations!  Now you should have a functional ITS.

A good way to get started is to run the INFO program (":INFO<cr>" to

timesharing DDT).  It is pretty much self documenting.  Type "H" the first

time you run it for a tutorial on its commands.  After you understand how

to use it, visit the nodes names "INTRO" and "DDT".  Then you are on your

own.

Since we haven't distributed many ITS, I don't know what problems you will

run into.  Just off the top of my head, here are a few problems that you

will probably notice:

ITS itself, and all programs that run under ITS assume the machine is

located in the time zone for the east coast of the United States.

Many programs have the string "MIT" built in to them.

A few programs have little tables of the names of other known ITS machines

built into them.  One of the worst offenders here is the database that

finds the name of a users home directory given his user name (binary in

INQUIR;DIRS BIN, source in INQUIR;DMUNCH >).  Another example is

SYS;ATSIGN DEVICE whose source is in SYSENG;@DEV >.

MacLisp will not work because it won't recognize the name of your machine.

Programs that are built on top of MacLisp won't work either.  An

adventurous hacker can probably find the place in the MacLisp sources to

fix this problem, and assemble a new version.  If I ever get around to

fixing MacLisp to handle this problem gracefully, I'll certainly send you

the fix.

I have no idea what you might have to do to get the ITS mail demon (COMSAT)

to run and deliver local mail.  It may not work to build a COMSAT for a

machine with no network interfaces.  I haven't been able to get a COMSAT

maintainer to sit down and think about the problem of distributing COMSATs

to far-away places.  Send mail to Bug-COMSAT at AI if you want to complain

about this.

The INQUIR system, which is a database of all the users who use ITS

machines, needs COMSAT to run in order to process changes.  Until somebody

figures out how to get you a working COMSAT, you are stuck with the

database that came on your tape.  This database contains all of us here at

MIT, but you won't have any way to add yourselves.  There are other changes

you have to make to get INQUIR run on your machine as well, for example,

one part of the INQUIR system is a MacLisp program.

I haven't told you anything about how to expand your filesystem to multiple

disk packs, or what to do if something seriously bad happens to your

filesystem, or what procedures are reasonable for backing up your

filesystem on tape.

Have fun.