From:	The Author (dsmaint@pa.dec.com)

To:	Users

Subj:	Simulator Usage, V2.2

Date:	27-Jan-96

This memorandum documents the PDP-8, PDP-11, PDP 18b, and Nova simulators.

These simulators are freeware; commercial use is prohibited, and support is

not available.  The best way to fix problems or add features is to read

and modify the sources yourself.  Alternately, you can send Internet mail

to dsmaint@pa.dec.com; but responses are NOT guaranteed.

The simulators use a common command interface.  The memorandum first

describes the common features of the command interface and then provides

information on each of the individual simulators.

1. Compiling And Running A Simulator

The simulators have been tested on VAX VMS, Alpha VMS, Alpha UNIX,

and Intel LINUX.  Porting to other environments will require changes

to the operating system dependent code in scp_tty.c.

[They have also been tested and used on VAX/Ultrix and on

MIPS/Ultrix.  In order to build on VAX/Ultrix, you will need

gcc - mbg]

To compile the simulators on Alpha UNIX, use these commands (note that

separate compilations are required for each of the 18b PDP's):

% cc pdp8*.c scp*.c -lm -o pdp8

% cc pdp11*.c scp*.c -o pdp11

% cc nova*.c scp*.c -lm -o nova

% cc -DPDP{4,7,9,15} pdp18b*.c scp*.c -lm -o pdp{4,7,9,15}

[There is also a rudimentary Makefile provided with the kit

available from

ftp://ftp.std.com/ftp/pub/mbg/pdp_simulators.tar.gz

Any comments on it are welcome at mbg@world.std.com - mbg]

To compile the simulators on LINUX, use these commands:

% cc -DLINUX pdp8*.c scp*.c -lm -lbsd -o pdp8

% cc -DLINUX pdp11*.c scp*.c -lbsd -o pdp11

% cc -DLINUX nova*.c scp*.c -lm -lbsd -o nova

% cc -DPDP{4,7,9,15} -DLINUX pdp18b*.c scp*.c -lm -o pdp{4,7,9,15}

To start the simulator, simply type its name.  The simulator takes

one optional argument, a startup command file.  If specified, this

file should contain a series of non-interactive simulator commands,

one per line.  These command can be used to set up standard parameters,

for example, disk sizes.

% pdp8 <startup file>(cr)		or

% pdp11 <startup file>(cr)		or

% nova <startup file>(cr)		or

% pdp{4,7,9,15} <startup file>(cr)

The simulator types out its name and version, executes the commands

in the startup file, if any, and then prompts for input with

sim>

2. Simulator Conventions

A simulator consists of a series of devices, the first of which is always

the CPU.  A device consists of named registers and one or more numbered

units.  Registers correspond to device state, units to device address

spaces.  Thus, the CPU device might have registers like PC, ION, etc,

and a unit corresponding to main memory; a disk device might have

registers like BUSY, DONE, etc, and units corresponding to individual

disk drives.  Except for main memory, device address spaces are simulated

as unstructured binary disk files in the host file system.  (Thus, files

with data widths greater than 8b are endian dependent.)  The SHOW CONFIG

command displays the simulator configuration.

A simulator keeps time in terms of arbitrary units, usually one time unit

per instruction executed.  Simulated events (such as completion of I/O)

are scheduled at some number of time units in the future.  The simulator

executes synchronously, invoking event processors when simulated events

are scheduled to occur.  Even asynchronous events, like keyboard input,

are handled by polling at synchronous intervals.  The SHOW QUEUE command

displays the simulator event queue.

3. Commands

3.1 Loading Programs

The LOAD command (abbreviation LO) loads a file in binary paper-tape

loader format:

sim> load <filename>(cr)

3.2 Saving and Restoring State

The SAVE command (abbreviation SA) save the complete state of the

simulator to a file.  This includes the contents of main memory and

all registers, and the I/O connections of devices:

sim> save <filename>(cr)

The RESTORE command (abbreviation REST, alternately GET) restores a

previously saved simulator state:

sim> restore <filename>(cr)

Note: SAVE file format compresses zeroes to minimize file size.

3.3 Resetting Devices

The RESET command (abbreviation RE) resets a device or the entire

simulator to a predefined condition:

sim> RESET(cr)			-- reset all devices

sim> RESET ALL(cr)		-- reset all devices

sim> RESET <device>(cr)		-- reset specified device

Typically, RESET stops any in-progress I/O operation, clears any

interrupt request, and returns the device to a quiescent state.  It

does not clear main memory or affect I/O connections.

3.4 Connecting and Disconnecting Devices

Except for main memory, simulated unit address spaces are simulated as

unstructured binary disk files in the host file file system.  Before using

a simulated unit the user must specify the file to be used by a particular

unit.  The ATTACH (abbreviation AT) command associates a unit and a file:

sim> ATTACH <device><unit number> <filename>(cr)

If the file does not exist, it is created, and an appropriate message

is printed.

The DETACH (abbreviation DET) command breaks the association between a

unit and a file and closes the file:

sim> DETACH ALL(cr)			-- detach all units

sim> DETACH <device><unit number>(cr)	-- detach specified unit

The EXIT command performs an automatic DETACH ALL.

3.5 Examining and Changing State

There are four commands to examine and change state:

EXAMINE (abbreviated E) examines state

DEPOSIT (abbreviated D) changes state

IEXAMINE (interactive examine, abbreviated IE) examines state

and allows the user to interactively change it

IDEPOSIT (interactive deposit, abbreviated ID) allows the user

to interactively change state

All four commands take the form

command {-switches} {<device>{<unit number>}} <state list>

Deposit must also include a deposit value at the end of the command.

The examine and deposit commands can work on the address space of any

unit.  If no device is specified, the CPU (main memory) is selected; if

a device but no unit is specified, unit 0 of the specified device is

selected.  The "state list" consists of one or more of the following,

separated by commas:

register		the specified register

register1-register2	all the registers starting at register1

up to and including register2

address			the specified location

address1-address2	all locations starting at address1 up to

and including address2

STATE			all registers in the device

ALL			all locations in the unit

Switches can be used to control the format of display information:

-a			display as ASCII

-c			display as character string

-m			display as instruction mnemonics

The simulators typically accept symbolic input (see simulator sections).

Examples:

sim> ex 1000-1100		-- examine 1100:1100

sim> de PC 1040			-- set PC to 1040

sim> ie 40-50			-- interactively examine 40:50

sim> ex rx0 50060		-- examine 50060, RX unit 0

sim> de all 0			-- set main memory to 0

Note: to terminate an interactive command, simply type a bad value (eg,

XXX) when input is requested.

3.6 Running Programs

The RUN command (abbreviated RU) resets all devices, deposits its argument

(if given) in the PC, and starts execution.  If no argument is given,

execution starts at the current PC.

The GO command does NOT reset devices, deposits its argument (if given) in

the PC, and starts execution.  If no argument is given, execution starts at

the current PC.

The CONT command (abbreviated CO) does NOT reset devices and resumes

execution at the current PC.

The STEP command (abbreviated S) resumes execution at the current PC for

the number of instructions given by its argument.  If no argument is

supplied, one instruction is executed.

The BOOT command (abbreviated BO) bootstraps the device and unit given

by its argument.  If no unit is supplied, unit 0 is boostrapped.  The

specified unit must be attached to a file.

3.7 Stopping Programs

Programs run until the simulator detects an error or stop condition, or

until the user forces a stop condition.

3.7.1 Simulator Detected Stop Conditions

These simulator-detected conditions stop simulation:

o HALT instruction.  If a HALT instruction is decoded, simulation stops.

o Breakpoint.  The IBKPT register provides a single virtual address

breakpoint.  If the PC matches the contents of the IBKPT register,

simulation stops.  The breakpoint is automatically disabled for

the next instruction execution.

o I/O error.  If an I/O error occurs during simulation of an I/O operation,

and the device stop-on-I/O-error flag is set, simulation usually stops.

o Processor condition.  Certain processor conditions can stop simulation;

these are described with the individual simulators.

3.7.2 User Specified Stop Conditions

Typing the interrupt character stops simulation.  The interrupt character

is defined by the WRU (where are you) register and is initially set to

005 (^E).

3.8 Setting Device Parameters

The SET command (abbreviated SE) changes the status of a device parameter:

sim> SET <unit> <parameter>

The parameters are simulator and device specific.  Disk drives, for

example, can usually be set write ENABLED or write LOCKED; if a device

supports multiple drive types, the SET command can be used to specify

the drive type.

3.8 Displaying Parameters and Status

The SHOW CONFIGURATION command shows the simulator configuration and the

status of all simulated I/O devices.

The SHOW <device> command shows the status of the named simulated I/O

device.

The SHOW QUEUE command shows the state of the simulator event queue.  Times

are in "simulation units", typically one unit per instruction execution,

relative to the current simulation time.

The SHOW TIME command shows the number of time units elapsed since

the last RUN command.

3.9 Exiting The Simulator

EXIT (synonyms QUIT and BYE) returns control to the operating system.

4. PDP-8 Features

The PDP-8 simulator is configured as follows:

device		simulates

name(s)

CPU		PDP-8/E CPU with 32KW of memory

-		KE8E extended arithmetic element (EAE)

-		KM8E memory management and timeshare control

PTR,PTP		PC8E paper tape reader/punch

TTI,TTO		KL8E console terminal

LPT		LE8E line printer

CLK		DK8E line frequency clock (also PDP-8/A compatible)

RK		RK8E/RK05 cartridge disk controller with four drives

RF		RF08/RS08 fixed head disk controller with four platters

RX		RX8E/RX01 floppy disk controller with two drives

MT		TM8E/TU10 magnetic tape controller with eight drives

The PDP-8 simulator implements one unique stop condition: if an undefined

instruction (unimplemented IOT or OPR) is decoded, and register STOP_INST

is set, the simulator halts.

4.1 CPU

The only CPU options are the presense of the EAE and the size of main memory;

the memory extension and time-share control is always included, even if

memory size is 4K.

SET CPU EAE		enable EAE

SET CPU NOEAE		disable EAE

SET CPU 4K		set memory size = 4K

SET CPU 8K		set memory size = 8K

SET CPU 12K		set memory size = 12K

SET CPU 16K		set memory size = 16K

SET CPU 20K		set memory size = 20K

SET CPU 24K		set memory size = 24K

SET CPU 28K		set memory size = 28K

SET CPU 32K		set memory size = 32K

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initial memory size is 32K.

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

name		size	comments

PC		15	program counter, including IF as high 3 bits

AC		12	accumulator

MQ		12	multiplier-quotient

L		1	link

SR		12	front panel switches

IF		3	instruction field

DF		3	data field

IB		3	instruction field buffer

SF		7	save field

UF		1	user mode flag

UB		1	user mode buffer

SC		5	EAE shift counter

GTF		1	EAE greater than flag

EMODE		1	EAE mode (0 = A, 1 = B)

ION		1	interrupt enable

ION_DELAY	1	interrupt enable delay for ION

CIF_DELAY	1	interrupt enable delay for CIF

PWR_INT		1	power fail interrupt

UF_INT		1	user mode violation interrupt

INT		15	interrupt pending flags

DONE		15	device done flags

ENABLE		15	device interrupt enable flags

OLDPC		15	PC prior to last JMP, JMS, or interrupt

STOP_INST	1	stop on undefined instruction

BREAK		16	breakpoint address (177777 to disable)

WRU		8	interrupt character

4.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

INT		1	interrupt pending flag

DONE		1	device done flag

ENABLE		1	interrupt enable flag

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		32	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

4.3 RK8E/RK05

RK8E options include the ability to place units online or offline,

write enabled or write locked:

SET RKn OFFLINE		set unit n offline

SET RKn ONLINE		set unit n online

SET RKn LOCKED		set unit n write locked

SET RKn ENABLED		set unit n write enabled

The RK8E implements these registers:

name		size	comments

STA		12	status

DA		12	disk address

MA		12	current memory address

CMD		12	disk command

BUSY		1	control busy flag

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

4.4 RX8E/RX01

RX8E options include the ability to set units write enabled or write locked:

SET RXn LOCKED		set unit n write locked

SET RXn ENABLED		set unit n write enabled

The RX8E implements these registers:

name		size	comments

RXCS		12	status

RXDB		12	data buffer

RXES		8	error status

RXTA		8	current track

RXSA		8	current sector

STAPTR		3	controller state

BUFPTR		3	buffer pointer

INT		1	interrupt pending flag

DONE		1	device done flag

ENABLE		1	interrupt enable flag

TR		1	transfer ready flag

ERR		1	error flag

CTIME		24	command completion time

STIME		24	seek time, per track

XTIME		24	transfer ready delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

4.5 RF08/RS08

The RF08 implements these registers:

name		size	comments

STA		12	status

DA		20	current disk address

MA		12	memory address (in memory)

WC		12	word count (in memory)

WLK		32	write lock switches

INT		1	interrupt pending flag

DONE		1	device done flag

TIME		24	rotational delay, per word

BURST		1	burst flag

STOP_IOE	1	stop on I/O error

The RF08 is a three-cycle data break device.  If BURST = 0, word transfers

are scheduled individually; if BURST = 1, the entire transfer occurs in

a single data break.

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RF08 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

4.6 Magnetic Tape (MT)

Magnetic tape options include the ability to place units write enabled or

or write locked.

SET MTn LOCKED		set unit n write locked

SET MTn ENABLED		set unit n write enabled

The magnetic tape controller implements these registers:

name		size	comments

CMD		12	command

FNC		12	function

CA		12	memory address

WC		12	word count

DB		12	data buffer

STA		12	main status

STA2		6	secondary status

DONE		1	device done flag

INT		1	interrupt pending flag

STOP_IOE	1	stop on I/O error

TIME		24	record delay

UST0..7		24	unit status, units 0..n

Error handling is as follows:

error		processed as

not attached	tape not ready

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	report error and stop

4.7 Symbolic Display and Input

The PDP-8 simulator implements symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as (sixbit) character string

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in:

'			ASCII character

"			two character sixbit string

alphabetic		instruction mnemonic

numeric			octal number

Instruction input uses standard PDP-8 assembler syntax.  There are four

instruction classes: memory reference, IOT, field change, and operate.

Memory reference instructions have the format

memref {I} {T/Z} address

where I signifies indirect, T a current page reference, and Z a zero page

reference.  The address is an octal number in the range 0 - 07777; if T or

Z is specified, the address is a page offset in the range 0 - 177.  Normally,

T is not needed; the simulator figures out from the address what mode to use.

However, when referencing memory outside the CPU (eg, disks), there is no

valid PC, and T must be used to specify current page addressing.

IOT instructions consist of single mnemonics, eg, KRB, TLS.  IOT instructions

may be or'd together

iot iot iot...

The simulator does not check the legality of the proposed combination.  IOT's

for which there is no opcode may be specified as IOT n, where n is an octal

number in the range 0 - 0777.

Field change instructions (CIF, CDF) have the format

fldchg field

where field is an octal number in the range 0 - 7.  Field change instructions

may be or'd together.

Operate instructions have the format

opr opr opr...

The simulator does not check the legality of the proposed combination.  EAE

mode A and B mnemonics may be specified regardless of the EAE mode.  The

operands for MUY and DVI must be deposited explicitly.

5. PDP-11 Features

The PDP-11 simulator is configured as follows:

device		simulates

name(s)

CPU		J-11 CPU with 256KB of memory

-		FP11 floating point unit (FPA)

PTR,PTP		PC11 paper tape reader/punch

TTI,TTO		DL11 console terminal

LPT		LP11 line printer

CLK		line frequency clock

RK		RK11/RK05 cartridge disk controller with eight drives

RL		RLV12/RL01(2) cartridge disk controller with four drives

RX		RX11/RX01 floppy disk controller with two drives

The PDP-11 simulator implements several unique stop conditions:

- abort during exception vector fetch, and register STOP_VEC is set

- abort during exception stack push, and register STOP_SPA is set

- trap condition 'n' occurs, and register STOP_TRAP<n> is set

- wait state entered, and no I/O operations outstanding

(ie, no interrupt can ever occur)

5.1 CPU

Memory size is a compile time parameter in pdp11_defs.h; the standard

memory size is 248KB.

These switches are recognized when examining or depositing in CPU memory:

-v			interpret address as virtual

-d			if mem mgt enabled, force data space

-k			if mem mgt enabled, force kernel mode

-s			if mem mgt enabled, force supervisor mode

-u			if mem mgt enabled, force user mode

-p			if mem mgt enabled, force previous mode

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

name		size	comments

PC		16	program counter

R0..R5		16	R0..R5, first register set

R10..R15	16	R0..R5, second register set

KSP		16	kernel stack pointer

SSP		16	supervisor stack pointer

USP		16	user stack pointer

PSW		16	processor status word

CM		2	  current mode, PSW<15:14>

PM		2	  previous mode, PSW<13:12>

RS		2	  register set, PSW<11>

IPL		3	  interrupt priority level, PSW<7:5>

T		1	  trace bit, PSW<4>

N		1	  negative flag, PSW<3>

Z		1	  zero flag, PSW<2>

V		1	  overflow flag, PSW<1>

C		1	  carry flag, PSW<0>

SR		16	front panel switches

DR		16	front panel display

MEMERR		16	memory error register

CCR		16	cache control register

MAINT		16	maintenance register

HITMISS		16	hit/miss register

CPUERR		16	CPU error register

PIRQ		16	programmed interrupt requests

FAC0H..FAC5H	32	FAC0..FAC5, high 32 bits

FAC0L..FAC5L	32	FAC0..FAC5, low 32 bits

FPS		16	floating point status

FEA		16	floating exception address

FEC		4	floating exception code

MMR0..3		16	memory management registers 0..3

{K/S/U}{I/D}{PAR/PDR}{0..7}

16	memory management registers

INT		32	interrupt pending flags

TRAP		18	trap pending flags

WAIT		0	wait state flag

WAIT_ENABLE	0	wait state enable flag

STOP_TRAPS	18	stop on trap flags

STOP_VECA	1	stop on read abort in trap or interrupt

STOP_SPA	1	stop on stack push abort in trap or interrupt

OLDPC		16	PC prior to last JMP, JMS, or interrupt

BREAK		16	breakpoint address (1 to disable)

WRU		8	interrupt character

5.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

CSR		16	control/status register

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		32	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

5.3 RK11/RK05

RK11 options include the ability to place units online or offline,

write enabled or write locked:

SET RKn OFFLINE		set unit n offline

SET RKn ONLINE		set unit n online

SET RKn LOCKED		set unit n write locked

SET RKn ENABLED		set unit n write enabled

The RK11 implements these registers:

name		size	comments

RKCS		16	control/status

RKDA		16	disk address

RKBA		16	memory address

RKWC		16	word count

RKDS		16	drive status

RKER		16	error status

INTQ		9	interrupt queue

DRVN		3	number of last selected drive

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

5.4 RX11/RX01

RX11 options include the ability to make units write enabled or write locked:

SET RXn LOCKED		set unit n write locked

SET RXn ENABLED		set unit n write enabled

The RX11 implements these registers:

name		size	comments

RXCS		12	status

RXDB		8	data buffer

RXES		8	error status

RXERR		8	error code

RXTA		8	current track

RXSA		8	current sector

STAPTR		3	controller state

BUFPTR		3	buffer pointer

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

TR		1	transfer ready flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

DONE		1	device done flag (CSR<5>)

CTIME		24	command completion time

STIME		24	seek time, per track

XTIME		24	transfer ready delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

5.5 RL11(V12)/RL01,RL02

RL11 options include the ability to set units write enabled or write locked,

and to set the drive size to RL01, RL02, or autosize:

SET RLn LOCKED		set unit n write locked

SET RLn ENABLED		set unit n write enabled

SET RLn RL01		set size to RL01

SET RLn RL02		set size to RL02

SET RLn AUTOSIZE	set size based on file size at attach

The size options can be used only when a unit is not attached to a file.

The RL11 implements these registers:

name		size	comments

RLCS		16	control/status

RLDA		16	disk address

RLBA		16	memory address

RLBAE		6	memory address extension (RLV12)

RLMP..RLMP2	16	multipurpose register queue

INT		1	interrupt pending flag

ERR		1	error flag (CSR<15>)

DONE		1	device done flag (CSR<7>)

IE		1	interrupt enable flag (CSR<6>)

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

5.6 Symbolic Display and Input

The PDP-11 simulator implements symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as two character ASCII string

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in:

'			ASCII character

"			two character ASCII string

alphabetic		instruction mnemonic

numeric			octal number

Instruction input uses standard PDP-11 assembler syntax.  There are sixteen

instruction classes:

class		operands		examples		comments

no operands	none			HALT, RESET

3b literal	literal, 0 - 7		SPL

6b literal	literal, 0 - 077	MARK

8b literal	literal, 0 - 0377	EMT, TRAP

register	register		RTS

sop		specifier		SWAB, CLR, ASL

reg-sop		register, specifier	JSR, XOR, MUL

fop		flt specifier		ABSf, NEGf

ac-fop		flt reg, flt specifier	LDf, MULf

ac-sop		flt reg, specifier	LDEXP, STEXP

ac-moded sop	flt reg, specifier	LDCif, STCfi

dop		specifier, specifier	MOV, ADD, BIC

cond branch	address			BR, BCC, BNE

sob		register, address	SOB

cc clear	cc clear instructions	CLC, CLV, CLZ, CLN	combinable

cc set		cc set instructions	SEC, SEV, SEZ, SEN	combinable

For floating point opcodes, F and D variants, and I and L variants, may be

specified regardless of the state of FPS.

The syntax for specifiers is as follows:

syntax		specifier	displacement	comments

Rn		0n		-

Fn		0n		-		only in flt reg classes

(Rn)		1n		-

@(Rn)		7n		0		equivalent to @0(Rn)

(Rn)+		2n		-

@(Rn)+		3n              -

-(Rn)		4n              -

@-(Rn)		5n              -

{+/-}d(Rn)	6n		{+/-}d

@{+/-}d(Rn)	7n		{+/-}d

#n		27		n

@#n		37		n

.+/-n		67		+/-n - 4

@.+/-n		77		+/-n - 4

{+/-}n		67		{+/-}n - PC - 4	if on disk, 37 and n

@{+/-}n		77		{+/-}n - PC - 4	if on disk, invalid

6. Nova Features

The Nova simulator is configured as follows:

device		simulates

name(s)

CPU		Nova CPU with 32KW of memory

-		hardware multiply/divide

PTR,PTP		paper tape reader/punch

TTI,TTO		console terminal

LPT		line printer

CLK		real-time clock

DK		head-per-track disk controller

DP		moving head disk controller with four drives

MT		magnetic tape controller with eight drives

The Nova simulator implements these unique stop conditions:

- reference to undefined I/O device, and STOP_DEV is set

- more than INDMAX indirect addresses are detected during

an interrupt

- more than INDMAX indirect addresses are detected during

memory reference address decoding

6.1 CPU

The only CPU options are the presense of the multiply/divide instructions

and the size of main memory.

SET CPU MDV		enable multiply/divide

SET CPU NOMDV		disable multiply/divide

SET CPU 4K		set memory size = 4K

SET CPU 8K		set memory size = 8K

SET CPU 12K		set memory size = 12K

SET CPU 16K		set memory size = 16K

SET CPU 20K		set memory size = 20K

SET CPU 24K		set memory size = 24K

SET CPU 28K		set memory size = 28K

SET CPU 32K		set memory size = 32K

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initial memory size is 32K.

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

name		size	comments

PC		15	program counter

AC0..AC3	16	accumulators 0..3

C		1	carry

SR		16	front panel switches

PI		16	priority interrupt mask

ION		1	interrupt enable

ION_DELAY	1	interrupt enable delay for ION

PWR		1	power fail interrupt

INT		15	interrupt pending flags

BUSY		15	device busy flags

DONE		15	device done flags

DISABLE		15	device interrupt disable flags

STOP_DEV	1	stop on undefined IOT

INDMAX		15	maximum number of nested indirects

OLDPC		15	PC prior to last JMP, JMS, or interrupt

BREAK		16	breakpoint address (177777 to disable)

WRU		8	interrupt character

6.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	interrupt disable flag

INT		1	interrupt pending flag

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		32	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

Terminal options include the ability to set limited Dasher compatibility

mode or normal mode:

SET TTI ANSI		normal mode

SET TTI DASHER		Dasher mode

SET TTO ANSI		normal mode

SET TTO DASHER		Dasher mode

Setting either TTI or TTO changes both devices.  In Dasher mode, carriage

return is changed to newline on input, and ^X is changed to backspace.

6.3 Fixed Head Disk (DK)

The fixed head disk controller implements these registers:

name		size	comments

STAT		16	status

DA		16	disk address

MA		16	memory address

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	device disable flag

INT		1	interrupt pending flag

WLK		8	write lock switches

TIME		24	rotational delay, per sector

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

Fixed head disk data files are buffered in memory; therefore, end of file

and OS I/O errors cannot occur.

6.4 Moving Head Disk (DP)

Moving head disk options include the ability to place units online or

offline, write enabled or write locked, and to select the type of drive:

SET DPn OFFLINE		set unit n offline

SET DPn ONLINE		set unit n online

SET DPn LOCKED		set unit n write locked

SET DPn ENABLED		set unit n write enabled

SET DPn FLOPPY		set unit n to floppy disk

SET DPn D31		set unit n to Diablo 31

SET DPn D44		set unit n to Diablo 44

SET DPn C111		set unit n to Century 111

SET DPn C114		set unit n to Century 114

All drives have 256 16b words per sector.  The other disk parameters are:

drive cylinders surfaces sectors size (MW) 	DG models

floppy	77	1	8	  .158		6038

D31	203	2	12	 1.247		4047, 4237, 4238

D44	408	4	12	 5.014		4234, 6045

C111	203	10	6	 3.118		4048

C114	203	20	12	12.472		4057, 2314

The moving head disk controller implements these registers:

name		size	comments

FCCY		16	flags, command, cylinder

USSC		16	unit, surface, sector, count

STAT		16	status

MA		16	memory address

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	interrupt disable flag

INT		1	interrupt pending flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

Error handling is as follows:

error		processed as

not attached	disk not ready

end of file	assume rest of disk is zero

OS I/O error	report error and stop

6.5 Magnetic Tape (MT)

Magnetic tape options include the ability to place units write enabled or

or write locked.

SET MTn LOCKED		set unit n write locked

SET MTn ENABLED		set unit n write enabled

The magnetic tape controller implements these registers:

name		size	comments

CU		16	command, unit

MA		16	memory address

WC		16	word count

STA1		16	status word 1

STA2		16	status word 2

EP		1	extended polling mode (not supported)

BUSY		1	device busy flag

DONE		1	device done flag

DISABLE		1	interrupt disable flag

INT		1	interrupt pending flag

STOP_IOE	1	stop on I/O error

CTIME		24	controller delay

RTIME		24	record delay

UST0..7		32	unit status, units 0..n

Error handling is as follows:

error		processed as

not attached	tape not ready

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	report error and stop

6.6 Symbolic Display and Input

The Nova simulator implements symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as two character ASCII string

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in:

'			ASCII character

"			two character ASCII string

alphabetic		instruction mnemonic

numeric			octal number

Instruction input uses standard Nova assembler syntax.  There are three

instruction classes: memory reference, IOT, and operate.

Memory reference instructions have the format

memref {ac,}{@}address{,index}

LDA and STA require an initial register; ISZ, DSZ, JSR, and JMP do not.

The syntax for addresses and indices is as follows:

syntax		mode	displacement	comments

0 <= n < 0400	0	n

{+/-}n >= 0400	1	{+/-}n - PC	must be in range [-200, 177]

invalid on disk

.+/-n		1	{+/-}n		must be in range [-200, 177]

{+/-}n,2	2	{+/-}n		must be in range [-200, 177]

{+/-}n,3	3	{+/-}n		must be in range [-200, 177]

IOT instructions have one of four formats

syntax			example

iot			HALT

iot reg			INTA

iot device		SKPDN

iot reg,device		DOAS

Devices may be specified as mnemonics or as numbers in the range 0 - 077.

Operate instructions have the format

opcode{#} reg,reg{,skip}

In all Nova instructions, blanks may be substituted for commas as field

delimiters.

7. 18b PDP Features

The four 18b PDP's (PDP-4, PDP-7, PDP-9, PDP-15) are very similar and

are configured as follows:

system	device		simulates

name(s)

PDP-4	CPU		PDP-4 CPU with 8KW of memory

PTR,PTP		integral paper tape/Type 75 punch

TTI,TTO		KSR28 console terminal (Baudot code)

LPT		Type 62 line printer (Hollerith code)

CLK		integral real-time clock

PDP-7	CPU		PDP-7 CPU with 32KW of memory

-		Type 177 extended arithmetic element (EAE)

-		Type 148 memory extension

PTR,PTP		Type 444 paper tape reader/Type 75 punch

TTI,TTO		KSR 33 console terminal

LPT		Type 647 line printer

CLK		integral real-time clock

DRM		Type 24 serial drum

PDP-9	CPU		PDP-9 CPU with 32KW of memory

-		KE09A extended arithmetic element (EAE)

-		KG09B memory extension

-		KP09A power detection

-		KX09A memory protection

PTR,PTP		PC09A paper tape reader/punch

TTI,TTO		KSR 33 console terminal

LPT		Type 647E line printer

CLK		integral real-time clock

RF		RF09/RS09 fixed-head disk

MT		TC59/TU10 magnetic tape

PDP-15	CPU		PDP-15 CPU with 32KW of memory

-		KE15 extended arithmetic element (EAE)

-		KF15 power detection

-		KM15 memory protection

PTR,PTP		PC15 paper tape reader/punch

TTI,TTO		KSR 35 console terminal

LPT		LP15 line printer

CLK		integral real-time clock

RP		RP15/RP02 disk pack

RF		RF15/RS09 fixed-head disk

MT		TC59/TU10 magnetic tape

The 18b PDP simulators implement one unique stop condition: if an undefined

instruction (unimplemented OPR) is decoded, and register STOP_INST is set,

the simulator halts.

7.1 CPU

The only CPU options are the presense of the EAE and the size of main memory.

SET CPU EAE		enable EAE

SET CPU NOEAE		disable EAE

SET CPU 4K		set memory size = 4K

SET CPU 8K		set memory size = 8K

SET CPU 12K		set memory size = 12K

SET CPU 16K		set memory size = 16K

SET CPU 20K		set memory size = 20K

SET CPU 24K		set memory size = 24K

SET CPU 28K		set memory size = 28K

SET CPU 32K		set memory size = 32K

SET CPU 48K		set memory size = 48K

SET CPU 64K		set memory size = 64K

SET CPU 80K		set memory size = 80K

SET CPU 96K		set memory size = 96K

SET CPU 112K		set memory size = 112K

SET CPU 128K		set memory size = 128K

Memory sizes greater than 8K are only available on the PDP-7, PDP-9, and

PDP-15; memory sizes greater than 32KW are only available on the PDP-15.

If memory size is being reduced, and the memory being truncated contains

non-zero data, the simulator asks for confirmation.  Data in the truncated

portion of memory is lost.  Initial memory size is 8K for the PDP-4, 32K

for the PDP-7 and PDP-9, and 128K for the PDP-15.

CPU registers include the visible state of the processor as well as the

control registers for the interrupt system.

system	name		size	comments

all	PC		addr	program counter

7,9	PC		15	program counter

15	PC		17	program counter

all	AC		18	accumulator

7,9,15	MQ		18	multiplier-quotient

7,9,15	SC		6	shift counter

7,9,15	EAE_AC_SIGN	1	EAE AC sign

all	L		1	link

7,9	EXTM		1	extend mode

15	BANKM		1	bank mode

7	TRAPM		1	trap mode

9,15	USMD		1	user mode

9,15	USMDBUF		1	user mode buffer

9,15	BR		addr	memory protection bounds

7,9,15	TRAPP		1	trap pending

9,15	NEXM		1	non-existent memory violation

9,15	PRVN		1	privilege violation

7,9	EMIRP		1	EMIR instruction pending

9,15	RESTP		1	DBR or RES instruction pending

15	XR		18	index register

15	LR		18	limit register

all	SR		18	front panel switches

all	INT		32	interrupt requests

all	IORS		18	IORS register

all	ION		1	interrupt enable

all	ION_DELAY	2	interrupt enable delay

all	OLDPC		addr	PC prior to last transfer

all	STOP_INST	1	stop on undefined instruction

all	BREAK		18	breakpoint address (177777 to disable)

all	WRU		8	interrupt character

"addr" signifies the address width of the system (13b for the PDP-4, 15b for

the PDP-7 and PDP-9, 17b for the PDP-15).

7.2 Programmed I/O Devices (PTR, PTP, TTI, TTO, CLK, LPT)

The paper tape reader (PTR), paper tape punch (PTP), and line printer

(LPT) read data from or write data to disk files.  The POS register

specifies the number of the next data item to be read or written.  Thus,

by changing POS, the user can backspace or advance these devices.

The programmed I/O devices typically implement these registers:

name		size	comments

BUF		8	last data item processed

INT		1	interrupt pending flag

DONE		1	device done flag

TIME		24	time from I/O initiation to interrupt

(for keyboard, polling interval)

POS		32	position in the input or output file

STOP_IOE	1	stop on I/O error

For the serial devices, error handling is as follows:

type	error	     STOP_IOE	processed as

in,out	not attached	1	report error and stop

0	out of tape or paper

in	end of file	1	report error and stop

0	out of tape or paper

in,out	OS I/O error	x	report error and stop

7.3 RP15/RP02

RP15 options include the ability to place units write enabled or

write locked:

SET RPn LOCKED		set unit n write locked

SET RPn ENABLED		set unit n write enabled

The RP15 implements these registers:

name		size	comments

STA		18	status A

STB		18	status B

DA		18	disk address

MA		18	current memory address

WC		18	word count

INT		1	interrupt pending flag

BUSY		1	control busy flag

STIME		24	seek time, per cylinder

RTIME		24	rotational delay

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

end of file	x	assume rest of disk is zero

OS I/O error	x	report error and stop

7.4 DRM

The drum has no options.  It implements these registers:

name		size	comments

DA		9	drum address (sector number)

MA		15	current memory address

INT		1	interrupt pending flag

DONE		1	device done flag

ERR		1	error flag

WLK		32	write lock switches

TIME		24	rotational latency, per word

STOP_IOE	1	stop on I/O error

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

Drum data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

7.5 RF09/RF15/RS09

The RF09/RF15 implements these registers:

name		size	comments

STA		18	status

DA		21	current disk address

MA		18	memory address (in memory)

WC		18	word count (in memory)

BUF		18	data buffer (diagnostic only)

INT		1	interrupt pending flag

WLK0..7		16	write lock switches for disks 0..7

TIME		24	rotational delay, per word

BURST		1	burst flag

STOP_IOE	1	stop on I/O error

The RF15/RF09 is a three-cycle data break device.  If BURST = 0, word

transfers are scheduled individually; if BURST = 1, the entire transfer

occurs in a single data break.

Error handling is as follows:

error	     STOP_IOE	processed as

not attached	1	report error and stop

0	disk not ready

RF15/RF09 data files are buffered in memory; therefore, end of file and OS

I/O errors cannot occur.

7.6 Magnetic Tape (MT)

Magnetic tape options include the ability to place units write enabled or

or write locked.

SET MTn LOCKED		set unit n write locked

SET MTn ENABLED		set unit n write enabled

The magnetic tape controller implements these registers:

name		size	comments

CMD		18	command

STA		18	main status

MA		18	memory address (in memory)

WC		18	word count (in memory)

INT		1	interrupt pending flag

STOP_IOE	1	stop on I/O error

TIME		24	record delay

UST0..7		24	unit status, units 0..n

Error handling is as follows:

error		processed as

not attached	tape not ready

end of file	(read or space) end of physical tape

(write) ignored

OS I/O error	report error and stop

7.7 Symbolic Display and Input

The 18b PDP simulators implement symbolic display and input.  Display is

controlled by command line switches:

-a			display as ASCII character

-c			display as (sixbit) character string

-m			display instruction mnemonics

The PDP-15 also recognizes an additional switch:

-p			display as packed ASCII (five 7b ASCII

characters in two 18b words)

Input parsing is controlled by the first character typed in:

'			ASCII character

"			three character sixbit string

alphabetic		instruction mnemonic

numeric			octal number

The PDP-15 also recognizes an additional input mode:

#			five character packed ASCII string in

two 18b words

Instruction input uses standard 18b PDP assembler syntax.  There are six

instruction classes: memory reference, EAE, index (PDP-15 only), IOT,

operate, and LAW.

Memory reference instructions have the format

memref {I/@} address{,X}

where I (PDP-4, PDP-7, PDP-9) /@ (PDP-15) signifies indirect reference,

and X signifies indexing (PDP-15 in page mode only).  The address is an

octal number in the range 0 - 017777 (PDP-4, PDP-7, PDP-9, and PDP-15 in

bank mode) or 0 - 07777 (PDP-15 in page mode).

IOT instructions consist of single mnemonics, eg, KRB, TLS.  IOT instructions

may be or'd together

iot iot iot...

IOT's may also include the number 10, signifying clear the accumulator

iot 10

The simulator does not check the legality of IOT combinations.  IOT's for

which there is no opcode may be specified as IOT n, where n is an octal

number in the range 0 - 07777.

EAE instructions have the format

eae {+/- shift count}

EAE instructions may be or'd together

eae eae eae...

The simulator does not check the legaility of EAE combinations.  EAE's for

which there is no opcode may be specified as EAE n, where n is an octal

number in the range 0 - 037777.

Index instructions (PDP-15 only) have the format

index {immediate}

The immediate, if allowed, must be in the range of -0400 to +0377.

Operate instructions have the format

opr opr opr...

The simulator does not check the legality of the proposed combination.  The

operands for MUY and DVI must be deposited explicitly.

Finally, the LAW instruction has the format

LAW immediate

where immediate is in the range of 0 to 017777.

Appendix 1: Notes On File Representations

1. Disks

Disks are represented as unstructured binary files of 16b data items.  Thus,

simulated disks are endian dependent.  To share a disk file between versions

of the simulator running on big and little endian systems, each 16b word

must be byte swapped prior to use.

2. Magnetic tapes

Magnetic tapes are represented as unstructured binary files of 16b data

items.  Each record consists of a 16b byte count n, followed by (n + 1)/2

16b words of data.  If the byte count is odd, the last byte in the record

is undefined.

Magnetic tapes are endian dependent.  To share a magnetic tape file between

versions of the simulator running on big and little endian systems, each

16b word must be byte swapped prior to use.

Note that the representation of magnetic tapes is only consistent within

each simulator family.  On a little endian system, a magnetic tape produced

by the Nova simulator will appear to be "correct" for the PDP-11 simulator,

even though on real hardware the results would be byte swapped.  Interchange

of tapes between simulators is not supported.

Revision History (since Rev 1.1)

Rev 2.2 Jan, 96	Added register buffers for save/restore

Added 18b PDP's

Guaranteed TTI, CLK times are non-zero

Fixed breakpoint/RUN interaction bug

Fixed magnetic tape backspace to EOF bug

Fixed ISZ/DCA inversion in PDP-8 symbol table

Fixed sixbit conversion in PDP-8 examine/deposit

Fixed unit number calculation bug in SCP and in

Nova, PDP-11, 18b PDP moving head disks

Rev 2.1	Dec, 95	Fixed PTR bug (setting done on EOF) in PDP-8, Nova

Fixed RX bug (setting error on INIT if drive 1 is

not attached) in PDP-8, PDP-11

Fixed RF treatment of photocell flag in PDP-8

Fixed autosize bug (always chose smallest disk if new

file) in PDP-11, Nova

Fixed not attached bug (reported as not attachable) in

most mass storage devices

Fixed Nova boot ROMs

Fixed bug in RESTORE (didn't requeue if delay = 0)

Fixed bug in RESTORE (clobbered device position)

Declared static constant arrays as static const

Added PDP-8, Nova magnetic tape simulators

Added Dasher mode to Nova terminal simulator

Added LINUX support

Rev 2.0	May, 95	Added symbolic assembly/disassembly